3GPP TSG-RAN5 Meeting #91-e	R5-212239

# Electronic Meeting, 17th May - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0130 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections regarding a=sendrecv and a=inactive |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UEs are allowed to include the direction attribute “a=sendrecv” in INVITE or UPDATE during call initiation but cannot be required to do so in the scenarios at hand. Also, usage of “a=inactive” has been removed quite some time ago from TS 24.229, see e.g. C1-150740 as a reference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed “a=sendrecv” and “a=inactive” from call initiation procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Compliant UEs might fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.1, A.15.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related to R5s210392 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## A.4.1	MTSI MO Voice Call / with preconditions / 5GS

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | UE sends INVITE with the first SDP offer. |
| 2 |  |  | 100 Trying | SS sends a 100 Trying provisional response. |
| 3 |  |  | 183 Session Progress | SS sends an SDP answer. |
| 4 |  |  | PRACK | UE acknowledges reception of 183 Session Progress. |
| 5 |  |  | 200 OK | SS responds to PRACK. |
| 6 |  |  | UPDATE | UE sends a second SDP offer in an UPDATE request. |
| 7 |  |  | 200 OK | SS responds to UPDATE. |
| 8 |  |  | 180 Ringing | SS sends 180 Ringing reliably. |
| 9 |  |  | PRACK | UE acknowledges reception of 180 Ringing. |
| 10 |  |  | 200 OK | SS responds to PRACK. |
| 11 |  |  | 200 OK | SS responds to INVITE. |
| 12 |  |  | ACK | UE acknowledges. |

Specific Message Contents

INVITE (Step 1)

Use the default message "INVITE for MO Call Setup" in Annex A.2.1 of TS 34.229-1 [2] applying conditions A1, A3, A4, A28, A29, A30, and A31, and with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) s=(session name) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Time description: t= (start-time) (stop-time) Media description: m=audio (transport port) RTP/AVP (fmt) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) [Note 2] b=RR: (bandwidth-value) [Note 2] Attributes for media: a=rtpmap: (payload type) EVS/16000 [Note 3, 9, 10] a=fmtp: (format) br=5.9-13.2; bw=nb-swb; max-red= (att-field) [Note 4, 5, 10] a=rtpmap: (payload type) EVS/16000 [Note 3, 9, 10] a=fmtp: (format) br=5.9-24.4; bw=nb-swb; max-red= (att-field) [Note 4, 5, 10] a=rtpmap: (payload type) EVS/16000 [Note 3, 9, 10] a=fmtp: (format) br=13.2; bw=swb; max-red= (att-field) [Note 4, 5, 10] a=rtpmap: (payload type) EVS/16000 [Note 3, 9, 10] a=fmtp: (format) br=9.6-13.2; bw=swb; max-red= (att-field) [Note 4, 5, 10] a=rtpmap: (payload type) EVS/16000 [Note 3, 9, 10] a=fmtp: (format) br=9.6-24.4; bw=swb; max-red= (att-field) [Note 4, 5, 10] a=rtpmap: (payload type) AMR-WB/16000 [Note 3, 9] a=fmtp: (format) mode-change-capability=2; max-red= (att-field) [Note 4, 6] a=rtpmap: (payload type) telephone-event/16000 a=fmtp: (format) a=rtpmap: (payload type) AMR/8000 [Note 3, 9] a=fmtp: (format) mode-change-capability=2; max-red= (att-field) [Note 4, 6] a=rtpmap: (payload type) telephone-event/8000 a=fmtp: (format) a=ecn-capable-rtp: leap ect=0 [Note 7] a=rtcp-fb:* nack ecn [Note 7] a=rtcp-xr:ecn-sum [Note 7] a=rtcp-rsize [Note 7] a=ptime:20 a=maxptime:240 Attributes for media security mechanism: a=3ge2ae: requested [Note 8] a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:WVNfX19zZW1jdGwgKCkgewkyMjA7fQp9CnVubGVz¦2^20¦ 1:4FEC_ORDER=FEC_SRTP" [Note 8] Attributes for preconditions: a=curr:qos local none a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The RR value shall be greater than 0. The RS value can be any value. Note 3: The channel number shall be "/1" or omitted. Note 4: The max-red values from 0 to 220 are allowed. Note 5: The parameters dtx, dtx-recv and evs-mode-switch shall not be present. Note 6: The parameters mode-set, mode-change-period, mode-change-neighbor, crc, robust-sorting and interleaving shall not be included. Note 7: Attributes for ECN Capability may be present if the UE supports Explicit Congestion Notification. Note 8: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 9: The ordering of payload types shall be as listed, i.e., EVS before AMR-WB before AMR according to NG.114 [31] and corresponding capability A.22/4 of TS 34.229-2 [3]. Note 10: The EVS payload type shall carry at least one of the five EVS configurations according to NG.114 [31] and corresponding capability A.22/4 of TS 34.229-2 [3]. |

100 Trying (Step 2)

Use the default message "100 Trying for INVITE" in Annex A.2.2 of TS 34.229-1 [2] applying condition A1.

183 Session Progress (Step 3)

Use the default message "183 Session Progress for INVITE" in Annex A.2.3 of TS 34.229-1 [2] applying condition A1, and with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: v=0 o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) s=- c=IN (addrtype) (connection-address for SS) b=AS:65 Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP (fmt) [Note 1, 2] b=AS:65 b=RS: (bandwidth-value) [Note 3] b=RR: (bandwidth-value) [Note 3] Attributes for media: a=rtpmap: (payload type) EVS/16000/1 [Note 1, 8] a=fmtp: (format) br=13.2; bw=swb; mode-set=0,1,2; max-red=220 [Note 8] a=rtpmap: (payload type) EVS/16000/1 [Note 1, 9] a=fmtp: (format) br=5.9-13.2; bw=nb-swb; mode-set=0,1,2, max-red=220 [Note 9] a=ecn-capable-rtp: leap ect=0 [Note 6] a=rtcp-fb:* nack ecn [Note 6] a=rtcp-xr:ecn-sum [Note 6] a=ptime:20 a=maxptime:240 Attributes for media security mechanism: a=3ge2ae: requested [Note 7] a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:PS1uQCVeeCFCanVmcjkpPywjNWhcYD0mXXtxaVBR¦2^20¦1:4 [Note 7] Attributes for preconditions: a=curr:qos local none a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos mandatory remote sendrecv a=conf:qos remote sendrecv Note 1: The values for fmt, payload type and format are copied from step 1. Note 2: Transport port is the port number of the SS (see RFC 3264 clause 6). Note 3: The bandwidth-value is copied from step 1. Note 4: All present br, br-send and br-recv parameter=value pairs are copied from step 1. Note 5: bw, bw-send and bw-recv parameter are copied from bw at step 1. Note 6: Attributes for ECN Capability are present if the UE supports Explicit Congestion Notification. Note 7: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 8: This EVS configuration is sent if UE sent it as the first of its EVS configurations in INVITE. Note 9: This EVS configuration is sent if UE did not send "br=13.2; bw=swb" as the first of its EVS configurations in INVITE. |

PRACK (Step 4)

Use the default message "PRACK" in Annex A.2.4 of TS 34.229-1 [2] applying conditions A1 and A7.

200 OK for PRACK (Step 5)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] applying conditions A10 and A22.

UPDATE (Step 6)

Use the default message "UPDATE" in Annex A.2.5 of TS 34.229-1 [2] applying conditions A1 and A6, and with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Require option-tag | precondition |
| Message-body | The following SDP types and values shall be present. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] s=(session name) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP (fmt) [Note 3] c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) b=RR: (bandwidth-value) Attributes for media: a=rtpmap: (payload type) EVS/16000 [Note 3] [Note 5] a=fmtp: (format) [Note 3] [Note 4] Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one Note 3: The value for fmt, payload type and format is not checked Note 4: Parameters for the codec are not checked Note 5: The channel number shall be "/1" or omitted. |

200 OK for UPDATE (Step 7)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] applying conditions A1, A10 and A22, and with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | SDP body of the 200 response copied from the received UPDATE and modified as follows: - IP address on "c=" lines and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; - "o=" line identical to previous SDP sent by SS except that sess-version is incremented; - Attributes for preconditions: a=curr:qos remote sendrecv |

180 Ringing (Step 8)

Use the default message "180 Ringing for INVITE" in Annex A.2.6 of TS 34.229-1 [2] applying conditions A1 and A3.

PRACK (Step 9)

Use the default message "PRACK" in Annex A.2.4 of TS 34.229-1 [2] applying conditions A1 and A7.

200 OK for PRACK (Step 10)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] applying condition A10.

200 OK for INVITE (Step 11)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] applying conditions A1, A10, and A19.

ACK (Step 12)

Use the default message "ACK" in Annex A.2.6 of TS 34.229-1 [2] applying conditions A1 and A3.

# *** Next Change ***

## A.15.1	MTSI MO Video Call / with preconditions / 5GS

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | UE sends INVITE with the first SDP offer. |
| 2 |  |  | 100 Trying | SS sends a 100 Trying provisional response. |
| 3 |  |  | 183 Session Progress | SS sends an SDP answer. |
| 4 |  |  | PRACK | UE acknowledges reception of 183 Session Progress. |
| 5 |  |  | 200 OK | SS responds to PRACK. |
| 6 |  |  | UPDATE | UE sends a second SDP offer in an UPDATE request. |
| 7 |  |  | 200 OK | SS responds to UPDATE. |
| 8 |  |  | 180 Ringing | SS sends 180 Ringing reliably. |
| 9 |  |  | PRACK | UE acknowledges reception of 180 Ringing. |
| 10 |  |  | 200 OK | SS responds to PRACK. |
| 11 |  |  | 200 OK | SS responds to INVITE. |
| 12 |  |  | ACK | UE acknowledges. |

Specific Message Contents

INVITE (Step 1)

Use the default message "INVITE for MO Call Setup" in Annex A.2.1 of TS 34.229-1 [2] applying conditions A1, A3, A4, A28, A29, A30, and A31, and with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) s=(session name) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Time description: t= (start-time) (stop-time) Media description: m=audio (transport port) RTP/AVP (fmt) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) [Note 2] b=RR: (bandwidth-value) [Note 2] Attributes for media: a=rtpmap: (payload type) EVS/16000 [Note 3, 9, 10] a=fmtp: (format) br=5.9-13.2; bw=nb-swb; max-red= (att-field) [Note 4, 5, 10] a=rtpmap: (payload type) EVS/16000 [Note 3, 9, 10] a=fmtp: (format) br=5.9-24.4; bw=nb-swb; max-red= (att-field) [Note 4, 5, 10] a=rtpmap: (payload type) EVS/16000 [Note 3, 9, 10] a=fmtp: (format) br=13.2; bw=swb; max-red= (att-field) [Note 4, 5, 10] a=rtpmap: (payload type) EVS/16000 [Note 3, 9, 10] a=fmtp: (format) br=9.6-13.2; bw=swb; max-red= (att-field) [Note 4, 5, 10] a=rtpmap: (payload type) EVS/16000 [Note 3, 9, 10] a=fmtp: (format) br=9.6-24.4; bw=swb; max-red= (att-field) [Note 4, 5, 10] a=rtpmap: (payload type) AMR-WB/16000 [Note 3, 9] a=fmtp: (format) mode-change-capability=2; max-red= (att-field) [Note 4, 6] a=rtpmap: (payload type) telephone-event/16000 a=fmtp: (format) a=rtpmap: (payload type) AMR/8000 [Note 3, 9] a=fmtp: (format) mode-change-capability=2; max-red= (att-field) [Note 4, 6] a=rtpmap: (payload type) telephone-event/8000 a=fmtp: (format) a=ecn-capable-rtp: leap ect=0 [Note 7] a=rtcp-fb:* nack ecn [Note 7] a=rtcp-xr:ecn-sum [Note 7] a=rtcp-rsize [Note 7] a=ptime:20 a=maxptime:240 Attributes for media security mechanism: a=3ge2ae: requested [Note 8] a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:WVNfX19zZW1jdGwgKCkgewkyMjA7fQp9CnVubGVz¦2^20¦ 1:4FEC_ORDER=FEC_SRTP" [Note 8] Attributes for preconditions: a=curr:qos local none a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv Media description: m=video (transport port) RTP/AVPF (fmt) or RTP/AVP (fmt) [Note 11] c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) b=RR: (bandwidth-value) Attributes for media: a=tcap:1 RTP/AVPF [Note 11] a=pcfg:1 t=1 [Note 11] a=rtpmap: (payload type) H265/90000 a=fmtp: (format) profile-id=1;level-id=(att-field) a=tcap:1 RTP/AVPF [Note 11] a=pcfg:1 t=1 [Note 11] a=rtpmap: (payload type) H264/90000 a=fmtp: (format) profile-level-id= (att-field) Attributes for preconditions: a=curr:qos local none a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The RR value shall be greater than 0. The RS value can be any value. Note 3: The channel number shall be "/1" or omitted. Note 4: The max-red values from 0 to 220 are allowed. Note 5: The parameters dtx, dtx-recv and evs-mode-switch shall not be present. Note 6: The parameters mode-set, mode-change-period, mode-change-neighbor, crc, robust-sorting and interleaving shall not be included. Note 7: Attributes for ECN Capability may be present if the UE supports Explicit Congestion Notification. Note 8: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 9: The ordering of payload types shall be as listed, i.e., EVS before AMR-WB before AMR according to NG.114 [31] and corresponding capability A.22/4 of TS 34.229-2 [3]. Note 10: The EVS payload type shall carry at least one of the five EVS configurations according to NG.114 [31] and corresponding capability A.22/4 of TS 34.229-2 [3]. Note 11: The tcap/pcfg attributes are present if RTP/AVP is present on the m line. |

100 Trying (Step 2)

Use the default message "100 Trying for INVITE" in Annex A.2.2 of TS 34.229-1 [2] applying condition A1.

183 Session Progress (Step 3)

Use the default message "183 Session Progress for INVITE" in Annex A.2.3 of TS 34.229-1 [2] applying condition A1, and with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: v=0 o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) s=- c=IN (addrtype) (connection-address for SS) b=AS:65 Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP (fmt) [Note 1, 2] b=AS:65 b=RS: (bandwidth-value) [Note 3] b=RR: (bandwidth-value) [Note 3] Attributes for media: a=rtpmap: (payload type) EVS/16000/1 [Note 1, 8] a=fmtp: (format) br=13.2; bw=swb; mode-set=0,1,2; max-red=220 [Note 8] a=rtpmap: (payload type) EVS/16000/1 [Note 1, 9] a=fmtp: (format) br=5.9-13.2; bw=nb-swb; mode-set=0,1,2, max-red=220 [Note 9] a=ecn-capable-rtp: leap ect=0 [Note 6] a=rtcp-fb:* nack ecn [Note 6] a=rtcp-xr:ecn-sum [Note 6] a=ptime:20 a=maxptime:240 Attributes for media security mechanism: a=3ge2ae: requested [Note 7] a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:PS1uQCVeeCFCanVmcjkpPywjNWhcYD0mXXtxaVBR¦2^20¦1:4 [Note 7] Attributes for preconditions: a=curr:qos local none a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos mandatory remote sendrecv a=conf:qos remote sendrecv Media description: m=video (transport port) RTP/AVPF (fmt) [Note 1] b=AS: (bandwidth-value) [Note 1] b=RS: (bandwidth-value) [Note 1] b=RR: (bandwidth-value) [Note 1] Attributes for media: a=acfg:1 t=1 [Note 10] a=rtpmap: (payload type) H265/90000 [Note 1] a=fmtp: (format) (format specific parameters) [Note 1] Attributes for preconditions: a=curr:qos local none a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos mandatory remotel sendrecv a=des:qos remote sendrecv Note 1: The values for fmt, bandwidth, payload type, format and format specific parameters are copied from step 1. Note 2: Transport port is the port number of the SS (see RFC 3264 clause 6). Note 3: The bandwidth-value is copied from step 1. Note 4: All present br, br-send and br-recv parameter=value pairs are copied from step 1. Note 5: bw, bw-send and bw-recv parameter are copied from bw at step 1. Note 6: Attributes for ECN Capability are present if the UE supports Explicit Congestion Notification. Note 7: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 8: This EVS configuration is sent if UE sent it as the first of its EVS configurations in INVITE. Note 9: This EVS configuration is sent if UE did not send "br=13.2; bw=swb" as the first of its EVS configurations in INVITE. Note 10: Present if tcap/pcfg attributes were included in step 1 |

PRACK (Step 4)

Use the default message "PRACK" in Annex A.2.4 of TS 34.229-1 [2] applying conditions A1 and A7.

200 OK for PRACK (Step 5)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] applying conditions A10 and A22.

UPDATE (Step 6)

Use the default message "UPDATE" in Annex A.2.5 of TS 34.229-1 [2] applying conditions A1 and A6, and with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Require option-tag | precondition |
| Message-body | The following SDP types and values shall be present. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] s=(session name) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP (fmt) [Note 3] c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) b=RR: (bandwidth-value) Attributes for media: a=rtpmap: (payload type) EVS/16000 [Note 3] [Note 5] a=fmtp: (format) [Note 3] [Note 4] Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Media description: m=video (transport port) RTP/AVPF (fmt) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) b=RR: (bandwidth-value) Attributes for media: a=rtpmap: (payload type) H265/90000 a=fmtp: (format) profile-id=1;level-id=(att-field) Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one Note 3: The value for fmt, payload type and format is not checked Note 4: Parameters for the codec are not checked Note 5: The channel number shall be "/1" or omitted. |

200 OK for UPDATE (Step 7)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] applying conditions A1, A10 and A22, and with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | SDP body of the 200 response copied from the received UPDATE and modified as follows: - IP address on "c=" lines and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; - "o=" line identical to previous SDP sent by SS except that sess-version is incremented; - Attributes for preconditions: a=curr:qos remote sendrecv |

180 Ringing (Step 8)

Use the default message "180 Ringing for INVITE" in Annex A.2.6 of TS 34.229-1 [2] applying conditions A1 and A3.

PRACK (Step 9)

Use the default message "PRACK" in Annex A.2.4 of TS 34.229-1 [2] applying conditions A1 and A7.

200 OK for PRACK (Step 10)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] applying condition A10.

200 OK for INVITE (Step 11)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] applying conditions A1, A10, and A19.

ACK (Step 12)

Use the default message "ACK" in Annex A.2.6 of TS 34.229-1 [2] applying conditions A1 and A3.