3GPP TSG-RAN5 Meeting #91-e	R5-212256

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1138 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Core alignment of TCI state of CSI-RS for TRS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TCI state of CSI-RS for TRS is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added TCI state based on 38.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TCI state of CSI-RS for TRS remains not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1.4A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

A.1.4A	CSI-RS for tracking

A.1.4A.1	FR1

A.1.4A.1.1	FDD

Table A.1.4A.1.1-1: CSI-RS for tracking FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.1.1 FDD | TRS.1.2 FDD |
| Bandwidth |  | BW of Active BWP1 | BW of Active BWP1 |
| SCS | kHz | 15 | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | -3 | -3 |
| TCI state |  | TCI.State.0 | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |  |

A.1.4A.1.2	TDD

Table A.1.4A.1.2-1: CSI-RS for TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.1.1 TDD | TRS.1.2 TDD |
| Bandwidth |  | BW of Active BWP1 | BW of Active BWP1 |
| SCS | kHz | 15 | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | -3 | -3 |
| TCI state |  | TCI.State.0 | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |  |

A.1.4A.2	FR2

A.1.4A.2.1	TDD

Table A.1.4A.2.1-1: CSI-RS for tracking for TDD FR2

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.2.1 TDD | TRS.2.2 TDD |
| Bandwidth |  | BW of Active BWPNote 1, 2 | BW of Active BWPNote 1,2 |
| SCS | kHz | 120 | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 l0 = 5 for CSI-RS resource 2 and 4 | l0 = 2 for CSI-RS resource 1 and 3 l0 = 6 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | -3 | -3 |
| TCI state |  | TCI.State.0 | TCI.State.1 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |  |

<<End of change>>