3GPP TSG-RAN5 Meeting #91-e	R5-212276

# Electronic Meeting, 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2124 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC IRAT test case 6.2.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The pre-test conditions specify: - for the UE to be in state (0N-B). According to TS 38.508-1 Table 4.4A.2-0, this state is defined as the UE switched off with the PLMN under test stored in the USIM. - The RPLMN in the UE to be cleared These 2 requirements are condradictory. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the pre-test conditions to specify for the UE to be in state (0N-A). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Waste of test time |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR R5s210410 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.2.1.3	Inter-RAT PLMN Selection / Selection of correct PLMN and RAT in shared network environment / Automatic mode

6.2.1.3.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and shared OPLMN cells available on NR and E-UTRAN }

ensure that {

when { UE is switched on }

then { UE selects the highest priority OPLMN and RAT combination and UE attempts to register on the selected cell and PLMN }

}

(2)

with { UE in Automatic network selection mode and shared OPLMN cells available on NR and E-UTRAN }

ensure that {

when { UE returns to coverage }

then { UE selects the highest priority OPLMN and RAT combination and UE attempts to attach on the selected cell and PLMN }

}

6.2.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3, 4.4.3.1  and 4.4.3.1.1. Unless otherwise stated these are Rel-15 requirements.

[TS 23.122, clause 4.4.3]

When the MS reselects to a cell in a shared network, and the cell is a suitable cell for multiple PLMN identities received on the BCCH or on the EC-BCCH the AS indicates these multiple PLMN identities to the NAS according to 3GPP TS 44.018 [34], 3GPP TS 44.060 [39], 3GPP TS 25.304 [32] and 3GPP TS 36.304 [43]. The MS shall choose one of these PLMNs. If the registered PLMN is available among these PLMNs, the MS shall not choose a different PLMN.

[TS 23.122, clause 4.4.3.1]

If successful registration is achieved, the MS indicates the selected PLMN.

...

NOTE 3:	If successful registration is achieved, then the current serving PLMN becomes the registered PLMN and the MS does not store the previous registered PLMN for later use.

[TS 23.122, clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

v)	other PLMN/access technology combinations in order of decreasing signal quality.

6.2.1.3.3	Test description

6.2.1.3.3.1	Pre-test conditions

System Simulator

-	2 NR cells as specified in TS 38.508-1 [4] table 4.4.2-3 are configured as shown in Table 6.2.1.3.3.2-1. System information combination NR-1 as defined in Ts 38.508-1 [4] sub-clause 4.4.3.1.2 is applied to all the NR cells.

-	1 cell belongs to E-UTRA. System information combination 1 as defined in TS 36.508 [7] clause 4.4.3.1 is used.

-	PLMN settings are defined in TS 36.523-1 [13] table 6.0.1-1.

Table 6.2.1.3.3.1-1: Cell PLMN identities

| Cell | PLMN name |
| --- | --- |
| NR Cell 1 | PLMN16 |
|  | PLMN15 |
| NR Cell 12 | PLMN16 |
|  | PLMN17 |
| E-UTRA Cell 1 | PLMN17 |
|  | PLMN16 |

UE

-	The UE is in Automatic PLMN selection mode.

-	The HPLMN is PLMN1.

-	USIM configuration 8 as specified in tables 6.4.1-8 in TS 38.508-1 [4] will be used.

Preamble

-	Ensure that the UE has cleared the RPLMN. And the UE is in state Switched OFF (state 0N-A) according to TS 38.508-1 [4].

6.2.1.3.3.2	Test procedure sequence

Table 6.2.1.3.3.2-1 for FR1 and table 6.2.1.3.3.2-2 FR2 illustrate the downlink power levels to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.1.3.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 12 | E-UTRA Cell 1 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 |  |  |
|  | RS EPRE | dBm/15kHz |  |  | -85 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 |  | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |
|  | RS EPRE | dBm/15kHz |  |  | -85 |  |

Table 6.2.1.3.3.2-2: Cell configuration changes over time for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 12 | E-UTRA Cell 1 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -82 |  |  |
|  | RS EPRE | dBm/15kHz | - | - | -85 |  |
|  | Qrxlevmin | dBm | -91+ Delta(NRf1) | -91+ Delta(NRf2) | - |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -82 | - | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |
|  | RS EPRE | dBm/15kHz | - | - | -85 |  |
|  | Qrxlevmin | dBm | - | -91+ Delta(NRf2) | - |  |

Table 6.2.1.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE send RRCSetupRequest on NR Cell 1? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 3-20a1 | Steps 3-20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 1. | - | - | - | - |
| 21 | The SS adjusts cell levels according to row T1 of table 6.2.1.3.3.2-1/2. | - | - | - | - |
| 22 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] table 4.9.7.2.2-1 is performed and the UE is camped on E-UTRAN Cell 1 with condition 'connected without release'? | - | - | 2 | P |
| 23 | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508 [7]. | - | - | - | - |

6.2.1.3.3.3	Specific message contents

Table 6.2.1.3.3.3-1: SIB1 for NR Cell 1 (all steps, Table 6.2.1.3.3.2-3)

| Derivation path: 38.508-1 [4] Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| CellAccessRelatedInfo SEQUENCE { |  |  |  |
| PLMN-IdentityInfoList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo { | 1 entry |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { | 2 entries |  |  |
| PLMN-Identity[1] SEQUENCE { |  | entry 1 |  |
| mcc | PLMN16 MCC |  |  |
| mnc | PLMN16 MNC |  |  |
| } |  |  |  |
| PLMN-Identity[2] SEQUENCE { |  | entry 2 |  |
| mcc | PLMN15 MCC |  |  |
| mnc | PLMN15 MNC |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.1.3.3.3-2: SIB1 for NR Cell 12 (all steps, Table 6.2.1.3.3.2-)

| Derivation path: 38.508-1 [4] Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| CellAccessRelatedInfo SEQUENCE { |  |  |  |
| PLMN-IdentityInfoList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo { | 1 entry |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { | 2 entries |  |  |
| PLMN-Identity[1] SEQUENCE { |  | entry 1 |  |
| mcc | PLMN16 MCC |  |  |
| mnc | PLMN16 MNC |  |  |
| } |  |  |  |
| PLMN-Identity[2] SEQUENCE { |  | entry 2 |  |
| mcc | PLMN17 MCC |  |  |
| mnc | PLMN17 MNC |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.1.3.3.3-3: SystemInformationBlockType1 for E-UTRAN Cell 1 (all steps, Table 6.2.1.3.3.2-3)

| Derivation path: 36.508[7] Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF PLMN-IdentityInfo { | 2 entries |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-Identity | PLMN17 |  |  |
| } |  |  |  |
| PLMN-IdentityInfo[2] SEQUENCE { |  | entry 2 |  |
| plmn-Identity | PLMN16 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.1.3.3.3-4: RRCSetupComplete (Step 6, Table 6.2.1.3.3.2-3)

| Derivation Path: 38.508-1 [4] Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupComplete::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity |  | 2 | PLMN15 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.2.1.3.3.3-5: RRCConnectionSetupComplete (Step 22, Table 6.2.1.3.3.2-3)

| Derivation Path: 36.331 [11] clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity |  | 1 | PLMN17 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |