3GPP TSG-WG RAN5 Meeting #91-e	R5-212281

# Electronic meeting, 17th - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4610 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further NB-IoT enhancements: new Test Models |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 03.05.2021 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Test case 22.3.2.8 requires RLC in User Plane mode of operation of DRBs on UE side. No test model exists for this feature. - Introduction of new narrowband wake-up signal (NWUS) test cases require corresponding update of the test model. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Created clause 4A.2.1.3A. - Updated clause 7A.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Test case 22.3.2.8 cannot be implemented. - NWUS test cases cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4A.2.1.3A(new), 7A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of changes>>

#### 4A.2.1.3	RLC test model (CP mode)

TTCN CODE

TM

TM

MAC

SRB1bis

PHY

Loopback above RLC in UE

SRB1bis

Config/Control

DCCH

DCCH

RLC

Figure 4A.2.1.3-1: Test model for RLC testing in NB-IoT CP mode

In general NB-IoT RLC test cases are implemented for control plane mode using loopback mode as described in clause 4A.2.1.1. At the SS RLC layer is configured in transparent mode with L2TestMode enabled. Transparent mode means that the TTCN fully controls processing of the RLC PDUs and the SS is responsible for encoding and decoding of the RLC PDUs only.

Only RLC AM mode of operation is used. On the SS Side, L1 and MAC are configured in the normal way. The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over the system control port.

#### 4A.2.1.3A	RLC test model (UP mode)

Figure 4A.2.1.3A-1: Test model for RLC testing in UP mode

This test model is suitable for testing RLC in User Plane mode of operation of DRBs on UE side.

The UE is configured in Test Loop Mode A, to loop back the user domain data above PDCP layer. On UE side Ciphering is enabled (since mandatory) but with dummy ciphering algorithm, which is equivalent to not using ciphering. ROHC is not configured on UE Side.

On the SS Side, L1 and MAC are configured in the normal way. The RLC is configured in transparent mode. Hence with this configuration PDUs out of SS RLC are same as the SDUs in it. There is no PDCP configured on SS Side. The ports are directly above RLC.

The PDUs exchanged between TTCN and SS, shall be the final RLC PDUs consisting of RLC and PDCP headers. TTCN code shall take care in DL of building RLC headers and PDCP headers and in UL handle RLC and PDCP headers. TTCN code shall take care of maintaining sequence numbers and state variables for RLC and PDCP layers.

The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over system control port.

NOTE:	In practice, this test model is only used with RLC UM mode of operation.

<<End of changes>>

<<Start of changes>>

# 7A	NB-IoT Test Methods and Design Considerations

NOTE:	Unless explicitly stated otherwise, the present clause and its subclauses apply to the NB-IoT anchor carrier.

## 7A.1	Physical signals and channels

For Signalling testing, single Tx antenna shall be used by the SS, unless specified otherwise in a test case.

The SS shall transmit all NB-IoT physical signals and channels according to TS 36.211 [35], with the following additional requirements, unless specified otherwise in a test case:

-	The NRS shall be transmitted on one antenna port.

-	The NPSS and NSSS shall be transmitted on one and the same antenna port.

-	The NWUS shall be transmitted on one and the same antenna port, which is different from the antenna port where NPSS and NSSS shall be transmitted.

No (WB-E-UTRA) LTE signals and channels shall be transmitted by the SS, except for the case of ‘in-band same PCI’ operation mode where the SS shall transmit the LTE CRS. For the in-band operation modes:

-	TTCN will provide the necessary LTE CRS configuration to the SS ; cyclic prefix (CP) is normal.

-	For the case of ‘in-band different PCI’ operation mode:

-	 The SS shall use this information to skip the resource elements, which should be occupied by the LTE CRS, when transmitting NB-IoT downlink physical channels;

-	The LTE physical cell identity  is calculated from the NB-IoT physical cell identity  as follows:

, where:

= (FLOOR(/ 3) + 1) MOD 168

= (( - 3 *) + 1) MOD 3

For NWUS generation as per TS 36.211 [35] clause 10.2.6B, TTCN provides SS with:

-	time offset between the end of NWUS and start of the first PO in frames, used to calculate nf_start_PO needed for generation of cinit_WUS,

-	with the actual duration of the NWUS signal in subframes,

-	and optionally with the power offset of NWUS relative to NRS in dB.

Actual transmission of the NWUS signal is triggered by the system request at the activation time calculated by TTCN.