3GPP TSG-RAN WG5 Meeting #91-e	R5-212291

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0131 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to generic test procedure 5.3.22 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Incorrect resp. missing message sequences: According to TS 33.180 clauses 7.3.3.3 and 5.7.3 the client gets a notification about user profile change after the user has been added to the new temporary group. The UE retrieves the user profile document and then subscribes to the changes of the new temporary group (TS 33.180 clause 5.3.2, TS 23.280 clause 10.1.5.3). The SS then sends a notification with the URI of the group document to retrieve and the client retrieves the group document – and dependent on subscription before – the group key is sent via SIP NOTIFY in the same dialog or another subscription for the group key happens. - Naming of the temporary group in procedure 5.3.22 and in TS 36.579-5 different |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Message flow and specific message content added and adapted accordingly - Group name aligned to TS 36.579-5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading procedures and message contents may result in incorrect MCPTT test case implementation, test cases cannot be validated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.22	Generic Test Procedure for NW initiated temporary group creation

5.3.22.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3.22.2	Definition of system information messages

-

5.3.22.3	Procedure

Table 5.3.22.3-1: NW initiated temporary group creation

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCPTT server) sends a SIP NOTIFY to the UE informing that the user profile has changed, containing the XCAP-URI of the MCPTT User Profile Configuration Document. | <-- | SIP NOTIFY | - | - |
| 2 | The UE sends a SIP 200 (OK) message to the SS. | --> | SIP 200 (OK) | - | - |
| 2A | The UE (MCPTT client) sends an HTTP GET Request message to the SS t that contains the access token and the XCAP-URI of the MCPTT User Profile Configuration Document | --> | HTTP GET | - | - |
| 2B | The SS sends the HTTP 200 (OK) message including the MCPTT User Profile Configuration Document. | <-- | HTTP 200 (OK) | - | - |
| 2C | The UE (MCPTT client) sends a SIP SUBSCRIBE to the SS, containing the access token and a resource list mime body with the new Group to be obtained. | --> | SIP SUBSCRIBE | - | P |
| 2D | The SS sends a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| - | EXCEPTION: In parallel to procedure of all steps below the behaviour of Table 5.3.2.3-2C Group communication key retrieval procedure takes place | - | - | - | - |
| 2E | The SS sends a SIP NOTIFY message to the UE that contains the XCAP-URI of the Group documents. | <-- | SIP NOTIFY | - | - |
| - | EXCEPTION: The order of steps 4 and 5 depends on UE and SS implementation and is not checked by the implementation | - | - | - | - |
| 2F | The UE (MCPTT client) sends a SIP 200 (OK) message. | --> | SIP 200 (OK) | - | P |
| 3 | The UE (MCPTT client) sends an HTTP GET Request message to the SS that contains the access token and the XCAP-URI of the Group Configuration document. | --> | HTTP GET | - | - |
| 4 | The SS (MCPTT server) sends the HTTP 200 (OK) message including the updated Group Document | <-- | HTTP 200 (OK) | - | - |
| - | EXCEPTION: Steps 5a1-5a2 describe behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that takes place when one or the other is the case. | - | - | - | - |
| 5a1 | IF the Resource-Lists received from the UE at step 2C contains an entry referring to an MCPTT-GKTP document THEN the SS (MCPTT server) sends a SIP NOTIFY message to the UE containing the group key transport payloads (GKTP) document including the temporary group keys. | <- | SIP NOTIFY | - | - |
| 5a2 | The UE (MCPTT client) sends a SIP 200 (OK) message. | --> | SIP 200 (OK) | - | - |
| 6 | Void | - | - | - | - |

5.3.22.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3.22.4-1: SIP NOTIFY (Step 1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1, condition CONFIG |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |  |
| MIME body part |  |  | xcap-diff |  |  |
| MIME-part-body |  | Xcap-diff as described in Table 5.3.22.4-1A |  |  |  |

Table 5.3.22.4-1A: Xcap-diff document in SIP NOTIFY (Table 5.3.22.4-1)

| Derivation Path: RFC 5874 [107] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| xcap-diff |  |  |  |  |
| document |  |  |  |  |
| sel | org.3gpp.mcptt.user-profile & "/users/" & "sip:" & px_MCPTT_ID_User_A & "/mcptt-user-profile-" & 0 & ".xml" | Editor’s note: to be clarified whether only the path to the additional attribute shall be sent |  |  |
| new-etag | same value as used in notification procedure for user profile change before - appendage increased by 1 |  |  |  |
| previous-etag | same as used in notification procedure for user profile change before |  |  |  |

Table 5.3.22.4-2: SIP 200 (OK) (Steps 2, 6)

Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1

Table 5.3.22.4-2A: HTTP GET (Step 2, Table 5.3.22.3-1)

Derivation Path: Table 5.5.4.2-1, condition UEUSERPROF.

Table 5.3.22.4-2C: HTTP 200 (OK) (Step 6, Table 5.3.22.3-1)

| Derivation Path: Table 5.5.4.6-1, condition UEUSERPROF |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| mcptt-user-profile | User Profile Document described in Table 5.3.22.4-2D | User Profile Document returned |  |  |

Table 5.3.22.4-2D: User Profile Document in HTTP 200 (OK) (Table 5.3.22.4-2C)

| Derivation Path: Table 5.5.8.3-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-user-profile |  |  |  |  |
| OnNetwork |  |  |  |  |
| MCPTTGroupInfo |  |  |  |  |
| entry[2] |  | Group 2 the MCPTT user is allowed to affiliate to |  |  |
| index attribute | "1" |  |  |  |
| uri-entry | px_MCPTT_Group_T_ID | The MCPTT group ID for the on-network MCPTT group that the MCPTT user is allowed to affiliate to. |  |  |
| display-name | px_MCPTT_Group_T_Name | The display name for the group |  |  |

Table 5.3.22.4-2E: SIP SUBSCRIBE (Step 1, Table 5.3.22.3-1)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource-lists |  |  |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.1-1 with condition GROUPCONFIG_T |  |  |  |

Table 5.3.22.4-2F: SIP NOTIFY (Step 3, Table 5.3.22.3-1)

| Derivation Path: Table 5.5.2.8-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  |  |  |  |
| MIME part body | xcap-diff document as described in Table 5.3.22.4-2G |  |  |  |

Table 5.3.22.4-2G: Xcap-diff document in SIP NOTIFY (Table 5.3.22.4-2F)

| Derivation Path: Table 5.5.3.12-2, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| xcap-diff |  |  |  |  |
| document[1] |  |  |  |  |
| sel | "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_T_ID |  |  |  |

Table 5.3.22.4-3: HTTP GET (Step 3)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.2-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | "/" & tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_T_ID | points to group configuration document |  |  |

Table 5.3.22.4-4: HTTP 200 (OK) (Step 4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  | GROUPCONFIG |
| ue-group-configuration | As described in Table 5.3.22.4-5 | Group Configuration document returned |  |  |

Table 5.3.22.4-5: Group Configuration document (Table 5.3.22.4-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.7.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service[1] |  |  |  |  |
| mcpttgi:on-network-temporary |  |  | TS 24.481 [31] clause 7.2.4.2 |  |
| temporary-MCPTT-group-requestor attribute | px_MCPTT_ID_User_B | Identity of the responsible for formatting the MCS temporary group. | TS 24.481 [31] clause 7.2.4.2 |  |

Table 5.3.22.4-5A: SIP SUBSCRIBE (Step 2a1, Table 5.3.2.3-2C)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource-lists |  |  |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.1-1 with condition GROUPKEY_T |  |  |  |

Table 5.3.22.4-6: SIP NOTIFY (Step 5)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Message-body |  |  |  |  |
| xcap-diff document | xcap-diff document as described in Table 5.3.22.4-7 |  |  |  |

Table 5.3.22.4-7: xcap-diff document for MCX group configuration (Table5.3.22.4-6)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.12-2, condition GROUPKEY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| xcap-diff | encrypted according to NOTE 1 of Table 5.5.3.12-2 |  |  |  |
| element[1] | group key transport payloads (GKTP) document as described in Table 5.3.22.4-8 |  |  |  |
| sel | Doc-Sel & "~~" & Node-Sel | Document and node selector for Group T according to NOTEs 2a, 2b and 3 of Table 5.5.3.12-2 |  |  |

Table 5.3.22.4-8: group key transport payloads (GKTP) document (Table 5.3.22.4-7)

| Derivation Path: TS 24.481 [11] clause 7.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| GKTPs |  |  |  |  |
| GMK-GKTPs |  |  |  |  |
| GKTP[1] | MIKEY message as used in group communication key retrieval procedure | MIKEY message containing the GMK for Group A | TS 33.180 [94] |  |
| id attribute | Same value as used in group communication key retrieval procedure |  |  |  |
| on-network-regrouped-GKTPs |  |  |  |  |
| temporary-MCPTT-group-ID attribute | px_MCPTT_Group_T_ID |  |  |  |
| GKTP[1] | MIKEY message as described in Table 5.3.22.4-9 | MIKEY message containing the GMK for Group T | TS 33.180 [94] |  |
| id attribute | arbitrary value | unique charstring assigned by the SS |  |  |

Table 5.3.22.4-9: MIKEY-SAKKE I_MESSAGE (GMK distribution by the SS) (Table 5.3.22.4-8)

| Derivation Path: TS 36.579-1 [2], Table 5.5.9.1-3 |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| General Extension Payload { |  |  |  |
| Data { |  | See TS 33.180 [94] clause E.6 |  |
| Group IDs { |  |  |  |
| Number of Group IDs | '1' |  |  |
| Group ID | px_MCPTT_Group_T_ID | The ID for the group associated with the key. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |