3GPP TSG RAN WG5 Meeting #91-e	R5-212384

# Electronic Meeting, 17th - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2139 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TC 11.4.11 N26 interface not supported - S1 to N1 transfer of an existing emergency PDN connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Handling of the Emergency call after it is established is not specified. Wrong steps in generic procedure are referenced. There is no need to handle PDU sessions establishment on new RRC connection (TP is satisfied before). Possibility for IMS re-registration depending on implementation is not handled Conformance requirements text is missing (clarifying that the UE is not mandated to support PDU transfer and hence the TC will be then applicable only to UEs which do support the transfer) Specific message content is incorrect and missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Handling of the Emergency call after it is established is specified. Wrong steps numbers in generic procedure being referenced are corrected. Handle PDU sessions establishment on new RRC connection is removed. Possibility for IMS re-registration depending on implementation is added Conformance requirements text is added (clarifying that the UE is not mandated to support PDU transfer and hence the TC will be then applicable only to UEs which do support the transfer) Specific message content is corrected and missing specific message is specified |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test is incorrect. Correctly implemented UEs may be failed and non-conformant ones may pass. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 11.4.11	5GMM-REGISTERED.NORMAL-SERVICE / N26 interface not supported / S1 mode to N1 mode transfer of an existing emergency PDN connection

11.4.11.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED and EMM-IDLE on an E-UTRA cell, and, the UE has an emergency PDN connection established }

ensure that {

when { UE performs a inter-system change from the S1 to N1 mode, to an NR Cell belonging to a NWK supporting Interworking without N26 interface }

then { UE correctly transfers the existing emergency PDN connection into an emergency PDU session }

}

11.4.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501 [22], subclause 6.1.4.2, TS 22.101 [42], subclause 10.1.1. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, subclause 6.1.4.2]

When the network does not support N26 interface, the SMF does not provide the UE with the mapped EPS bearer context for a PDU session.

NOTE 1:	Since the SMF does not provide the UE with the mapped EPS bearer context for a PDU session, the UE does not know whether interworking with EPS is supported for a PDU session before attempting to transfer the PDU session from N1 mode to S1 mode.

NOTE 2:	It is up to UE implementation to decide which PDU session(s) to be attempted to transfer from N1 mode to S1 mode, e.g. based on UE policy or UE local configuration.

...

Upon inter-system change from S1 mode to N1 mode in 5GMM-IDLE mode, the UE uses the parameters from the default EPS bearer context of each PDN connection which the UE intends to transfer to 5GS and for which the UE has allocated a PDU session identity to create a PDU SESSION ESTABLISHMENT REQUEST message as follows:

a)	if the PDN connection is for emergency bearer services, the request type shall be set to "existing emergency PDU session". Otherwise the request type shall be set to "existing PDU session";

b)	the PDN type of the default EPS bearer context shall be mapped to the PDU session type of the PDU session as follows:

...

2)	if the PDN type is "IPv4" the PDU session type is set to "IPv4";

3)	if the PDN type is "IPv6", the PDU session type is set to "IPv6"; and

4)	the PDN type shall be set to "IPv4v6" if the PDU session type is "IPv4v6";

c)	the APN of the default EPS bearer context shall be mapped to the DNN of the PDU session;

d)	the PDU session ID shall be set to the PDU session identity included by the UE in the Protocol configuration options IE or Extended protocol configuration options IE in the PDN CONNECTIVITY REQUEST message, or to the PDU session ID associated with the default EPS bearer context; and

e)	the S-NSSAI of the PDU session shall be set to the S-NSSAI included by the network in the Protocol configuration options IE or Extended protocol configuration options IE in the ACTIVATE DEFAULT EPS BEARER REQUEST message, if provided by the network, or the S-NSSAI associated with the default EPS bearer context, if available.

NOTE 5:	If T3584 is running or deactivated for the S-NSSAI and optionally the DNN combination, the UE is allowed to initiate ESM procedures in EPS with or without APN corresponding to that DNN, and if the APN is congested in EPS, the MME can send a back-off timer for the APN to the UE as specified in 3GPP TS 24.301 [15].

The UE shall locally release the PDN connection(s) and EPS bearer(s) associated with the 3GPP access which have not been transferred to 5GS.

[TS 22.101, subclause 10.1.1]

The ME shall identify an emergency number dialled by the end user as a valid emergency number and initiate emergency call establishment if it occurs under one or more of the following conditions. If it occurs outside of the following conditions, the ME should not initiate emergency call establishment but normal call establishment. Emergency number identification takes place before and takes precedence over any other (e.g. supplementary service related) number analysis.

a)	112 and 911 shall always be available. These numbers shall be stored on the ME.

11.4.11.3	Test description

11.4.11.3.1	Pre-test conditions

System Simulator:

-	2 cells

-	E-UTRA Cell 1 as defined in TS 36.508 [7] Table 4.4.2-2. System information combination 31 as defined in TS 36.508 [7], subclause 4.4.3.1.1.

-	NR Cell 1 as defined in TS 38.508-1 [4] Table 4.4.2-3. System information combination NR-6 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2.

UE:

None.

Preamble:

-	With NR Cell 1 "Serving cell" and E-UTRA Cell 1 "Non-suitable "Off" cell", the UE is brought to state 1N-A, RRC_IDLE Connectivity (NR), in accordance with the procedure described in TS 38.508-1 [4], subclause 4.5.2 (Table 4.5.2.2-2 and depending on UE implementation Table 4.5.2.2-4). 5G-GUTI and ngKSI are assigned and security context established. During the registration the NWK indicates Interworking without N26 interface supported in accordance with the definition specified in TS 38.508-1 [4], subclause 4.5.1.

-	the UE is switched-off

-	With E-UTRA Cell 1 "Serving cell" and NR Cell 1 "Non-suitable "Off" cell", the UE is brought to state RRC_IDLE Connectivity (E-UTRA/EPC) in accordance with the procedure described in TS 38.508-1 [4], Table 4.5.2.2-1. 4G GUTI and eKSI are assigned and security context established

11.4.11.3.2	Test procedure sequence

Table 11.4.11.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE attempt an IMS emergency call dialling an emergency number. (NOTE 1) | - | - | - | - |
| - | The following messages are to be observed on E-UTRA Cell 1 unless explicitly stated otherwise. | - | - | - | - |
| 2-15 | Steps 2-15, from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508-1 [7], subclause 4.5A.4 take place. | - | - | - | - |
| 15A | Make the UE release the IMS emergency call. (NOTE 1) | - | - | - | - |
| 15B | The Generic test procedure for MO release of IMS call - EPS as defined in TS 34.229-1 [35], subclause C.32, takes place. | - | - | - | - |
| 15C | The Generic Test Procedure for EPS Bearer Deactivation TS 36.508-1 [7], subclause 4.5A.15, take place. The SS deactivates the dedicated for IMS voice EPS bearer context of the Emergency PDN connection. The default bearer is kept i.e the Emergency session is not released. | - | - | - | - |
| 16 | SS releases the RRC connection on E-UTRA. | <-- | RRC: RRCConnectionRelease | - | - |
| 17 | The SS configures: - NR Cell 1 as "Serving cell" - E-UTRA Cell 1 as "Non-suitable "off" cell". | - | - | - | - |
| - | The following messages are to be observed on NR Cell 1 unless explicitly stated otherwise. | - | - | - | - |
| 18-28 | Steps 1-11 from the test procedure for UE for Tracking area updating / Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode as specified in TS 38.508-1 [4], Table 4.9.9, take place. | - | - | - | - |
| - | EXCEPTION: In parallel to step 29Aa1 below the steps described in table 11.4.11.3.2-2 take place. | - | - | - | - |
| 29a1 | Step 14a1 from the test procedure for UE for Tracking area updating / Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode as specified in TS 38.508-1 [4], Table 4.9.9.2.2-1, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. NOTE: The pc_noOf_PDUsSameConnection covers the "normal" PDUs the UE will establish when it attaches to 5GC. This does not include the PDU connection for Emergency services - this is handled separately and in parallel in Table 11.4.11.3.2-2. | - | - | - | - |
| 29a2a1-29b3a9 | Void | - | - | - | - |
| 30 | Wait 3 sec. | - | - | - | - |
| 31 | The SS transmits a RRCReconfiguration message and a PDU SESSION RELEASE COMMAND message to release the Emmergency session. | <-- | NR RRC: RRCReconfiguration NR NAS: PDU SESSION RELEASE COMMAND | - | - |
| 32 | The UE transmits a RRCReconfigurationComplete | - | NR RRC: RRCReconfigurationComplete | - | - |
| 33 | The UE transmits a PDU SESSION RELEASE COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION RELEASE COMPLETE | - | - |
| 34 | SS releases the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| NOTE 1: This could be done by e.g. MMI or AT command. |  |  |  |  |  |

Table 11.4.11.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit an UL NAS TRANSPORT message with 'Request type' set to "existing emergency PDU session", and, a PDU SESSION ESTABLISHMENT REQUEST? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 1 | P |
| 2 | The SS transmits an RRCReconfiguration message and an PDU SESSION ESTABLISHMENT ACCEPT. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| - | EXCEPTION: Step 3a1 describes a step sequence depending on UE implementation. | - | - | - | - |
| 3a1 | IMS re-registration on NR as specified in TS 34.229-5 [41], annex A.12, takes place. | - | - | - | - |

11.4.11.3.3	Specific message contents

Table 11.4.11.3.3-1: REGISTRATION ACCEPT (Preamble)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7, condition EMERGENCY |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS network feature support | ‘0100 1101 0000 0000’B | IMS voice over PS session supported over 3GPP access, Emergency services supported in NR connected to 5GCN and E-UTRA connected to 5GCN. Interworking without N26 interface supported (octet 3, bit 7) All other features set to "not supported". | Interworking without N26 interface supported |

Table 11.4.11.3.3-2: PDU SESSION ESTABLISHMENT REQUEST (step 1, Table 11.4.11.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDU session ID | Shall be set to the PDU session identity included by the UE in the Protocol configuration options IE or Extended protocol configuration options IE in the PDN CONNECTIVITY REQUEST message, or to the PDU session ID associated with the default EPS bearer context |  |  |
| PTI | A value currently not used |  |  |
| SSC mode | '001'B | SSC mode 1 |  |

Table 11.4.11.3.3-3: UL NAS TRANSPORT (step 1, Table 11.4.11.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-10, condition INITIAL_PDU_REQUEST. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| Request type | '100'B | existing emergency PDU session |  |
| S-NSSAI |  | Not Present |  |  |  |  |  |
| DNN |  | Not Present |  |  |  |  |  |