3GPP TSG-RAN5 Meeting #91-e	R5-212637

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5302 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding RMCs for subPRB allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the efeMTC WP, subPRB allocation should be added to several test cases. RMC is missing for subPRB allocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligning RMC for subPRB allocation with 36.101 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | efeMTC WI cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.2.4 (new) A.2.3.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is dependent on RAN4 CR R4-XXXXXX |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

### A.2.2.3	Reference measurement channels for sustained downlink data rate provided by lower layers

Table A.2.2.3-1: Uplink Reference Channels for sustained data-rate test (FDD)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference Channel |  | R.1-1 FDD | R.1-2 FDD | R.1-3 FDD | R.1-3A FDD | R.1-4 FDD | R.1-4B FDD |
| Channel Bandwidth | MHz | 10 | 10 | 20 | 10 | 20 | 15 |
| Allocated Resource Blocks |  | 40 (Note 2) | 40 (Note 2) | 90 (Note 3) | 40(Note 2) | 90 (Note 3) | 60 (Note 4) |
| Allocated Sub-Frames per Radio-Frame |  | 10 | 10 | 10 | 10 | 10 | 10 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Coding Rate |  | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 |
| Information Bit Payload per Sub-Frame | Bits | 3496 | 3496 | 7992 | 3496 | 7992 | 5352 |
| Number of Code Blocks per Sub-Frame (Note 1) |  | 1 | 1 | 2 | 1 | 2 | 2 |
| Modulation Symbols per Sub-Frame |  | 5760 | 5760 | 12960 | 5760 | 12960 | 5760 |
| Binary Channel Bits per Sub-Frame |  | 11520 | 11520 | 25920 | 11520 | 25920 | 17260 |
| Max Throughput over 1 Radio-Frame | Mbps | 3.496 | 3.496 | 7.992 | 3.496 | 7.992 | 5.352 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 1 | ≥ 2 | ≥ 1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: RB-s 5-44 allocated with PUSCH. Note 3: RB-s 5-94 allocated with PUSCH. Note 4: RB-s 7-66 allocated with PUSCH. |  |  |  |  |  |  |  |

### A.2.2.4	subPRB allocation

### The location of allocated RB for subPRB allocation is chosen according to values specified in the Tx requirements.

Table A.2.2.4-1: Reference Channels for SubPRB allocation

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 – 20 | 1.4 – 20 | 1.4 – 20 |
| Allocated resource blocks |  | 1 | 1 | 1 |
| Number of subcarriers |  | 2 out of 3 | 3 | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 |
| Modulation |  | π/2 BPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 |
| Payload size | Bits | 32 | 72 | 72 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| Number of code blocks |  | 1 | 1 | 1 |
| Total number of bits per resource unit | Bits | 192 | 288 | 288 |
| Total symbols per resource unit |  | 192 | 144 | 144 |
| Tx time | ms | 8 | 4 | 2 |
| UE UL Category |  | M1/M2 | M1/M2 | M1/M2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |

<< Clauses skipped here>>

### A.2.3.3	Reference measurement channels for sustained downlink data rate provided by lower layers

Table A.2.3.3-1: Uplink Reference Channels for sustained data-rate test (TDD)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference Channel |  | R.1-1 TDD | R.1-2 TDD | R.1-3 TDD | R.1-3B TDD | R.1-4 TDD | R.1-5 TDD |
| Channel Bandwidth | MHz | 10 | 10 | 20 | 15 | 20 | 10 |
| Uplink-Downlink Configuration (Note 2) |  | 5 | 5 | 5 | 1 | 1 | 1 |
| Allocated Resource Blocks |  | 40 (Note 3) | 40 (Note 3) | 90 (Note 5) | 60 (Note 4) | 90 (Note 5) | 40 (Note 3) |
| Allocated Sub-Frames per Radio-Frame |  | 1 | 1 | 1 | 1 | 1 | 1 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Coding Rate |  |  |  |  |  |  |  |
| For Sub-Frame 2 |  | 0.43 | 0.61 | 0.49 | 0.42 | 0.49 | 0.61 |
| For Sub-Frame 3,7,8 |  | n/a | n/a | n/a | 0.42 | 0.49 | 0.61 |
| Information Bit Payload per Sub-Frame | Bits |  |  |  |  |  |  |
| For Sub-Frame 2 |  | 4968 | 6968 | 12576 | 7224 | 12576 | 6968 |
| For Sub-Frame 3,7,8 |  | 0 | 0 | 0 | 7224 | 12576 | 6968 |
| Number of Code Blocks per Sub-Frame (Note 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2 |  | 1 | 2 | 3 | 2 | 3 | 2 |
| For Sub-Frame 3,7,8 |  | 0 | 0 | 0 | 2 | 3 | 2 |
| Modulation Symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2 |  | 5760 | 5760 | 12960 | 8640 | 10240 | 5760 |
| For Sub-Frame 3,7,8 |  | 0 | 0 | 0 | 8640 | 10240 | 5760 |
| Binary Channel Bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2 |  | 11520 | 11520 | 25920 | 17280 | 25920 | 11520 |
| For Sub-Frame 3,7,8 |  | n/a | n/a | n/a | 17280 | 25920 | 11520 |
| Max Throughput over 1 Radio-Frame | Mbps | 0.4968 | 0.6968 | 1.2576 | 2.8896 | 5.0304 | 2.7872 |
| UE Category |  | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: As per Table 4.2-2 in TS 36.211 [8] Note 3: RB-s 5-44 allocated with PUSCH. Note 4: RB-s 7-66 allocated with PUSCH. Note 5: RB-s 5-94 allocated with PUSCH. |  |  |  |  |  |  |  |

### A.2.3.4	subPRB allocation

### The location of allocated RB for subPRB allocation is chosen according to values specified in the Tx requirements.

Table A.2.3.4-1: Reference Channels for SubPRB allocation

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 – 20 | 1.4 – 20 | 1.4 – 20 |
| Allocated resource blocks |  | 1 | 1 | 1 |
| Number of subcarriers |  | 2 out of 3 | 3 | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 |
| Modulation |  | π/2 BPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 |
| Payload size | Bits | 32 | 72 | 72 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| Number of code blocks |  | 1 | 1 | 1 |
| Total number of bits per resource unit | Bits | 192 | 288 | 288 |
| Total symbols per resource unit |  | 192 | 144 | 144 |
| Tx time | ms | 8 | 4 | 2 |
| UE UL Category |  | M1/M2 | M1/M2 | M1/M2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |

<< End of changes>>