3GPP TSG-RAN5 Meeting #91-e	R5-212664

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0313 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of derivation paths to 38.508-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Several message exception tables have incorrect derivation path to table 5.4.2-X in 38.521-1. The derivation path should instead be to table 5.4.3.1-X. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All incorrect derivation paths to table 5.4.2-X in 38.521-1 is replaced by 5.4.3.1-X. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect derivation paths will remain in 38.521-4. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.1_1.3.3_1, 5.2.2.1.2_1.3.3_1, 5.2.2.1.3_1.3.3_1, 5.2.2.1.4_1.3.3_1, 5.2.2.1.4_1.3.3_2, 5.2.2.2.1_1.3.3_1, 5.2.2.2.2_1.4.3_1, 5.2.3.1.1_1.3.3_1, 5.2.3.1.3_1.3.3_1, 5.2.3.2.1_1.3.3_1, 5.2.3.2.2_1.3.3_1, 5.2.3.2.3_1.3.3_1, 5.3.2.1.1.4.3.1, 5.3.2.1.2.4.3.1, 5.3.2.2.1.4.3.1, 5.3.2.2.2.4.3.1, 5.3.3.1.1.4.3.1, 5.3.3.1.2.4.3.1, 5.3.3.2.1.4.3.1, 5.3.3.2.2.4.3.1, 6.3.2.1.2.4.3.1, 6.3.2.2.1.4.3_1, 6.3.2.2.2.4.3_1, 6.3.3.1.2.4.3.1, 6.3.3.2.1.4.3.1, 6.3.3.2.2.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

5.2.2.1.1_1.3.3_1	Message exceptions for SA

Table 5.2.2.1.1_1.3.3_1-1: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | n4, n2 | n4 for test 1-1 n2 for other tests |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.1.1_1.3.3_1-2: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos2 | For test 1-1 and 1-5 |  |
|  | pos1 | For other tests |  |
| } |  |  |  |

Table 5.2.2.1.1_1.3.3_1-3: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n8, n4 | n8 for Test 1-4 n4 for other tests |  |
| } |  |  |  |

Table 5.2.2.1.1_1.3.3_1-4: CSI-ResourcePeriodicityAndOffset for CSI Tracking

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots10 | 1 (for CSI-RS resources 1 and 2) 2 (for CSI-RS resources 3 and 4) | For test 1-5: CSI-RS offset: 1 for CSI-RS resources 1 and 2 2 for CSI-RS resources 3 and 4 CSI-RS periodicity: 10 slots |  |
| slots40 | 20 (for CSI-RS resources 1 and 2) 21 (for CSI-RS resources 3 and 4) | For test 2-2: CSI-RS offset: 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 CSI-RS periodicity: 40 slots |  |
| slots20 | 10 (for CSI-RS resources 1 and 2) 11 (for CSI-RS resources 3 and 4) | For other tests: CSI-RS offset: 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 CSI-RS periodicity: 20 slots |  |
| } |  |  |  |

<< Clauses skipped here>>

5.2.2.1.2_1.3.3_1	Message exceptions for SA

Same as for test number 1-2 in 5.2.2.1.1_1.3.3_1 with following exceptions:

Table 5.2.2.1.2_1.3.3_1-1: NZP CSI-RS-ResourceMapping for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 13 | l0 = 13 |  |
| } |  |  |  |

Table 5.2.2.1.2_1.3.3_1-2: CSI-ResourcePeriodicityAndOffset for ZP and NZP CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 0 | CSI-RS offset: 0 CSI-RS periodicity: 5 slots |  |
| } |  |  |  |

Table 5.2.2.1.2_1.3.3_1-3: ZP CSI-RS-ResourceMapping for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], clause5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 011110 | (k0, k1, k2, k3)=(2, 4, 6, 8) |  |
| } |  |  |  |
| nrofPorts | P8 | Number of CSI-RS ports (X) = 8 |  |
| firstOFDMSymbolInTimeDomain | 12 | l0 = 12 |  |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL | Density (ρ) = 1 |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation | Frequency Occupation: Start PRB 0 (see Table 4.6.3-33 in TS 38.508-1) Number of PRB = 52 (see Table5.4.2.0-23:in TS 38.508-1 [6]. |  |
| } |  |  |  |

Table 5.2.2.1.2_1.3.3_1-4: Void

<< Clauses skipped here>>

5.2.2.1.3_1.3.3_1	Message exceptions for SA

Table 5.2.2.1.3_1.3.3_1-1: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n4 |  |  |
| } |  |  |  |

Table 5.2.2.1.3_1.3.3_1-2: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF { | 2 entry |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeB |  |  |
| startSymbolAndLength | 89 | Start symbol(S)=5, Length(L)=7 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.1.3_1.3.3_1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeB CHOICE { |  |  |  |
| setup | DMRS-DownlinkConfig |  |  |
| } |  |  |  |
| } |  |  |  |

<< Clauses skipped here>>

5.2.2.1.4_1.3.3_1	Message exceptions for SA

As defined in clause 5.4.2 of TS 38.508-1 [6] with the following exceptions:

Table 5.2.2.1.4_1.3.3_1-1: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF { | 2 entry |  | FR1 |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| k0 | 0 |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 94 | Start symbol(S)=3, Length(L)=9 for Test 1-1 |  |
|  | 66 | Start symbol(S)=3, Length(L)=11 for Test 1-2 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  |  |  |
| k0 | 0 |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 66 | Start symbol(S)=3, Length(L)=11 for Test 1-2 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.1.4_1.3.3_1-2: SearchSpace

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-162 and5.4.2.0-7 using condition USS, FR1_10MHz, Long_DCI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSymbolsWithinSlot | 00100000000000 |  |  |
| } |  |  |  |

Table 5.2.2.1.4_1.3.3_1-3: ServingCellConfigCommon

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| dmrs-TypeA-Position | pos3 |  |  |
| lte-CRS-ToMatchAround | RateMatchPatternLTE-CRS |  |  |
| } |  |  |  |

Table 5.2.2.1.4_1.3.3_1-4: RateMatchPatternLTE-CRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternLTE-CRS ::= SEQUENCE { |  |  |  |
| carrierFreqDL | Same as NR carrier centre subcarrier location |  |  |
| carrierBandwidthDL | n50 | 10MHz |  |
| mbsfn-SubframeConfigList | Not present |  |  |
| nrofCRS-Ports | n4 |  |  |
| v-Shift | n0 |  |  |
| } |  |  |  |

<< Clauses skipped here>>

5.2.2.1.4_1.3.3_2	Message exceptions for NSA

Same as 5.2.2.1.4_1.3.3_1 with the following exceptions:

Table 5.2.2.1.4_1.3.3_2-1: SearchSpace for CSS

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-162 and5.4.2.0-7 using condition USS, FR1_10MHz, Long_DCI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | SearchSpaceId with condition CSS |  | CSS |
| controlResourceSetId | 1 |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| duration | Not present | 1 slot per default |  |
| monitoringSymbolsWithinSlot | 00100000000000 |  |  |
| } |  |  |  |

<< Clauses skipped here>>

5.2.2.2.1_1.3.3_1	Message exceptions for SA

Table 5.2.2.2.1_1.3.3_1-1: Void

Table 5.2.2.2.1_1.3.3_1-2: Void

Table 5.2.2.2.1_1.3.3_1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-DownlinkConfig |  |  |
| } |  |  |  |
| mcs-Table | qam256 | 256qam table for test 1-3 |  |
|  | Not present | 64qam table for all tests except test 1-3 |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | n4, n2 | n4 for test 1-1 n2 for other tests |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.2.1_1.3.3_1-4: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos2 | For tests 1-1, 1-7, 1-8, and 1-9 |  |
|  | pos1 | For other tests |  |
| } |  |  |  |

Table 5.2.2.2.1_1.3.3_1-5: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n16, n10, n8 | n16 for Test 1-4, n10 for Test 1-9 n8 for other tests |  |
| } |  |  |  |

Table 5.2.2.2.1_1.3.3_1-6: RACH-ConfigGeneric

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 163 | Only for test 2-2 |  |
| } |  |  |  |

Table 5.2.2.2.1_1.3.3_1-7: CSI-ResourcePeriodicityAndOffset for CSI Tracking

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots20 | 1 (for CSI-RS resources 1 and 2) 2 (for CSI-RS resources 3 and 4) | For test 1-7: CSI-RS offset: 1 for CSI-RS resources 1 and 2 2 for CSI-RS resources 3 and 4 CSI-RS periodicity: 20 slots |  |
| Slots40 | 20 (for CSI-RS resources 1 and 2) 21 (for CSI-RS resources 3 and 4) | For other tests: CSI-RS offset: 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 CSI-RS periodicity: 40 slots |  |
| } |  |  |  |

Table 5.2.2.2.1_1.3.3_1-8: CSI-FrequencyOccupation for CSI Tracking

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| nrofRBs | 52 | 52 for tests 1-7, 2-2 |  |
|  | 108 | 108 for other tests |  |
| } |  |  |  |

Table 5.2.2.2.1_1.3.3_1-9: SchedulingRequestResourceConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl20 | 7 | For test 1-9 |  |
| sl20 | 5 | For test 2-2 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.2.1_1.3.3_1-10: Physical layer parameters for DCI format 1_1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PUCCH resource indicator | PUCCH-ResourceId[1] = 6 in pucch-ResourceSetID[1] or PUCCH-ResourceId[1] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘110’B | Slot S1 for test 1-9 |

<< Clauses skipped here>>

5.2.2.2.2_1.4.3_1	Message exceptions for SA

Table 5.2.2.2.2_1.4.3_1-1: Void

Table 5.2.2.2.2_1.4.3_1-2: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPDSCH | 0 |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA CHOICE { |  |  |  |
| Setup | DMRS-DownlinkConfig |  |  |
| } |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Used_for_Type0 |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize |  | If a bundleSize(Set) value is absent, the UE applies the value n2. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.2.2_1.4.3_1-3: Void

Table 5.2.2.2.2_1.4.3_1-4: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP |  |
| tci-StatesPDCCH-ToAddList { |  |  |  |
|  | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.2.2_1.4.3_1-5: Void

Table 5.2.2.2.2_1.4.3_1-6: NZP CSI-RS-ResourceMapping for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 13 | l0 = 13 |  |
| } |  |  |  |

Table 5.2.2.2.2_1.4.3_1-7: CSI-ResourcePeriodicityAndOffset for CSI Acquisition for NZP CSI-RS

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots5 | 0 | Periodicity 5 slots and offset 0 |  |
| } |  |  |  |

Table 5.2.2.2.2_1.4.3_1-8: ZP CSI-RS-ResourceMapping for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| nrofPorts | P8 | Eight Ports |  |
| } |  |  |  |

Table 5.2.2.2.2_1.4.3_1-9: DMRS-DownlinkConfig

| Derivation Path: TS 38.508 [6], Table5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | Not present | pos2 If the field is absent, the UE applies the value pos2 | FR1_TDD, |
| } |  |  |  |

Table 5.2.2.2.2_1.4.3_1-10: CSI-ResourcePeriodicityAndOffset for CSI Acquisition for ZP CSI-RS

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots5 | 0 | Periodicity 5 slots and offset 0 |  |
| } |  |  |  |

<< Clauses skipped here>>

5.2.3.1.1_1.3.3_1	Message exceptions for SA

Table 5.2.3.1.1_1.3.3_1-1: BWP

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP ::= SEQUENCE { |  |  |  |
| locationAndBandwidth | 13750 | For Test 2-2 (20MHz BW, SCS 30kHz) |  |
|  | 14025 | For other tests (10MHz BW, SCS 15kHz) |  |
| } |  |  |  |

Table 5.2.3.1.1_1.3.3_1-2: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 | resourceAllocationType0 for all tests except test 1-2 |  |
|  | resourceAllocationType1 | resourceAllocationType1 for test 1-2 |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | n4 | n4 for test 1-1 |  |
|  | wideband | wideband for test 3-1 |  |
|  | Not present | n2 for other tests |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.3.1.1_1.3.3_1-3: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos1 | pos1 for all tests except test 1-1, 1-5, 1-6, 1-7 |  |
|  | Not present | pos2 for test 1-1, 1-5, 1-6, 1-7 |  |
| } |  |  |  |

Table 5.2.3.1.1_1.3.3_1-4: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | Not present | n8 for test 1-4, 2.1 |  |
|  | n4 | n4 for other tests |  |
| } |  |  |  |

Table 5.2.3.1.1_1.3.3_1-5: CSI-ResourcePeriodicityAndOffset for CSI Tracking

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots10 | 1 (for CSI-RS resources 1 and 2) 2 (for CSI-RS resources 3 and 4) | Periodicity 10 slots and offset 1/2 for test 1-5, 1-6, 1-7 |  |
| } |  |  |  |

<< Clauses skipped here>>

5.2.3.1.2_1.3.3_1	Message exceptions for SA

Table 5.2.3.1.2_1.3.3_1-1: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n4 |  |  |
| } |  |  |  |

Table 5.2.3.1.2_1.3.3_1-2: NZP CSI-RS-ResourceMapping for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row4 | 001 | k0=0 |  |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 13 | l0 = 13 |  |
| } |  |  |  |

Table 5.2.3.1.2_1.3.3_1-3: CSI-ResourcePeriodicityAndOffset for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 0 | Periodicity 5 slots and offset 0 |  |
| } |  |  |  |

Table 5.2.3.1.2_1.3.3_1-4: ZP CSI-RS-ResourceMapping for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], clause5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 011110 | (k0, k1, k2, k3)=(2, 4, 6, 8) |  |
| } |  |  |  |
| nrofPorts | P8 | Eight Ports |  |
| firstOFDMSymbolInTimeDomain | 12 | l0 = 12 |  |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

<< Clauses skipped here>>

5.2.3.1.3_1.3.3_1	Message exceptions for SA

Table 5.2.3.1.3_1.3.3_1-1: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n4 |  |  |
| } |  |  |  |

Table 5.2.3.1.3_1.3.3_1-2: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF { | 2 entry |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeB |  |  |
| startSymbolAndLength | 89 | Start symbol(S)=5, Length(L)=7 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | TypeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

<< Clauses skipped here>>

5.2.3.2.1_1.3.3_1	Message exceptions for SA

Table 5.2.3.2.1_1.3.3_1-1: BWP

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP ::= SEQUENCE { |  |  |  |
| locationAndBandwidth | 13750 | For Test 2-2 (20MHz BW, SCS 30kHz) |  |
|  | 28875 | For other tests (40MHz BW, SCS 30kHz) |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-2: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | n4 | n4 for tests 1-1, 1-8, 1-9 |  |
|  | wideband | wideband for test 3-1 |  |
|  | Not present | n2 for other tests |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-3: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos1 | pos1 for all tests except tests 1-1, 1-7, 1-8, 1-9 |  |
|  | Not present | pos2 for tests 1-1, 1-7, 1-8, 1-9 |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-4: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | Not present | n8 for other tests |  |
|  | n16 | n16 for test 1-4 |  |
|  | n10 | n10 for test 1-9 |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-5: CSI-ResourcePeriodicityAndOffset for CSI Tracking

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots20 | 1 (for CSI-RS resources 1 and 2) 2 (for CSI-RS resources 3 and 4) | Periodicity 20 slots and offset 1/2 for test 1-7 |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-5A: CSI-RS-ResourceMapping for TRS

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 4 | For Tests 1-8, 1-9: l0 = 4 for CSI-RS resource 1 and 3 | TRS |
|  | 8 | For Tests 1-8, 1-9: l0 = 8 for CSI-RS resource 2 and 4 | TRS |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-6: CSI-FrequencyOccupation for CSI Tracking

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| nrofRBs | 52 | 52 for tests 1-7, 2-2 | TRS |
|  | 108 | 108 for other tests | TRS |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-7: RACH-ConfigGeneric

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 163 | Only for test 2-2 |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-8: SchedulingRequestResourceConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl20 | 7 | For test 1-9 |  |
| sl20 | 5 | For test 2-2 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-9: Physical layer parameters for DCI format 1_1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PUCCH resource indicator | PUCCH-ResourceId[1] = 6 in pucch-ResourceSetID[1] or PUCCH-ResourceId[1] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘110’B | Slot S1 for test 1-9 |

<< Clauses skipped here>>

5.2.3.2.2_1.3.3_1	Message exceptions for SA

Table 5.2.3.2.2_1.3.3_1-1: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | Not present |  |  |
| } |  |  |  |

Table 5.2.3.2.2_1.3.3_1-2: NZP CSI-RS-ResourceMapping for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 13 | l0 = 13 |  |
| } |  |  |  |

Table 5.2.3.2.2_1.3.3_1-3: CSI-ResourcePeriodicityAndOffset for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 0 | Periodicity 5 slots and offset 0 |  |
| } |  |  |  |

Table 5.2.3.2.2_1.3.3_1-4: ZP CSI-RS-ResourceMapping for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], clause5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 011110 | (k0, k1, k2, k3)=(2, 4, 6, 8) |  |
| } |  |  |  |
| nrofPorts | P8 | Eight Ports |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

Table 5.2.3.2.2_1.3.3_1-4A: ZP CSI-ResourcePeriodicityAndOffset for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 0 | Periodicity 5 slots and offset 0 |  |
| } |  |  |  |

<< Clauses skipped here>>

5.2.3.2.3_1.3.3_1	Message exceptions for SA

Table 5.2.3.2.3_1.3.3_1-1: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | Not present |  |  |
| } |  |  |  |

Table 5.2.3.2.3_1.3.3_1-2: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF { | 2 entry |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeB |  |  |
| startSymbolAndLength | 89 | Start symbol(S)=5, Length(L)=7 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

<< Clauses skipped here>>

5.3.2.1.1.4.3.1	Message exceptions for SA

Table 5.3.2.1.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 3, 4, 5 |  |
|  | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP Test 1, 2 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 1, 2, 3, 5 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 4 |  |
| tci-StatesPDCCH-ToAddList { |  |  |  |
|  | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 11000000000000 | Symbols 0 and 1 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1, Test 2 |
| aggregationLevel4 | n1 | AL4 | Test 3, Test 4 |
| aggregationLevel8 | n0 |  |  |
| aggregationLevel16 | n1 | AL16 | Test 5 |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for tests 2 and 3 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for tests 1, 4, 5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.1.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: this table is a duplication of the aboveand was renumbered to 3A. Conflict has to be resolved!

Table 5.3.2.1.1.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1, Test 4, Test 5 |
| } |  |  |  |

<< Clauses skipped here>>

5.3.2.1.2.4.3.1	Message exceptions for SA

Table 5.3.2.1.2.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 2, 3 |  |
|  | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP Test 1 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 1, 2 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 3 |  |
| tci-StatesPDCCH-ToAddList { |  |  |  |
|  | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.2.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n1 | AL4 | Test 1 |
| aggregationLevel8 | n1 | AL8 | Test 2, 3 |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for tests 2 and 3 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for test 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.2.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1 |
| } |  |  |  |

<< Clauses skipped here>>

5.3.2.2.1.4.3.1	Message exceptions for SA

Table 5.3.2.2.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 3 |  |
|  | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP Test 1, 2 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 3 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 1, 2 |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  | Test 1, Test 2 |
| reg-BundleSize | n2 |  | 1 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| nonInterleaved | null |  | Test 3 |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList { | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1 |
| aggregationLevel4 | n1 | AL4 | Test 2 |
| aggregationLevel16 | n1 | AL16 | Test 3 |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for test 2 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for tests 1 and 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.1.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: following table was renumbered to 3A. Conflict has to be resolved!

Table 5.3.2.2.1.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1, Test 3 |
| } |  |  |  |

<< Clauses skipped here>>

5.3.2.2.2.4.3.1	Message exceptions for SA

Table 5.3.2.2.2.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 11111110 00000000 00000000 00000000 00000 | CORESET to use the least significant 90 RBs of the BWP Test 1 |  |
| Duration | 1 | SearchSpace duration of 1 symbols Test 1 |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  |  |
| reg-BundleSize | n6 |  | 2 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList { | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.2.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel8 | n1 | AL8 | Test 1 |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.2.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<< Clauses skipped here>>

5.3.3.1.1.4.3.1	Message exceptions for SA

Table 5.3.3.1.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 3, 4, 5 |  |
|  | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP Test 1, 2 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 1, 2, 3, 5 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 4 |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList { | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1, Test 2 |
| aggregationLevel4 | n1 | AL4 | Test 3, Test 4 |
| aggregationLevel16 | n1 | AL16 | Test 5 |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for tests 2 and 3 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for tests 1, 4, 5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.1.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: duplicated table below was renamed to 3A. Conflict has to be resolved!

Table 5.3.3.1.1.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1, Test 4, Test 5 |
| } |  |  |  |

<< Clauses skipped here>>

5.3.3.1.2.4.3.1	Message exceptions for SA

Table 5.3.3.1.2.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 2, 3 |  |
|  | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP Test 1 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 1, 2 |  |
|  | 1 | SearchSpace duration of 1 symbol Test3 |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList { | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.2.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | AL4 | Test 1, Test 3 |
| aggregationLevel8 | n1 | AL8 | Test 2 |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for tests 2 and 3 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for test 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.2.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: duplicated table below was renamed to 3A. Conflict has to be resolved!

Table 5.3.3.1.2.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1 |
| } |  |  |  |

<< Clauses skipped here>>

5.3.3.2.1.4.3.1	Message exceptions for SA

Table 5.3.3.2.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 3 |  |
|  | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP Test 1, 2 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 3 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 1, 2 |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  | Test 1, Test 2 |
| reg-BundleSize | n2 |  | 1 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| nonInterleaved | null |  | Test 3 |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList { | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1 |
| aggregationLevel4 | n1 | AL4 | Test 2 |
| aggregationLevel16 | n1 | AL16 | Test 3 |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for test 2 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for test 1 and 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.1.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: duplicated table below was renamed to 3A. Conflict has to be resolved!

Table 5.3.3.2.1.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1, Test 3 |
| } |  |  |  |

<< Clauses skipped here>>

5.3.3.2.2.4.3.1	Message exceptions for SA

Table 5.3.3.2.2.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 11111110 00000000 00000000 00000000 00000 | CORESET to use the least significant 90 RBs of the BWP Test 1 |  |
| Duration | 1 | SearchSpace duration of 1 symbols Test 1 |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  |  |
| reg-BundleSize | n6 |  | 2 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList { | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.2.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel8 | n1 | AL8 | Test 1 |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.2.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<< Clauses skipped here>>

6.3.2.1.2.4.3.1	Message exceptions for SA

Table 6.3.2.1.2.4.3.1-1: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| resourceType | aperiodic |  |  |
| } |  |  |  |

Table 6.3.2.1.2.4.3.1-2: CSI-RS-ResourceMapping for NZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 5.4.2, Table5.4.2.0-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 001100 |  |  |
| } |  |  |  |
| nrofPorts | p8 |  |  |
| firstOFDMSymbolInTimeDomain | 5 |  |  |
| cdm-Type | cdm4-FD2-TD2 |  |  |
| } |  |  |  |

Table 6.3.2.1.2.4.3.1-3: CSI-RS-ResourceMapping for ZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 5.4.2, Table5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 |  |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 |  |  |
| } |  |  |  |

Table 6.3.2.1.2.4.3.1-4: CSI-IM-Resource

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| csi-IM-ResourceElementPattern |  |  |  |
| pattern0 SEQUENCE { |  |  |  |
| subcarrierLocation-p0 | s4 |  |  |
| symbolLocation-p0 | 9 |  |  |
| } |  |  |  |

Table 6.3.2.1.2.4.3.1-5: CodebookConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrOfAntennaPorts CHOICE { |  |  |  |
| moreThanTwo SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| four-one-TypeI-SinglePanel-Restriction | FFFF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | 00000010 |  |  |

Table 6.3.2.1.2.4.3.1-6: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList | 5 |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands7 | 1111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<< Clauses skipped here>>

6.3.2.2.1.4.3_1	Message exceptions for SA

Table 6.3.2.2.1.4.3_1-1: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| resourceType | aperiodic |  |  |
| } |  |  |  |

Table 6.3.2.2.1.4.3_1-2: CSI-RS-ResourceMapping for NZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause5.4.2, Table 5.4.2.0-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| Row4 | 001 |  |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 13 |  |  |
| } |  |  |  |

Table 6.3.2.2.1.4.3_1-3: CSI-RS-ResourceMapping for ZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause5.4.2, Table5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 |  |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 |  |  |
| } |  |  |  |

Table 6.3.2.2.1.4.3_1-4: CSI-IM-Resource

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| csi-IM-ResourceElementPattern |  |  |  |
| pattern0 SEQUENCE { |  |  |  |
| subcarrierLocation-p0 | s4 |  |  |
| symbolLocation-p0 | 9 |  |  |
| } |  |  |  |

Table 6.3.2.2.1.4.3_1-5: CodebookConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.2, Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrOfAntennaPorts CHOICE { |  |  |  |
| moreThanTwo SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| two-one-TypeI-SinglePanel-Restriction | 11111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | 00000001 |  |  |

Table 6.3.2.2.1.4.3_1-6: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList | 0 |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands7 | 1111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| subbandSize | value2 |  |  |
| } |  |  |  |

<< Clauses skipped here>>

6.3.2.2.2.4.3_1	Message exceptions for SA

Table 6.3.2.2.2.4.3_1-1: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| resourceType | aperiodic |  |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3_1-2: CSI-RS-ResourceMapping for NZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 5.4.2, Table5.4.2.0-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 001100 |  |  |
| } |  |  |  |
| nrofPorts | p8 |  |  |
| firstOFDMSymbolInTimeDomain | 5 |  |  |
| cdm-Type | cdm4-FD2-TD2 |  |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3_1-3: CSI-RS-ResourceMapping for ZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 5.4.2, Table5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 |  |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 |  |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3_1-4: CSI-IM-Resource

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| csi-IM-ResourceElementPattern |  |  |  |
| pattern0 SEQUENCE { |  |  |  |
| subcarrierLocation-p0 | s4 |  |  |
| symbolLocation-p0 | 9 |  |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3_1-5: CodebookConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.2, Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrOfAntennaPorts CHOICE { |  |  |  |
| moreThanTwo SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| four-one-TypeI-SinglePanel-Restriction | FFFF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | 00000010 |  |  |

Table 6.3.2.2.2.4.3_1-6: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList | 8 |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands7 | 1111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<< Clauses skipped here>>

6.3.3.1.2.4.3.1	Message exceptions for SA

Table 6.3.3.1.2.4.3.1-1: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| resourceType | aperiodic |  |  |
| } |  |  |  |

Table 6.3.3.1.2.4.3.1-2: CSI-RS-ResourceMapping for NZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 5.4.2, Table5.4.2.0-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 001100 |  |  |
| } |  |  |  |
| nrofPorts | p8 |  |  |
| firstOFDMSymbolInTimeDomain | 5 |  |  |
| cdm-Type | cdm4-FD2-TD2 |  |  |
| } |  |  |  |

Table 6.3.3.1.2.4.3.1-3: CSI-RS-ResourceMapping for ZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 5.4.2, Table5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 |  |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 |  |  |
| } |  |  |  |

Table 6.3.3.1.2.4.3.1-4: CSI-IM-Resource

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| csi-IM-ResourceElementPattern |  |  |  |
| pattern0 SEQUENCE { |  |  |  |
| subcarrierLocation-p0 | s4 |  |  |
| symbolLocation-p0 | 9 |  |  |
| } |  |  |  |

Table 6.3.3.1.2.4.3.1-5: CodebookConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrOfAntennaPorts CHOICE { |  |  |  |
| moreThanTwo SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| four-one-TypeI-SinglePanel-Restriction | FFFF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | 00000010 |  |  |

Table 6.3.3.1.2.4.3.1-6: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList | 5 |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands7 | 1111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<< Clauses skipped here>>

6.3.3.2.1.4.3.1	Message exceptions for SA

Table 6.3.3.2.1.4.3.1-1: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| resourceType | Aperiodic |  |  |
| } |  |  |  |

Table 6.3.3.2.1.4.3.1-2: CSI-RS-ResourceMapping for NZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause5.4.2, Table5.4.2.0-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| Row4 | 001 |  |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 13 |  |  |
| } |  |  |  |

Table 6.3.3.2.1.4.3.1-3: CSI-RS-ResourceMapping for ZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 5.4.2, Table5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| Row5 | 000100 |  |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 |  |  |
| } |  |  |  |

Table 6.3.3.2.1.4.3.1-4: CSI-IM-Resource

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| csi-IM-ResourceElementPattern |  |  |  |
| pattern0 SEQUENCE { |  |  |  |
| subcarrierLocation-p0 | s4 |  |  |
| symbolLocation-p0 | 9 |  |  |
| } |  |  |  |

Table 6.3.3.2.1.4.3.1-5: CodebookConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrOfAntennaPorts CHOICE { |  |  |  |
| moreThanTwo SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| two-one-TypeI-SinglePanel-Restriction | 11111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | 00000001 |  |  |

Table 6.3.3.2.1.4.3.1-6: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList | 0 |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands7 | [1111111] |  |  |
| } |  |  |  |
| } |  |  |  |
| subbandSize | value2 |  |  |
| } |  |  |  |

<< Clauses skipped here>>

6.3.3.2.2.4.3.1	Message contents for SA

Table 6.3.3.2.2.4.3.1-1: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| resourceType | Aperiodic |  |  |
| } |  |  |  |

Table 6.3.3.2.2.4.3.1-2: CSI-RS-ResourceMapping for NZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause5.4.2, Table5.4.2.0-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 001100 |  |  |
| } |  |  |  |
| nrofPorts | p8 |  |  |
| firstOFDMSymbolInTimeDomain | 5 |  |  |
| cdm-Type | cdm4-FD2-TD2 |  |  |
| } |  |  |  |

Table 6.3.3.2.2.4.3.1-3: CSI-RS-ResourceMapping for ZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 5.4.2, Table5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 |  |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 |  |  |
| } |  |  |  |

Table 6.3.3.2.2.4.3.1-4: CSI-IM-Resource

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| csi-IM-ResourceElementPattern |  |  |  |
| pattern0 SEQUENCE { |  |  |  |
| subcarrierLocation-p0 | s4 |  |  |
| symbolLocation-p0 | 9 |  |  |
| } |  |  |  |

Table 6.3.3.2.2.4.3.1-5: CodebookConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrOfAntennaPorts CHOICE { |  |  |  |
| moreThanTwo SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| four-one-TypeI-SinglePanel-Restriction | FFFF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | 00000010 |  |  |

Table 6.3.3.2.2.4.3.1-6: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList | 8 |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands7 | 1111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<< End of changes>>