3GPP TSG-RAN5 Meeting #91-e	R5-212674

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5307 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of band 7, 42, 43, 65 to spurious emission band UE co-existence for cat NB1 and NB2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_bands_R16_M1_NB1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE category NB1 and NB2 Rel-16 spurious emission band and UE coexcistence test requirements are not defined in TS 36.521-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update of table for NB1 and NB2 Rel-16 test requirements. - Editorial; table references updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3F.2.3, 6.6.3F.2.4.1, 6.6.3F.2.4.2, 6.6.3F.2.4.3, 6.6.3F.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## [Start of changes]

6.6.3F.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2F. All of these configurations shall be tested with applicable test parameters, and are shown in Table 6.6.3F.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.6.3F.2.4.1-1: Test Configuration Table for FDD & TDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 8.1.1 |  | NC |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.3 (Note 2) |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Spurious Emissions testing | Modulation | Ntones | Subcarrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75 |
| 2 |  | QPSK | 1@47 | 3.75 |
| 3 |  | BPSK | 1@0 | 15 |
| 4 |  | BPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 12@0 | 15 |
| 6 (Note 3) |  | QPSK | 1@5 | 3.75 |
| 7 (Note 3) |  | QPSK | 1@3 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions Note 2: Do not apply for requirements under Note 13 and Note 34 in Tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1C. Test frequencies for these requirements are defined in Table 6.6.3F.2.4.1-2 and 6.6.3F.2.4.1-3. Note 3: Only apply for requirements under Note 39 in Table 6.6.3F.2.3-1 to Table 6.6.3F.2.3-1C, Note 42 in Table 6.6.3F.2.3-1A to Table 6.6.3F.2.3-1C and Note 44 in Table 6.6.3F.2.3-1B to Table 6.6.3F.2.3-1C, replacing Configuration ID 1 and 3. |  |  |  |  |

Table 6.6.3F.2.4.1-2: Test frequencies for E-UTRA channel bandwidth for operating band 3 
with Note 13 (in Tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1C)

| Bandwidth [kHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| 200 | 19948 | 1784.8 | 1948 | 1879.8 |

Table 6.6.3F.2.4.1-3: Test frequencies for E-UTRA channel bandwidth for operating band 28 
with Note 34 (in Tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1C)

| Bandwidth [kHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| 200 | 27361 | 718.1 | 9361 | 773.1 |

1.	Connect the SS to the UE to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.7 using only main Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.3F.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 6.6.3F.2.4.3.

6.6.3F.2.4.2	Test procedure

1.	SS sends uplink scheduling information via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.6.3F.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC (UE should be already transmitting PUMAX after Initial Conditions setting).

2.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1C. The centre frequency of the filter shall be stepped in contiguous steps according to table 6.6.3F.2.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

NOTE 2:	For requirements under Note 39 in Tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1C, Note 42 in Tables 6.6.3F.2.3-1A to 6.6.3F.2.3-1C and Note 44 in Tables 6.6.3F.2.3-1B to 6.6.3F.2.3-1C, the resolution bandwidth is <=1 kHz to improve the measurement accuracy.

6.6.3F.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6.

##### 6.6.3F.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements and are not repeated in this section.

The measured average power of spurious emission, derived in step 2, shall not exceed the described value in tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1C according to the following rule:

The requirements for the UE are release specific and can be found in Tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1C. If the UE support a band, which is not defined in the table corresponding UE’s release, the requirements for this band are taken from the table of earliest release where requirements for this band are defined. This has been described in following Table 6.6.3F.2.5-1.

Table 6.6.3F.2.5-1: UE Requirements according to UE NB-IoT release and supported NB-IoT band

| Band | UE Requirements per release |  |  |  |
| --- | --- | --- | --- | --- |
|  | Rel-13 | Rel-14 | Rel-15 | Rel-16 |
| 1 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 2 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 3 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 4 | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 5 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 7 | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C |
| 8 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 11 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 12 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 13 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 14 | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 17 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 18 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 19 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 20 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 21 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 25 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 26 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 28 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 31 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 42 | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C |
| 43 | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C |
| 65 | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C |
| 66 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 70 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |
| 71 | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C |

## [End of changes]