3GPP TSG-RAN5 Meeting #91-e	R5-212691

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1868 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of nominal channel spacing in test frequencies for CA_n261x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Nominal channel spacing used in test frequency tables for some of the n261 NR intra-band contiguous configurations and CC CBW combinations are not in accordance to TS 38.101-2, clause 5.4A.1. Table 4.3.1.2.3.5.13-2: Incorrect SCS in table header. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 4.3.1.2.3.5.3-1: Corrected nominal channel spacing for CA_n261D and CCBW combination 50+200. Table 4.3.1.2.3.5.6-1: Corrected nominal channel spacing for CA_n261G and CCBW combination 50+100. Table 4.3.1.2.3.5.7-1: Corrected nominal channel spacing for CA_n261H and CCBW combination 50+100+100. Table 4.3.1.2.3.5.8-1: Corrected nominal channel spacing for CA_n261I and CCBW combination 50+100+100+100. Table 4.3.1.2.3.5.9-1: Corrected nominal channel spacing for CA_n261J and CCBW combination 50+100+100+100+100. Table 4.3.1.2.3.5.13-1: Corrected nominal channel spacing for CA_n261O and CCBW combination 100+50. Table 4.3.1.2.3.5.13-2: Corrected SCS from 60 to 120kHz. Table 4.3.1.2.3.5.14-1: Corrected nominal channel spacing for CA_n261O and CCBW combinations 50+100+100, 50+100+50, 50+100+100, 100+50+100, 100+50+100 and 100+100+50. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test frequencies for CA_n261D, CA_n261G, CA_n261H, CA_n261I, CA_n261J, CA_n261O and CA_n261P remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.2.3.5.3, 4.3.1.2.3.5.6, 4.3.1.2.3.5.7, 4.3.1.2.3.5.8, 4.3.1.2.3.5.9, 4.3.1.2.3.5.13 and 4.3.1.2.3.5.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.5.3	CA_n261D

Table 4.3.1.2.3.5.3-1: NR Intra-Band contiguous CA configuration CA_n261D (PCC=CC1 and SCC=CC2), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+200 | CC1 | 50 | 60 | 66 | Downlink | Low | 27525 | 2071249 | 27501.24 | 2070853 | 0 | 120 |  | 22446 | 2071387 |  | 6 |  | 16 | 1 (8) | 24 |
|  |  |  |  |  | & | Mid | 27825 | 2076249 | 27727.8 | 2074629 | 102 |  |  | 22463 | 2076283 |  | 10 |  | 7 | 1 (8) | 117 |
|  |  |  |  |  | Uplink | High | 28128 | 2081299 | 27741.36 | 2074855 | 504 |  |  | 22480 | 2081179 |  | 0 |  | 3 | 0 (0) | 507 |
|  | Channel spacing CC1-CC2=121.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 60 | 264 | Downlink | Low | 27646.92 | 2073281 | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 2 |  | 2 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27946.92 | 2078281 | 27778.44 | 2075473 | 102 |  |  | 22466 | 2077147 |  | 6 |  | 9 | 1 (8) | 119 |
|  |  |  |  |  | Uplink | High | 28249.92 | 2083331 | 27792 | 2075699 | 504 |  |  | 22483 | 2082043 |  | 8 |  | 4 | 0 (0) | 508 |
| 100+200 | CC1 | 100 | 60 | 132 | Downlink | Low | 27550.08 | 2071667 | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 8 |  | 14 | 1 (8) | 22 |
|  |  |  |  |  | & | Mid | 27825 | 2076249 | 27704.04 | 2074233 | 102 |  |  | 22461 | 2075707 |  | 10 |  | 0 | 0 (0) | 102 |
|  |  |  |  |  | Uplink | High | 28102.44 | 2080873 | 27692.04 | 2074033 | 504 |  |  | 22478 | 2080603 |  | 6 |  | 15 | 1 (8) | 527 |
|  | Channel spacing CC1-CC2=147.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 60 | 264 | Downlink | Low | 27697.56 | 2074125 | 27602.52 | 2072541 | 0 | 120 |  | 22451 | 2072827 |  | 10 |  | 3 | 0 (0) | 3 |
|  |  |  |  |  | & | Mid | 27972.48 | 2078707 | 27804 | 2075899 | 102 |  |  | 22467 | 2077435 |  | 0 |  | 6 | 0 (0) | 108 |
|  |  |  |  |  | Uplink | High | 28249.92 | 2083331 | 27792 | 2075699 | 504 |  |  | 22483 | 2082043 |  | 8 |  | 4 | 0 (0) | 508 |
| 200+200 | CC1 | 200 | 60 | 264 | Downlink | Low | 27600 | 2072499 | 27504.96 | 2070915 | 0 | 120 |  | 22446 | 2071387 |  | 4 |  | 11 | 1 (8) | 19 |
|  |  |  |  |  | & | Mid | 27825 | 2076249 | 27656.52 | 2073441 | 102 |  |  | 22459 | 2075131 |  | 10 |  | 10 | 1 (8) | 120 |
|  |  |  |  |  | Uplink | High | 28050 | 2079999 | 27592.08 | 2072367 | 504 |  |  | 22472 | 2078875 |  | 4 |  | 10 | 1 (8) | 522 |
|  | Channel spacing CC1-CC2=199.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 60 | 264 | Downlink | Low | 27799.92 | 2075831 | 27704.88 | 2074247 | 0 | 120 |  | 22457 | 2074555 |  | 8 |  | 5 | 0 (0) | 5 |
|  |  |  |  |  | & | Mid | 28024.92 | 2079581 | 27856.44 | 2076773 | 102 |  |  | 22470 | 2078299 |  | 2 |  | 5 | 0 (0) | 107 |
|  |  |  |  |  | Uplink | High | 28249.92 | 2083331 | 27792 | 2075699 | 504 |  |  | 22483 | 2082043 |  | 8 |  | 4 | 0 (0) | 508 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.5.3-2: NR Intra-Band contiguous CA configuration CA_n261D (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+200 | CC1 | 50 | 120 | 32 | Downlink | Low | 27525 | 2071249 | 27501.96 | 2070865 | 0 | 120 |  | 22446 | 2071387 |  | 9 |  | 7 | 1 (4) | 22 |
|  |  |  |  |  | & | Mid | 27825 | 2076249 | 27655.08 | 2073417 | 102 |  |  | 22463 | 2076283 |  | 5 |  | 3 | 1 (4) | 218 |
|  |  |  |  |  | Uplink | High | 28128 | 2081299 | 27379.2 | 2068819 | 504 |  |  | 22480 | 2081179 |  | 0 |  | 1 | 0 (0) | 1010 |
|  | Channel spacing CC1-CC2=121.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 120 | 132 | Downlink | Low | 27646.92 | 2073281 | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 1 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27946.92 | 2078281 | 27705 | 2074249 | 102 |  |  | 22466 | 2077147 |  | 9 |  | 4 | 1 (4) | 220 |
|  |  |  |  |  | Uplink | High | 28249.92 | 2083331 | 27429.12 | 2069651 | 504 |  |  | 22483 | 2082043 |  | 4 |  | 2 | 0 (0) | 1012 |
| 100+200 | CC1 | 100 | 120 | 66 | Downlink | Low | 27550.08 | 2071667 | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 4 |  | 7 | 1 (4) | 22 |
|  |  |  |  |  | & | Mid | 27825 | 2076249 | 27630.6 | 2073009 | 102 |  |  | 22461 | 2075707 |  | 5 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 28102.44 | 2080873 | 27329.16 | 2067985 | 504 |  |  | 22478 | 2080603 |  | 9 |  | 7 | 1 (4) | 1030 |
|  | Channel spacing CC1-CC2=147.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 120 | 132 | Downlink | Low | 27697.56 | 2074125 | 27602.52 | 2072541 | 0 | 120 |  | 22451 | 2072827 |  | 11 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27972.48 | 2078707 | 27730.56 | 2074675 | 102 |  |  | 22467 | 2077435 |  | 0 |  | 3 | 0 (0) | 210 |
|  |  |  |  |  | Uplink | High | 28249.92 | 2083331 | 27429.12 | 2069651 | 504 |  |  | 22483 | 2082043 |  | 4 |  | 2 | 0 (0) | 1012 |
| 200+200 | CC1 | 200 | 120 | 132 | Downlink | Low | 27600 | 2072499 | 27504.96 | 2070915 | 0 | 120 |  | 22446 | 2071387 |  | 8 |  | 5 | 1 (4) | 18 |
|  |  |  |  |  | & | Mid | 27825 | 2076249 | 27583.08 | 2072217 | 102 |  |  | 22459 | 2075131 |  | 5 |  | 5 | 1 (4) | 222 |
|  |  |  |  |  | Uplink | High | 28050 | 2079999 | 27229.2 | 2066319 | 504 |  |  | 22472 | 2078875 |  | 2 |  | 5 | 1 (4) | 1026 |
|  | Channel spacing CC1-CC2=199.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 120 | 132 | Downlink | Low | 27799.92 | 2075831 | 27704.88 | 2074247 | 0 | 120 |  | 22457 | 2074555 |  | 10 |  | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 28024.92 | 2079581 | 27783 | 2075549 | 102 |  |  | 22470 | 2078299 |  | 7 |  | 2 | 0 (0) | 208 |
|  |  |  |  |  | Uplink | High | 28249.92 | 2083331 | 27429.12 | 2069651 | 504 |  |  | 22483 | 2082043 |  | 4 |  | 2 | 0 (0) | 1012 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.5.4	CA_n261E

Table 4.3.1.2.3.5.4-1: NR Intra-Band contiguous CA configuration CA_n261E (PCC=CC1 and SCC=CC2), SCS=60 kHz, nominal channel spacing.

FFS

Table 4.3.1.2.3.5.4-2: NR Intra-Band contiguous CA configuration CA_n261E (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing.

FFS

4.3.1.2.3.5.5	CA_n261F

Table 4.3.1.2.3.5.5-1: NR Intra-Band contiguous CA configuration CA_n261F (PCC=CC1 and SCC=CC2), SCS=60 kHz, nominal channel spacing.

FFS

Table 4.3.1.2.3.5.5-2: NR Intra-Band contiguous CA configuration CA_n261F (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing.

FFS

4.3.1.2.3.5.6	CA_n261G

Table 4.3.1.2.3.5.6-1: NR Intra-Band contiguous CA configuration CA_n261G (PCC=CC1 and SCC=CC2), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 60 | 66 | Downlink | Low | 27525 | 2071249 | 27501.24 | 2070853 | 0 | 120 |  | 22446 | 2071387 |  | 6 |  | 16 | 1 (8) | 24 |
|  |  |  |  |  | & | Mid | 27874.92 | 2077081 | 27777.72 | 2075461 | 102 |  |  | 22466 | 2077147 |  | 6 |  | 10 | 1 (8) | 120 |
|  |  |  |  |  | Uplink | High | 28225.56 | 2082925 | 27838.92 | 2076481 | 504 |  |  | 22486 | 2082907 |  | 6 |  | 3 | 1 (8) | 515 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 27599.4 | 2072489 | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 2 |  | 2 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27949.32 | 2078321 | 27828.36 | 2076305 | 102 |  |  | 22469 | 2078011 |  | 2 |  | 12 | 1 (8) | 122 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27889.56 | 2077325 | 504 |  |  | 22489 | 2083771 |  | 2 |  | 5 | 1 (8) | 517 |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink | Low | 27550.08 | 2071667 | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 8 |  | 14 | 1 (8) | 22 |
|  |  |  |  |  | & | Mid | 27874.92 | 2077081 | 27753.96 | 2075065 | 102 |  |  | 22464 | 2076571 |  | 6 |  | 3 | 0 (0) | 105 |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27789.6 | 2075659 | 504 |  |  | 22483 | 2082043 |  | 0 |  | 0 | 1 (8) | 512 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 27650.04 | 2073333 | 27602.52 | 2072541 | 0 | 120 |  | 22451 | 2072827 |  | 10 |  | 3 | 0 (0) | 3 |
|  |  |  |  |  | & | Mid | 27974.88 | 2078747 | 27853.92 | 2076731 | 102 |  |  | 22470 | 2078299 |  | 8 |  | 0 | 1 (8) | 110 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27889.56 | 2077325 | 504 |  |  | 22489 | 2083771 |  | 2 |  | 5 | 1 (8) | 517 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.5.6-2: NR Intra-Band contiguous CA configuration CA_n261G (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 120 | 32 | Downlink | Low | 27525 | 2071249 | 27501.96 | 2070865 | 0 | 120 |  | 22446 | 2071387 |  | 9 |  | 7 | 1 (4) | 22 |
|  |  |  |  |  | & | Mid | 27875.04 | 2077083 | 27705.12 | 2074251 | 102 |  |  | 22466 | 2077147 |  | 8 |  | 4 | 1 (4) | 220 |
|  |  |  |  |  | Uplink | High | 28225.56 | 2082925 | 27476.76 | 2070445 | 504 |  |  | 22486 | 2082907 |  | 3 |  | 1 | 1 (4) | 1018 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 27599.4 | 2072489 | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 1 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27949.44 | 2078323 | 27755.04 | 2075083 | 102 |  |  | 22469 | 2078011 |  | 0 |  | 6 | 1 (4) | 224 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27526.68 | 2071277 | 504 |  |  | 22489 | 2083771 |  | 7 |  | 2 | 1 (4) | 1020 |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink | Low | 27550.08 | 2071667 | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 4 |  | 7 | 1 (4) | 22 |
|  |  |  |  |  | & | Mid | 27875.04 | 2077083 | 27680.64 | 2073843 | 102 |  |  | 22464 | 2076571 |  | 8 |  | 1 | 0 (0) | 206 |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27426.72 | 2069611 | 504 |  |  | 22483 | 2082043 |  | 0 |  | 0 | 1 (4) | 1016 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 27650.04 | 2073333 | 27602.52 | 2072541 | 0 | 120 |  | 22451 | 2072827 |  | 11 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27975 | 2078749 | 27780.6 | 2075509 | 102 |  |  | 22470 | 2078299 |  | 3 |  | 0 | 1 (4) | 212 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27526.68 | 2071277 | 504 |  |  | 22489 | 2083771 |  | 7 |  | 2 | 1 (4) | 1020 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.5.7	CA_n261H

Table 4.3.1.2.3.5.7-1: NR Intra-Band contiguous CA configuration CA_n261H (PCC=CC1 and SCC=CC2, CC3), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 60 | 66 | Downlink | Low | 27525 | 2071249 | 27501.24 | 2070853 | 0 | 120 |  | 22446 | 2071387 |  | 6 |  | 16 | 1 (8) | 24 |
| +100 |  |  |  |  | & | Mid | 27825 | 2076249 | 27727.8 | 2074629 | 102 |  |  | 22463 | 2076283 |  | 10 |  | 7 | 1 (8) | 117 |
|  |  |  |  |  | Uplink | High | 28125.6 | 2081259 | 27738.96 | 2074815 | 504 |  |  | 22480 | 2081179 |  | 4 |  | 6 | 0 (0) | 510 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 27599.4 | 2072489 | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 2 |  | 2 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27899.4 | 2077489 | 27778.44 | 2075473 | 102 |  |  | 22466 | 2077147 |  | 6 |  | 9 | 1 (8) | 119 |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27789.6 | 2075659 | 504 |  |  | 22483 | 2082043 |  | 0 |  | 0 | 1 (8) | 512 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 27699.36 | 2074155 | 27651.84 | 2073363 | 0 | 120 |  | 22454 | 2073691 |  | 4 |  | 7 | 0 (0) | 7 |
|  |  |  |  |  | & | Mid | 27999.36 | 2079155 | 27878.4 | 2077139 | 102 |  |  | 22472 | 2078875 |  | 8 |  | 14 | 1 (8) | 124 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27889.56 | 2077325 | 504 |  |  | 22489 | 2083771 |  | 2 |  | 5 | 1 (8) | 517 |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink | Low | 27550.08 | 2071667 | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 8 |  | 14 | 1 (8) | 22 |
| +100 |  |  |  |  | & | Mid | 27825 | 2076249 | 27704.04 | 2074233 | 102 |  |  | 22461 | 2075707 |  | 10 |  | 0 | 0 (0) | 102 |
|  |  |  |  |  | Uplink | High | 28100.04 | 2080833 | 27689.64 | 2073993 | 504 |  |  | 22477 | 2080315 |  | 10 |  | 2 | 0 (0) | 506 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 27650.04 | 2073333 | 27602.52 | 2072541 | 0 | 120 |  | 22451 | 2072827 |  | 10 |  | 3 | 0 (0) | 3 |
|  |  |  |  |  | & | Mid | 27924.96 | 2077915 | 27804 | 2075899 | 102 |  |  | 22467 | 2077435 |  | 0 |  | 6 | 0 (0) | 108 |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27789.6 | 2075659 | 504 |  |  | 22483 | 2082043 |  | 0 |  | 0 | 1 (8) | 512 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 27750 | 2074999 | 27702.48 | 2074207 | 0 | 120 |  | 22457 | 2074555 |  | 0 |  | 1 | 1 (8) | 9 |
|  |  |  |  |  | & | Mid | 28024.92 | 2079581 | 27903.96 | 2077565 | 102 |  |  | 22473 | 2079163 |  | 2 |  | 3 | 1 (8) | 113 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27889.56 | 2077325 | 504 |  |  | 22489 | 2083771 |  | 2 |  | 5 | 1 (8) | 517 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.5.7-2: NR Intra-Band contiguous CA configuration CA_n261H (PCC=CC1 and SCC=CC2, CC3), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 120 | 32 | Downlink | Low | 27525 | 2071249 | 27501.96 | 2070865 | 0 | 120 |  | 22446 | 2071387 |  | 9 | 7 | 1 (4) | 22 |
| +100 |  |  |  |  | & | Mid | 27825 | 2076249 | 27655.08 | 2073417 | 102 |  |  | 22463 | 2076283 |  | 5 | 3 | 1 (4) | 218 |
|  |  |  |  |  | Uplink | High | 28125.6 | 2081259 | 27376.8 | 2068779 | 504 |  |  | 22480 | 2081179 |  | 8 | 2 | 0 (0) | 1012 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 27599.4 | 2072489 | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 1 | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27899.4 | 2077489 | 27705 | 2074249 | 102 |  |  | 22466 | 2077147 |  | 9 | 4 | 1 (4) | 220 |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27426.72 | 2069611 | 504 |  |  | 22483 | 2082043 |  | 0 | 0 | 1 (4) | 1016 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 27699.36 | 2074155 | 27651.84 | 2073363 | 0 | 120 |  | 22454 | 2073691 |  | 8 | 3 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27999.36 | 2079155 | 27804.96 | 2075915 | 102 |  |  | 22472 | 2078875 |  | 4 | 7 | 1 (4) | 226 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27526.68 | 2071277 | 504 |  |  | 22489 | 2083771 |  | 7 | 2 | 1 (4) | 1020 |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink | Low | 27550.08 | 2071667 | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 4 | 7 | 1 (4) | 22 |
| +100 |  |  |  |  | & | Mid | 27825 | 2076249 | 27630.6 | 2073009 | 102 |  |  | 22461 | 2075707 |  | 5 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 28100.04 | 2080833 | 27326.76 | 2067945 | 504 |  |  | 22477 | 2080315 |  | 5 | 1 | 0 (0) | 1010 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 27650.04 | 2073333 | 27602.52 | 2072541 | 0 | 120 |  | 22451 | 2072827 |  | 11 | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27924.96 | 2077915 | 27730.56 | 2074675 | 102 |  |  | 22467 | 2077435 |  | 0 | 3 | 0 (0) | 210 |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27426.72 | 2069611 | 504 |  |  | 22483 | 2082043 |  | 0 | 0 | 1 (4) | 1016 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 27750 | 2074999 | 27702.48 | 2074207 | 0 | 120 |  | 22457 | 2074555 |  | 6 | 0 | 1 (4) | 8 |
|  |  |  |  |  | & | Mid | 28024.92 | 2079581 | 27830.52 | 2076341 | 102 |  |  | 22473 | 2079163 |  | 7 | 1 | 1 (4) | 214 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27526.68 | 2071277 | 504 |  |  | 22489 | 2083771 |  | 7 | 2 | 1 (4) | 1020 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.5.8	CA_n261I

Table 4.3.1.2.3.5.8-1: NR Intra-Band contiguous CA configuration CA_n261I (PCC=CC1, SCC=CC2, CC3, CC4), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 60 | 66 | Downlink | Low | 27525 | 2071249 | 27501.24 | 2070853 | 0 | 120 |  | 22446 | 2071387 |  | 6 |  | 16 | 1 (8) | 24 |
| +100+100 |  |  |  |  | & | Mid | 27774.96 | 2075415 | 27677.76 | 2073795 | 102 |  |  | 22460 | 2075419 |  | 4 |  | 5 | 1 (8) | 115 |
|  |  |  |  |  | Uplink | High | 28025.64 | 2079593 | 27639 | 2073149 | 504 |  |  | 22474 | 2079451 |  | 2 |  | 1 | 0 (0) | 505 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 27599.4 | 2072489 | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 2 |  | 2 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27849.36 | 2076655 | 27728.4 | 2074639 | 102 |  |  | 22463 | 2076283 |  | 0 |  | 7 | 1 (8) | 117 |
|  |  |  |  |  | Uplink | High | 28100.04 | 2080833 | 27689.64 | 2073993 | 504 |  |  | 22477 | 2080315 |  | 10 |  | 2 | 0 (0) | 506 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 27699.36 | 2074155 | 27651.84 | 2073363 | 0 | 120 |  | 22454 | 2073691 |  | 4 |  | 7 | 0 (0) | 7 |
|  |  |  |  |  | & | Mid | 27949.32 | 2078321 | 27828.36 | 2076305 | 102 |  |  | 22469 | 2078011 |  | 2 |  | 12 | 1 (8) | 122 |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27789.6 | 2075659 | 504 |  |  | 22483 | 2082043 |  | 0 |  | 0 | 1 (8) | 512 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 60 | 132 | Downlink | Low | 27799.32 | 2075821 | 27751.8 | 2075029 | 0 | 120 |  | 22460 | 2075419 |  | 6 |  | 4 | 1 (8) | 12 |
|  |  |  |  |  | & | Mid | 28049.28 | 2079987 | 27928.32 | 2077971 | 102 |  |  | 22474 | 2079451 |  | 4 |  | 1 | 0 (0) | 103 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27889.56 | 2077325 | 504 |  |  | 22489 | 2083771 |  | 2 |  | 5 | 1 (8) | 517 |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink | Low | 27550.08 | 2071667 | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 8 |  | 14 | 1 (8) | 22 |
| +100+100 |  |  |  |  | & | Mid | 27774.96 | 2075415 | 27654 | 2073399 | 102 |  |  | 22459 | 2075131 |  | 4 |  | 14 | 1 (8) | 124 |
|  |  |  |  |  | Uplink | High | 28000.08 | 2079167 | 27589.68 | 2072327 | 504 |  |  | 22472 | 2078875 |  | 8 |  | 13 | 1 (8) | 525 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 27650.04 | 2073333 | 27602.52 | 2072541 | 0 | 120 |  | 22451 | 2072827 |  | 10 |  | 3 | 0 (0) | 3 |
|  |  |  |  |  | & | Mid | 27874.92 | 2077081 | 27753.96 | 2075065 | 102 |  |  | 22464 | 2076571 |  | 6 |  | 3 | 0 (0) | 105 |
|  |  |  |  |  | Uplink | High | 28100.04 | 2080833 | 27689.64 | 2073993 | 504 |  |  | 22477 | 2080315 |  | 10 |  | 2 | 0 (0) | 506 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 27750 | 2074999 | 27702.48 | 2074207 | 0 | 120 |  | 22457 | 2074555 |  | 0 |  | 1 | 1 (8) | 9 |
|  |  |  |  |  | & | Mid | 27974.88 | 2078747 | 27853.92 | 2076731 | 102 |  |  | 22470 | 2078299 |  | 8 |  | 0 | 1 (8) | 110 |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27789.6 | 2075659 | 504 |  |  | 22483 | 2082043 |  | 0 |  | 0 | 1 (8) | 512 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 60 | 132 | Downlink | Low | 27849.96 | 2076665 | 27802.44 | 2075873 | 0 | 120 |  | 22463 | 2076283 |  | 2 |  | 6 | 1 (8) | 14 |
|  |  |  |  |  | & | Mid | 28074.84 | 2080413 | 27953.88 | 2078397 | 102 |  |  | 22476 | 2080027 |  | 10 |  | 5 | 1 (8) | 115 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27889.56 | 2077325 | 504 |  |  | 22489 | 2083771 |  | 2 |  | 5 | 1 (8) | 517 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.5.8-2: NR Intra-Band contiguous CA configuration CA_n261I (PCC=CC1, SCC=CC2, CC3,CC4), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 120 | 32 | Downlink | Low | 27525 | 2071249 | 27501.96 | 2070865 | 0 | 120 |  | 22446 | 2071387 |  | 9 |  | 7 | 1 (4) | 22 |
| +100+100 |  |  |  |  | & | Mid | 27774.96 | 2075415 | 27605.04 | 2072583 | 102 |  |  | 22460 | 2075419 |  | 2 |  | 2 | 1 (4) | 216 |
|  |  |  |  |  | Uplink | High | 28025.64 | 2079593 | 27276.84 | 2067113 | 504 |  |  | 22474 | 2079451 |  | 1 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 27599.4 | 2072489 | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 1 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27849.36 | 2076655 | 27654.96 | 2073415 | 102 |  |  | 22463 | 2076283 |  | 6 |  | 3 | 1 (4) | 218 |
|  |  |  |  |  | Uplink | High | 28100.04 | 2080833 | 27326.76 | 2067945 | 504 |  |  | 22477 | 2080315 |  | 5 |  | 1 | 0 (0) | 1010 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 27699.36 | 2074155 | 27651.84 | 2073363 | 0 | 120 |  | 22454 | 2073691 |  | 8 |  | 3 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27949.32 | 2078321 | 27754.92 | 2075081 | 102 |  |  | 22469 | 2078011 |  | 1 |  | 6 | 1 (4) | 224 |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27426.72 | 2069611 | 504 |  |  | 22483 | 2082043 |  | 0 |  | 0 | 1 (4) | 1016 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 120 | 66 | Downlink | Low | 27799.32 | 2075821 | 27751.8 | 2075029 | 0 | 120 |  | 22460 | 2075419 |  | 3 |  | 2 | 1 (4) | 12 |
|  |  |  |  |  | & | Mid | 28049.28 | 2079987 | 27854.88 | 2076747 | 102 |  |  | 22474 | 2079451 |  | 8 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27526.68 | 2071277 | 504 |  |  | 22489 | 2083771 |  | 7 |  | 2 | 1 (4) | 1020 |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink | Low | 27550.08 | 2071667 | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 4 |  | 7 | 1 (4) | 22 |
| +100+100 |  |  |  |  | & | Mid | 27774.96 | 2075415 | 27580.56 | 2072175 | 102 |  |  | 22459 | 2075131 |  | 2 |  | 7 | 1 (4) | 226 |
|  |  |  |  |  | Uplink | High | 28000.08 | 2079167 | 27226.8 | 2066279 | 504 |  |  | 22472 | 2078875 |  | 10 |  | 6 | 1 (4) | 1028 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 27650.04 | 2073333 | 27602.52 | 2072541 | 0 | 120 |  | 22451 | 2072827 |  | 11 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27874.92 | 2077081 | 27680.52 | 2073841 | 102 |  |  | 22464 | 2076571 |  | 9 |  | 1 | 0 (0) | 206 |
|  |  |  |  |  | Uplink | High | 28100.04 | 2080833 | 27326.76 | 2067945 | 504 |  |  | 22477 | 2080315 |  | 5 |  | 1 | 0 (0) | 1010 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 27750 | 2074999 | 27702.48 | 2074207 | 0 | 120 |  | 22457 | 2074555 |  | 6 |  | 0 | 1 (4) | 8 |
|  |  |  |  |  | & | Mid | 27974.88 | 2078747 | 27780.48 | 2075507 | 102 |  |  | 22470 | 2078299 |  | 4 |  | 0 | 1 (4) | 212 |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27426.72 | 2069611 | 504 |  |  | 22483 | 2082043 |  | 0 |  | 0 | 1 (4) | 1016 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 120 | 66 | Downlink | Low | 27849.96 | 2076665 | 27802.44 | 2075873 | 0 | 120 |  | 22463 | 2076283 |  | 1 |  | 3 | 1 (4) | 14 |
|  |  |  |  |  | & | Mid | 28074.84 | 2080413 | 27880.44 | 2077173 | 102 |  |  | 22476 | 2080027 |  | 11 |  | 2 | 1 (4) | 216 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27526.68 | 2071277 | 504 |  |  | 22489 | 2083771 |  | 7 |  | 2 | 1 (4) | 1020 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.5.9	CA_n261J

Table 4.3.1.2.3.5.9-1: NR Intra-Band contiguous CA configuration CA_n261J (PCC=CC1, SCC=CC2, CC3, CC4), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 |  | CORESET#0 Index (Offset [RBs]) Note 4 |  | offsetToPointA (SIB1) [PRBs] Note 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 60 | 66 | Downlink | Low | 27525 | 2071249 | 27501.24 | 2070853 | 0 | 120 |  | 22446 | 2071387 | 6 |  | 16 |  | 1 (8) |  | 24 |  |
| +100+100 |  |  |  |  | & | Mid | 27724.92 | 2074581 | 27627.72 | 2072961 | 102 |  |  | 22457 | 2074555 | 10 |  | 2 |  | 1 (8) |  | 112 |  |
| +100 |  |  |  |  | Uplink | High | 27925.68 | 2077927 | 27539.04 | 2071483 | 504 |  |  | 22469 | 2078011 | 0 |  | 12 |  | 1 (8) |  | 524 |  |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 27599.4 | 2072489 | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 | 2 |  | 2 |  | 0 (0) |  | 2 |  |
|  |  |  |  |  | & | Mid | 27799.32 | 2075821 | 27678.36 | 2073805 | 102 |  |  | 22460 | 2075419 | 6 |  | 4 |  | 1 (8) |  | 114 |  |
|  |  |  |  |  | Uplink | High | 28000.08 | 2079167 | 27589.68 | 2072327 | 504 |  |  | 22472 | 2078875 | 8 |  | 13 |  | 1 (8) |  | 525 |  |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 27699.36 | 2074155 | 27651.84 | 2073363 | 0 | 120 |  | 22454 | 2073691 | 4 |  | 7 |  | 0 (0) |  | 7 |  |
|  |  |  |  |  | & | Mid | 27899.28 | 2077487 | 27778.32 | 2075471 | 102 |  |  | 22466 | 2077147 | 8 |  | 9 |  | 1 (8) |  | 119 |  |
|  |  |  |  |  | Uplink | High | 28100.04 | 2080833 | 27689.64 | 2073993 | 504 |  |  | 22477 | 2080315 | 10 |  | 2 |  | 0 (0) |  | 506 |  |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 60 | 132 | Downlink | Low | 27799.32 | 2075821 | 27751.8 | 2075029 | 0 | 120 |  | 22460 | 2075419 | 6 |  | 4 |  | 1 (8) |  | 12 |  |
|  |  |  |  |  | & | Mid | 27999.24 | 2079153 | 27878.28 | 2077137 | 102 |  |  | 22472 | 2078875 | 10 |  | 14 |  | 1 (8) |  | 124 |  |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27789.6 | 2075659 | 504 |  |  | 22483 | 2082043 | 0 |  | 0 |  | 1 (8) |  | 512 |  |
|  | Channel spacing CC4-CC5=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC5 | 100 | 60 | 132 | Downlink | Low | 27899.28 | 2077487 | 27851.76 | 2076695 | 0 | 120 |  | 22466 | 2077147 | 8 |  | 9 |  | 1 (8) |  | 17 |  |
|  |  |  |  |  | & | Mid | 28099.2 | 2080819 | 27978.24 | 2078803 | 102 |  |  | 22477 | 2080315 | 0 |  | 4 |  | 0 (0) |  | 106 |  |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27889.56 | 2077325 | 504 |  |  | 22489 | 2083771 | 2 |  | 5 |  | 1 (8) |  | 517 |  |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink | Low | 27550.08 | 2071667 | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 | 8 |  | 14 |  | 1 (8) |  | 22 |  |
| +100+100 |  |  |  |  | & | Mid | 27724.92 | 2074581 | 27603.96 | 2072565 | 102 |  |  | 22456 | 2074267 | 10 |  | 11 |  | 1 (8) |  | 121 |  |
| +100 |  |  |  |  | Uplink | High | 27900.12 | 2077501 | 27489.72 | 2070661 | 504 |  |  | 22466 | 2077147 | 6 |  | 8 |  | 1 (8) |  | 520 |  |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 27650.04 | 2073333 | 27602.52 | 2072541 | 0 | 120 |  | 22451 | 2072827 | 10 |  | 3 |  | 0 (0) |  | 3 |  |
|  |  |  |  |  | & | Mid | 27824.88 | 2076247 | 27703.92 | 2074231 | 102 |  |  | 22461 | 2075707 | 0 |  | 1 |  | 0 (0) |  | 103 |  |
|  |  |  |  |  | Uplink | High | 28000.08 | 2079167 | 27589.68 | 2072327 | 504 |  |  | 22472 | 2078875 | 8 |  | 13 |  | 1 (8) |  | 525 |  |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 27750 | 2074999 | 27702.48 | 2074207 | 0 | 120 |  | 22457 | 2074555 | 0 |  | 1 |  | 1 (8) |  | 9 |  |
|  |  |  |  |  | & | Mid | 27924.84 | 2077913 | 27803.88 | 2075897 | 102 |  |  | 22467 | 2077435 | 2 |  | 6 |  | 0 (0) |  | 108 |  |
|  |  |  |  |  | Uplink | High | 28100.04 | 2080833 | 27689.64 | 2073993 | 504 |  |  | 22477 | 2080315 | 10 |  | 2 |  | 0 (0) |  | 506 |  |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 60 | 132 | Downlink | Low | 27849.96 | 2076665 | 27802.44 | 2075873 | 0 | 120 |  | 22463 | 2076283 | 2 |  | 6 |  | 1 (8) |  | 14 |  |
|  |  |  |  |  | & | Mid | 28024.8 | 2079579 | 27903.84 | 2077563 | 102 |  |  | 22473 | 2079163 | 4 |  | 3 |  | 1 (8) |  | 113 |  |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27789.6 | 2075659 | 504 |  |  | 22483 | 2082043 | 0 |  | 0 |  | 1 (8) |  | 512 |  |
|  | Channel spacing CC4-CC5=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC5 | 100 | 60 | 132 | Downlink | Low | 27949.92 | 2078331 | 27902.4 | 2077539 | 0 | 120 |  | 22469 | 2078011 | 4 |  | 11 |  | 1 (8) |  | 19 |  |
|  |  |  |  |  | & | Mid | 28124.76 | 2081245 | 28003.8 | 2079229 | 102 |  |  | 22479 | 2080891 | 6 |  | 8 |  | 1 (8) |  | 118 |  |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27889.56 | 2077325 | 504 |  |  | 22489 | 2083771 | 2 |  | 5 |  | 1 (8) |  | 517 |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.5.9-2: NR Intra-Band contiguous CA configuration CA_n261J (PCC=CC1, SCC=CC2, CC3, CC4), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 |  | CORESET#0 Index (Offset [RBs]) Note 4 |  | offsetToPointA (SIB1) [PRBs] Note 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 120 | 32 | Downlink | Low | 27525 | 2071249 | 27501.96 | 2070865 | 0 | 120 |  | 22446 | 2071387 | 9 |  | 7 |  | 1 (4) |  | 22 |  |
| +100+100 |  |  |  |  | & | Mid | 27725.04 | 2074583 | 27555.12 | 2071751 | 102 |  |  | 22457 | 2074555 | 10 |  | 0 |  | 1 (4) |  | 212 |  |
| +100 |  |  |  |  | Uplink | High | 27925.68 | 2077927 | 27176.88 | 2065447 | 504 |  |  | 22469 | 2078011 | 6 |  | 5 |  | 1 (4) |  | 1026 |  |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 27599.4 | 2072489 | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 | 1 |  | 1 |  | 0 (0) |  | 2 |  |
|  |  |  |  |  | & | Mid | 27799.44 | 2075823 | 27605.04 | 2072583 | 102 |  |  | 22460 | 2075419 | 2 |  | 2 |  | 1 (4) |  | 216 |  |
|  |  |  |  |  | Uplink | High | 28000.08 | 2079167 | 27226.8 | 2066279 | 504 |  |  | 22472 | 2078875 | 10 |  | 6 |  | 1 (4) |  | 1028 |  |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 27699.36 | 2074155 | 27651.84 | 2073363 | 0 | 120 |  | 22454 | 2073691 | 8 |  | 3 |  | 0 (0) |  | 6 |  |
|  |  |  |  |  | & | Mid | 27899.4 | 2077489 | 27705 | 2074249 | 102 |  |  | 22466 | 2077147 | 9 |  | 4 |  | 1 (4) |  | 220 |  |
|  |  |  |  |  | Uplink | High | 28100.04 | 2080833 | 27326.76 | 2067945 | 504 |  |  | 22477 | 2080315 | 5 |  | 1 |  | 0 (0) |  | 1010 |  |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 120 | 66 | Downlink | Low | 27799.32 | 2075821 | 27751.8 | 2075029 | 0 | 120 |  | 22460 | 2075419 | 3 |  | 2 |  | 1 (4) |  | 12 |  |
|  |  |  |  |  | & | Mid | 27999.36 | 2079155 | 27804.96 | 2075915 | 102 |  |  | 22472 | 2078875 | 4 |  | 7 |  | 1 (4) |  | 226 |  |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27426.72 | 2069611 | 504 |  |  | 22483 | 2082043 | 0 |  | 0 |  | 1 (4) |  | 1016 |  |
|  | Channel spacing CC4-CC5=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC5 | 100 | 120 | 66 | Downlink | Low | 27899.28 | 2077487 | 27851.76 | 2076695 | 0 | 120 |  | 22466 | 2077147 | 10 |  | 4 |  | 1 (4) |  | 16 |  |
|  |  |  |  |  | & | Mid | 28099.32 | 2080821 | 27904.92 | 2077581 | 102 |  |  | 22477 | 2080315 | 11 |  | 1 |  | 0 (0) |  | 206 |  |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27526.68 | 2071277 | 504 |  |  | 22489 | 2083771 | 7 |  | 2 |  | 1 (4) |  | 1020 |  |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink | Low | 27550.08 | 2071667 | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 | 4 |  | 7 |  | 1 (4) |  | 22 |  |
| +100+100 |  |  |  |  | & | Mid | 27725.04 | 2074583 | 27530.64 | 2071343 | 102 |  |  | 22456 | 2074267 | 10 |  | 5 |  | 1 (4) |  | 222 |  |
| +100 |  |  |  |  | Uplink | High | 27900.12 | 2077501 | 27126.84 | 2064613 | 504 |  |  | 22466 | 2077147 | 3 |  | 4 |  | 1 (4) |  | 1024 |  |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 27650.04 | 2073333 | 27602.52 | 2072541 | 0 | 120 |  | 22451 | 2072827 | 11 |  | 1 |  | 0 (0) |  | 2 |  |
|  |  |  |  |  | & | Mid | 27825 | 2076249 | 27630.6 | 2073009 | 102 |  |  | 22461 | 2075707 | 5 |  | 0 |  | 0 (0) |  | 204 |  |
|  |  |  |  |  | Uplink | High | 28000.08 | 2079167 | 27226.8 | 2066279 | 504 |  |  | 22472 | 2078875 | 10 |  | 6 |  | 1 (4) |  | 1028 |  |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 27750 | 2074999 | 27702.48 | 2074207 | 0 | 120 |  | 22457 | 2074555 | 6 |  | 0 |  | 1 (4) |  | 8 |  |
|  |  |  |  |  | & | Mid | 27924.96 | 2077915 | 27730.56 | 2074675 | 102 |  |  | 22467 | 2077435 | 0 |  | 3 |  | 0 (0) |  | 210 |  |
|  |  |  |  |  | Uplink | High | 28100.04 | 2080833 | 27326.76 | 2067945 | 504 |  |  | 22477 | 2080315 | 5 |  | 1 |  | 0 (0) |  | 1010 |  |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 120 | 66 | Downlink | Low | 27849.96 | 2076665 | 27802.44 | 2075873 | 0 | 120 |  | 22463 | 2076283 | 1 |  | 3 |  | 1 (4) |  | 14 |  |
|  |  |  |  |  | & | Mid | 28024.92 | 2079581 | 27830.52 | 2076341 | 102 |  |  | 22473 | 2079163 | 7 |  | 1 |  | 1 (4) |  | 214 |  |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 | 27426.72 | 2069611 | 504 |  |  | 22483 | 2082043 | 0 |  | 0 |  | 1 (4) |  | 1016 |  |
|  | Channel spacing CC4-CC5=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC5 | 100 | 120 | 66 | Downlink | Low | 27949.92 | 2078331 | 27902.4 | 2077539 | 0 | 120 |  | 22469 | 2078011 | 8 |  | 5 |  | 1 (4) |  | 18 |  |
|  |  |  |  |  | & | Mid | 28124.88 | 2081247 | 27930.48 | 2078007 | 102 |  |  | 22479 | 2080891 | 2 |  | 4 |  | 1 (4) |  | 220 |  |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 | 27526.68 | 2071277 | 504 |  |  | 22489 | 2083771 | 7 |  | 2 |  | 1 (4) |  | 1020 |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.5.10	CA_n261K

FFS

4.3.1.2.3.5.11	CA_n261L

FFS

4.3.1.2.3.5.12	CA_n261M

FFS

4.3.1.2.3.5.13	CA_n261O

Table 4.3.1.2.3.5.13-1: NR Intra-Band contiguous CA configuration CA_n261O (PCC=CC1 and SCC=CC2), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 CC2 | 50 100 | 60 | 66 132 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid, High range in Table 4.3.1.2.3.5.6-1 for CBW combination 50+100, CC1, CC2 and SCS=60 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 100+50 | CC1 | 100 | 60 | 132 | Downlink | Low | 27550.08 | 2071667 |  | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 8 |  | 14 | 1 (8) | 22 |
|  |  |  |  |  | & | Mid | 27900 | 2077499 |  | 27779.04 | 2075483 | 102 |  |  | 22466 | 2077147 |  | 8 |  | 8 | 1 (8) | 118 |
|  |  |  |  |  | Uplink | High | 28250.52 | 2083341 |  | 27840.12 | 2076501 | 504 |  |  | 22486 | 2082907 |  | 10 |  | 1 | 1 (8) | 513 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 60 | 66 | Downlink | Low | 27624.48 | 2072907 |  | 27600.72 | 2072511 | 0 | 120 |  | 22451 | 2072827 |  | 4 |  | 6 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27974.4 | 2078739 |  | 27877.2 | 2077119 | 102 |  |  | 22471 | 2078587 |  | 4 |  | 0 | 0 (0) | 102 |
|  |  |  |  |  | Uplink | High | 28324.92 | 2084581 |  | 27938.28 | 2078137 | 504 |  |  | 22492 | 2084635 |  | 6 |  | 9 | 1 (8) | 521 |
| 100+100 | CC1 CC2 | 100 100 | 60 | 132 132 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid, High range in Table 4.3.1.2.3.5.6-1 for CBW combination 100+100, CC1, CC2 and SCS=60 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.5.13-2: NR Intra-Band contiguous CA configuration CA_n261O (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 CC2 | 50 100 | 120 | 32 66 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid, High range in Table 4.3.1.2.3.5.6-2 for CBW combination 50+100, CC1, CC2 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 100+50 | CC1 | 100 | 120 | 66 | Downlink | Low | 27550.08 | 2071667 |  | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 4 |  | 7 | 1 (4) | 22 |
|  |  |  |  |  | & | Mid | 27900 | 2077499 |  | 27705.6 | 2074259 | 102 |  |  | 22466 | 2077147 |  | 4 |  | 4 | 1 (4) | 220 |
|  |  |  |  |  | Uplink | High | 28250.52 | 2083341 |  | 27477.24 | 2070453 | 504 |  |  | 22486 | 2082907 |  | 11 |  | 0 | 1 (4) | 1016 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 120 | 32 | Downlink | Low | 27624.48 | 2072907 |  | 27601.44 | 2072523 | 0 | 120 |  | 22451 | 2072827 |  | 8 |  | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 27974.4 | 2078739 |  | 27804.48 | 2075907 | 102 |  |  | 22472 | 2078875 |  | 8 |  | 7 | 1 (4) | 226 |
|  |  |  |  |  | Uplink | High | 28324.92 | 2084581 |  | 27576.12 | 2072101 | 504 |  |  | 22492 | 2084635 |  | 3 |  | 4 | 1 (4) | 1024 |
| 100+100 | CC1 CC2 | 100 100 | 120 | 66 66 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid, High range in Table 4.3.1.2.3.5.6-2 for CBW combination 100+100, CC1, CC2 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.5.14	CA_n261P

Table 4.3.1.2.3.5.14-1: NR Intra-Band contiguous CA configuration CA_n261P (PCC=CC1 and SCC=CC2, CC3), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+50 | CC1 | 50 | 60 | 66 | Downlink | Low | 27525 | 2071249 |  | 27501.24 | 2070853 | 0 | 120 |  | 22446 | 2071387 |  | 6 |  | 16 | 1 (8) | 24 |
| +50 |  |  |  |  | & | Mid | 27874.92 | 2077081 |  | 27777.72 | 2075461 | 102 |  |  | 22466 | 2077147 |  | 6 |  | 10 | 1 (8) | 120 |
|  |  |  |  |  | Uplink | High | 28225.08 | 2082917 |  | 27838.44 | 2076473 | 504 |  |  | 22486 | 2082907 |  | 2 |  | 4 | 1 (8) | 516 |
|  | Channel spacing CC1-CC2=49.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 60 | 66 | Downlink | Low | 27574.92 | 2072081 |  | 27551.16 | 2071685 | 0 | 120 |  | 22448 | 2071963 |  | 2 |  | 3 | 0 (0) | 3 |
|  |  |  |  |  | & | Mid | 27924.84 | 2077913 |  | 27827.64 | 2076293 | 102 |  |  | 22469 | 2078011 |  | 2 |  | 13 | 1 (8) | 123 |
|  |  |  |  |  | Uplink | High | 28275 | 2083749 |  | 27888.36 | 2077305 | 504 |  |  | 22489 | 2083771 |  | 10 |  | 6 | 1 (8) | 518 |
|  | Channel spacing CC2-CC3=49.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 50 | 60 | 66 | Downlink | Low | 27624.84 | 2072913 |  | 27601.08 | 2072517 | 0 | 120 |  | 22451 | 2072827 |  | 10 |  | 5 | 0 (0) | 5 |
|  |  |  |  |  | & | Mid | 27974.76 | 2078745 |  | 27877.56 | 2077125 | 102 |  |  | 22472 | 2078875 |  | 10 |  | 15 | 1 (8) | 125 |
|  |  |  |  |  | Uplink | High | 28324.92 | 2084581 |  | 27938.28 | 2078137 | 504 |  |  | 22492 | 2084635 |  | 6 |  | 9 | 1 (8) | 521 |
| 50+50 | CC1 | 50 | 60 | 66 | Downlink | Low | 27525 | 2071249 |  | 27501.24 | 2070853 | 0 | 120 |  | 22446 | 2071387 |  | 6 |  | 16 | 1 (8) | 24 |
| +100 |  |  |  |  | & | Mid | 27849.96 | 2076665 |  | 27752.76 | 2075045 | 102 |  |  | 22464 | 2076571 |  | 2 |  | 5 | 0 (0) | 107 |
|  |  |  |  |  | Uplink | High | 28175.64 | 2082093 |  | 27789 | 2075649 | 504 |  |  | 22483 | 2082043 |  | 10 |  | 0 | 1 (8) | 512 |
|  | Channel spacing CC1-CC2=49.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 60 | 66 | Downlink | Low | 27574.92 | 2072081 |  | 27551.16 | 2071685 | 0 | 120 |  | 22448 | 2071963 |  | 2 |  | 3 | 0 (0) | 3 |
|  |  |  |  |  | & | Mid | 27899.88 | 2077497 |  | 27802.68 | 2075877 | 102 |  |  | 22467 | 2077435 |  | 10 |  | 7 | 0 (0) | 109 |
|  |  |  |  |  | Uplink | High | 28225.56 | 2082925 |  | 27838.92 | 2076481 | 504 |  |  | 22486 | 2082907 |  | 6 |  | 3 | 1 (8) | 515 |
|  | Channel spacing CC2-CC3=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 27649.32 | 2073321 |  | 27601.8 | 2072529 | 0 | 120 |  | 22451 | 2072827 |  | 10 |  | 4 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 27974.28 | 2078737 |  | 27853.32 | 2076721 | 102 |  |  | 22470 | 2078299 |  | 6 |  | 1 | 1 (8) | 111 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 |  | 27889.56 | 2077325 | 504 |  |  | 22489 | 2083771 |  | 2 |  | 5 | 1 (8) | 517 |
| 50+100 | CC1 | 50 | 60 | 66 | Downlink | Low | 27525 | 2071249 |  | 27501.24 | 2070853 | 0 | 120 |  | 22446 | 2071387 |  | 6 |  | 16 | 1 (8) | 24 |
| +50 |  |  |  |  | & | Mid | 27849.96 | 2076665 |  | 27752.76 | 2075045 | 102 |  |  | 22464 | 2076571 |  | 2 |  | 5 | 0 (0) | 107 |
|  |  |  |  |  | Uplink | High | 28176.12 | 2082101 |  | 27789.48 | 2075657 | 504 |  |  | 22483 | 2082043 |  | 2 |  | 0 | 1 (8) | 512 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 27599.4 | 2072489 |  | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 2 |  | 2 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27924.36 | 2077905 |  | 27803.4 | 2075889 | 102 |  |  | 22467 | 2077435 |  | 10 |  | 6 | 0 (0) | 108 |
|  |  |  |  |  | Uplink | High | 28250.52 | 2083341 |  | 27840.12 | 2076501 | 504 |  |  | 22486 | 2082907 |  | 10 |  | 1 | 1 (8) | 513 |
|  | Channel spacing CC2-CC3=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 50 | 60 | 66 | Downlink | Low | 27673.8 | 2073729 |  | 27650.04 | 2073333 | 0 | 120 |  | 22454 | 2073691 |  | 10 |  | 1 | 1 (8) | 9 |
|  |  |  |  |  | & | Mid | 27998.76 | 2079145 |  | 27901.56 | 2077525 | 102 |  |  | 22473 | 2079163 |  | 6 |  | 6 | 1 (8) | 116 |
|  |  |  |  |  | Uplink | High | 28324.92 | 2084581 |  | 27938.28 | 2078137 | 504 |  |  | 22492 | 2084635 |  | 6 |  | 9 | 1 (8) | 521 |
| 50+100 | CC1 | 50 | 60 | 66 | Downlink | Low | 27525 | 2071249 |  | 27501.24 | 2070853 | 0 | 120 |  | 22446 | 2071387 |  | 6 |  | 16 | 1 (8) | 24 |
| +100 |  |  |  |  | & | Mid | 27825 | 2076249 |  | 27727.8 | 2074629 | 102 |  |  | 22463 | 2076283 |  | 10 |  | 7 | 1 (8) | 117 |
|  |  |  |  |  | Uplink | High | 28125.6 | 2081259 |  | 27738.96 | 2074815 | 504 |  |  | 22480 | 2081179 |  | 4 |  | 6 | 0 (0) | 510 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 27599.4 | 2072489 |  | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 2 |  | 2 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27899.4 | 2077489 |  | 27778.44 | 2075473 | 102 |  |  | 22466 | 2077147 |  | 6 |  | 9 | 1 (8) | 119 |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 |  | 27789.6 | 2075659 | 504 |  |  | 22483 | 2082043 |  | 0 |  | 0 | 1 (8) | 512 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 27699.36 | 2074155 |  | 27651.84 | 2073363 | 0 | 120 |  | 22454 | 2073691 |  | 4 |  | 7 | 0 (0) | 7 |
|  |  |  |  |  | & | Mid | 27999.36 | 2079155 |  | 27878.4 | 2077139 | 102 |  |  | 22472 | 2078875 |  | 8 |  | 14 | 1 (8) | 124 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 |  | 27889.56 | 2077325 | 504 |  |  | 22489 | 2083771 |  | 2 |  | 5 | 1 (8) | 517 |
| 100+50 | CC1 | 100 | 60 | 132 | Downlink | Low | 27550.08 | 2071667 |  | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 8 |  | 14 | 1 (8) | 22 |
| +50 |  |  |  |  | & | Mid | 27874.92 | 2077081 |  | 27753.96 | 2075065 | 102 |  |  | 22464 | 2076571 |  | 6 |  | 3 | 0 (0) | 105 |
|  |  |  |  |  | Uplink | High | 28200.6 | 2082509 |  | 27790.2 | 2075669 | 504 |  |  | 22483 | 2082043 |  | 2 |  | 7 | 0 (0) | 511 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 60 | 66 | Downlink | Low | 27624.48 | 2072907 |  | 27600.72 | 2072511 | 0 | 120 |  | 22451 | 2072827 |  | 4 |  | 6 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27949.32 | 2078321 |  | 27852.12 | 2076701 | 102 |  |  | 22470 | 2078299 |  | 2 |  | 3 | 1 (8) | 113 |
|  |  |  |  |  | Uplink | High | 28275 | 2083749 |  | 27888.36 | 2077305 | 504 |  |  | 22489 | 2083771 |  | 10 |  | 6 | 1 (8) | 518 |
|  | Channel spacing CC2-CC3=49.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 50 | 60 | 66 | Downlink | Low | 27674.4 | 2073739 |  | 27650.64 | 2073343 | 0 | 120 |  | 22454 | 2073691 |  | 0 |  | 1 | 1 (8) | 9 |
|  |  |  |  |  | & | Mid | 27999.24 | 2079153 |  | 27902.04 | 2077533 | 102 |  |  | 22473 | 2079163 |  | 10 |  | 5 | 1 (8) | 115 |
|  |  |  |  |  | Uplink | High | 28324.92 | 2084581 |  | 27938.28 | 2078137 | 504 |  |  | 22492 | 2084635 |  | 6 |  | 9 | 1 (8) | 521 |
| 100+50 | CC1 | 100 | 60 | 132 | Downlink | Low | 27550.08 | 2071667 |  | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 8 |  | 14 | 1 (8) | 22 |
| +100 |  |  |  |  | & | Mid | 27849.96 | 2076665 |  | 27729 | 2074649 | 102 |  |  | 22463 | 2076283 |  | 2 |  | 6 | 1 (8) | 116 |
|  |  |  |  |  | Uplink | High | 28151.16 | 2081685 |  | 27740.76 | 2074845 | 504 |  |  | 22480 | 2081179 |  | 10 |  | 3 | 0 (0) | 507 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 60 | 66 | Downlink | Low | 27624.48 | 2072907 |  | 27600.72 | 2072511 | 0 | 120 |  | 22451 | 2072827 |  | 4 |  | 6 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27924.36 | 2077905 |  | 27827.16 | 2076285 | 102 |  |  | 22469 | 2078011 |  | 10 |  | 13 | 1 (8) | 123 |
|  |  |  |  |  | Uplink | High | 28225.56 | 2082925 |  | 27838.92 | 2076481 | 504 |  |  | 22486 | 2082907 |  | 6 |  | 3 | 1 (8) | 515 |
|  | Channel spacing CC2-CC3=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 27698.88 | 2074147 |  | 27651.36 | 2073355 | 0 | 120 |  | 22454 | 2073691 |  | 0 |  | 0 | 1 (8) | 8 |
|  |  |  |  |  | & | Mid | 27998.76 | 2079145 |  | 27877.8 | 2077129 | 102 |  |  | 22472 | 2078875 |  | 6 |  | 15 | 1 (8) | 125 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 |  | 27889.56 | 2077325 | 504 |  |  | 22489 | 2083771 |  | 2 |  | 5 | 1 (8) | 517 |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink | Low | 27550.08 | 2071667 |  | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 8 |  | 14 | 1 (8) | 22 |
| +50 |  |  |  |  | & | Mid | 27849.96 | 2076665 |  | 27729 | 2074649 | 102 |  |  | 22463 | 2076283 |  | 2 |  | 6 | 1 (8) | 116 |
|  |  |  |  |  | Uplink | High | 28150.56 | 2081675 |  | 27740.16 | 2074835 | 504 |  |  | 22480 | 2081179 |  | 8 |  | 4 | 0 (0) | 508 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 27650.04 | 2073333 |  | 27602.52 | 2072541 | 0 | 120 |  | 22451 | 2072827 |  | 10 |  | 3 | 0 (0) | 3 |
|  |  |  |  |  | & | Mid | 27949.92 | 2078331 |  | 27828.96 | 2076315 | 102 |  |  | 22469 | 2078011 |  | 4 |  | 11 | 1 (8) | 121 |
|  |  |  |  |  | Uplink | High | 28250.52 | 2083341 |  | 27840.12 | 2076501 | 504 |  |  | 22486 | 2082907 |  | 10 |  | 1 | 1 (8) | 513 |
|  | Channel spacing CC2-CC3=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 50 | 60 | 66 | Downlink | Low | 27724.44 | 2074573 |  | 27700.68 | 2074177 | 0 | 120 |  | 22457 | 2074555 |  | 6 |  | 3 | 1 (8) | 11 |
|  |  |  |  |  | & | Mid | 28024.32 | 2079571 |  | 27927.12 | 2077951 | 102 |  |  | 22474 | 2079451 |  | 0 |  | 3 | 0 (0) | 105 |
|  |  |  |  |  | Uplink | High | 28324.92 | 2084581 |  | 27938.28 | 2078137 | 504 |  |  | 22492 | 2084635 |  | 6 |  | 9 | 1 (8) | 521 |
| 100+100 +100 | CC1 CC2 CC3 | 100 100 100 | 60 | 132 132 132 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid, High range in Table 4.3.1.2.3.5.7-1 for CBW combination 100+100+100, CC1, CC2, CC3 and SCS=60 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.5.14-2: NR Intra-Band contiguous CA configuration CA_n261P (PCC=CC1 and SCC=CC2, CC3), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+50 | CC1 | 50 | 120 | 32 | Downlink | Low | 27525 | 2071249 |  | 27501.96 | 2070865 | 0 | 120 |  | 22446 | 2071387 |  | 9 |  | 7 | 1 (4) | 22 |
| +50 |  |  |  |  | & | Mid | 27875.04 | 2077083 |  | 27705.12 | 2074251 | 102 |  |  | 22466 | 2077147 |  | 8 |  | 4 | 1 (4) | 220 |
|  |  |  |  |  | Uplink | High | 28225.08 | 2082917 |  | 27476.28 | 2070437 | 504 |  |  | 22486 | 2082907 |  | 7 |  | 1 | 1 (4) | 1018 |
|  | Channel spacing CC1-CC2=49.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 120 | 32 | Downlink | Low | 27574.92 | 2072081 |  | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 1 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27924.96 | 2077915 |  | 27755.04 | 2075083 | 102 |  |  | 22469 | 2078011 |  | 0 |  | 6 | 1 (4) | 224 |
|  |  |  |  |  | Uplink | High | 28275 | 2083749 |  | 27526.2 | 2071269 | 504 |  |  | 22489 | 2083771 |  | 11 |  | 2 | 1 (4) | 1020 |
|  | Channel spacing CC2-CC3=49.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 50 | 120 | 32 | Downlink | Low | 27624.84 | 2072913 |  | 27601.8 | 2072529 | 0 | 120 |  | 22451 | 2072827 |  | 5 |  | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 27974.88 | 2078747 |  | 27804.96 | 2075915 | 102 |  |  | 22472 | 2078875 |  | 4 |  | 7 | 1 (4) | 226 |
|  |  |  |  |  | Uplink | High | 28324.92 | 2084581 |  | 27576.12 | 2072101 | 504 |  |  | 22492 | 2084635 |  | 3 |  | 4 | 1 (4) | 1024 |
| 50+50 | CC1 | 50 | 120 | 32 | Downlink | Low | 27525 | 2071249 |  | 27501.96 | 2070865 | 0 | 120 |  | 22446 | 2071387 |  | 9 |  | 7 | 1 (4) | 22 |
| +100 |  |  |  |  | & | Mid | 27849.96 | 2076665 |  | 27680.04 | 2073833 | 102 |  |  | 22464 | 2076571 |  | 1 |  | 2 | 0 (0) | 208 |
|  |  |  |  |  | Uplink | High | 28175.64 | 2082093 |  | 27426.84 | 2069613 | 504 |  |  | 22483 | 2082043 |  | 11 |  | 3 | 0 (0) | 1014 |
|  | Channel spacing CC1-CC2=49.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 120 | 32 | Downlink | Low | 27574.92 | 2072081 |  | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 1 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27899.88 | 2077497 |  | 27729.96 | 2074665 | 102 |  |  | 22467 | 2077435 |  | 5 |  | 3 | 0 (0) | 210 |
|  |  |  |  |  | Uplink | High | 28225.56 | 2082925 |  | 27476.76 | 2070445 | 504 |  |  | 22486 | 2082907 |  | 3 |  | 1 | 1 (4) | 1018 |
|  | Channel spacing CC2-CC3=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 27649.32 | 2073321 |  | 27601.8 | 2072529 | 0 | 120 |  | 22451 | 2072827 |  | 5 |  | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 27974.28 | 2078737 |  | 27779.88 | 2075497 | 102 |  |  | 22470 | 2078299 |  | 9 |  | 0 | 1 (4) | 212 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 |  | 27526.68 | 2071277 | 504 |  |  | 22489 | 2083771 |  | 7 |  | 2 | 1 (4) | 1020 |
| 50+100 | CC1 | 50 | 120 | 32 | Downlink | Low | 27525 | 2071249 |  | 27501.96 | 2070865 | 0 | 120 |  | 22446 | 2071387 |  | 9 |  | 7 | 1 (4) | 22 |
| +50 |  |  |  |  | & | Mid | 27849.96 | 2076665 |  | 27680.04 | 2073833 | 102 |  |  | 22464 | 2076571 |  | 1 |  | 2 | 0 (0) | 208 |
|  |  |  |  |  | Uplink | High | 28176.12 | 2082101 |  | 27427.32 | 2069621 | 504 |  |  | 22483 | 2082043 |  | 7 |  | 3 | 0 (0) | 1014 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 27599.4 | 2072489 |  | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 1 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27924.36 | 2077905 |  | 27729.96 | 2074665 | 102 |  |  | 22467 | 2077435 |  | 5 |  | 3 | 0 (0) | 210 |
|  |  |  |  |  | Uplink | High | 28250.52 | 2083341 |  | 27477.24 | 2070453 | 504 |  |  | 22486 | 2082907 |  | 11 |  | 0 | 1 (4) | 1016 |
|  | Channel spacing CC2-CC3=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 50 | 120 | 32 | Downlink | Low | 27673.8 | 2073729 |  | 27650.76 | 2073345 | 0 | 120 |  | 22454 | 2073691 |  | 5 |  | 0 | 1 (4) | 8 |
|  |  |  |  |  | & | Mid | 27998.76 | 2079145 |  | 27828.84 | 2076313 | 102 |  |  | 22473 | 2079163 |  | 9 |  | 2 | 1 (4) | 216 |
|  |  |  |  |  | Uplink | High | 28324.92 | 2084581 |  | 27576.12 | 2072101 | 504 |  |  | 22492 | 2084635 |  | 3 |  | 4 | 1 (4) | 1024 |
| 50+100 | CC1 | 50 | 120 | 32 | Downlink | Low | 27525 | 2071249 |  | 27501.96 | 2070865 | 0 | 120 |  | 22446 | 2071387 |  | 9 |  | 7 | 1 (4) | 22 |
| +100 |  |  |  |  | & | Mid | 27825 | 2076249 |  | 27655.08 | 2073417 | 102 |  |  | 22463 | 2076283 |  | 5 |  | 3 | 1 (4) | 218 |
|  |  |  |  |  | Uplink | High | 28125.6 | 2081259 |  | 27376.8 | 2068779 | 504 |  |  | 22480 | 2081179 |  | 8 |  | 2 | 0 (0) | 1012 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 27599.4 | 2072489 |  | 27551.88 | 2071697 | 0 | 120 |  | 22448 | 2071963 |  | 1 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27899.4 | 2077489 |  | 27705 | 2074249 | 102 |  |  | 22466 | 2077147 |  | 9 |  | 4 | 1 (4) | 220 |
|  |  |  |  |  | Uplink | High | 28200 | 2082499 |  | 27426.72 | 2069611 | 504 |  |  | 22483 | 2082043 |  | 0 |  | 0 | 1 (4) | 1016 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 27699.36 | 2074155 |  | 27651.84 | 2073363 | 0 | 120 |  | 22454 | 2073691 |  | 8 |  | 3 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27999.36 | 2079155 |  | 27804.96 | 2075915 | 102 |  |  | 22472 | 2078875 |  | 4 |  | 7 | 1 (4) | 226 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 |  | 27526.68 | 2071277 | 504 |  |  | 22489 | 2083771 |  | 7 |  | 2 | 1 (4) | 1020 |
| 100+50 | CC1 | 100 | 120 | 66 | Downlink | Low | 27550.08 | 2071667 |  | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 4 |  | 7 | 1 (4) | 22 |
| +50 |  |  |  |  | & | Mid | 27875.04 | 2077083 |  | 27680.64 | 2073843 | 102 |  |  | 22464 | 2076571 |  | 8 |  | 1 | 0 (0) | 206 |
|  |  |  |  |  | Uplink | High | 28200.6 | 2082509 |  | 27427.32 | 2069621 | 504 |  |  | 22483 | 2082043 |  | 7 |  | 3 | 0 (0) | 1014 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 120 | 32 | Downlink | Low | 27624.48 | 2072907 |  | 27601.44 | 2072523 | 0 | 120 |  | 22451 | 2072827 |  | 8 |  | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 27949.44 | 2078323 |  | 27779.52 | 2075491 | 102 |  |  | 22470 | 2078299 |  | 0 |  | 1 | 1 (4) | 214 |
|  |  |  |  |  | Uplink | High | 28275 | 2083749 |  | 27526.2 | 2071269 | 504 |  |  | 22489 | 2083771 |  | 11 |  | 2 | 1 (4) | 1020 |
|  | Channel spacing CC2-CC3=49.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 50 | 120 | 32 | Downlink | Low | 27674.4 | 2073739 |  | 27651.36 | 2073355 | 0 | 120 |  | 22454 | 2073691 |  | 0 |  | 0 | 1 (4) | 8 |
|  |  |  |  |  | & | Mid | 27999.36 | 2079155 |  | 27829.44 | 2076323 | 102 |  |  | 22473 | 2079163 |  | 4 |  | 2 | 1 (4) | 216 |
|  |  |  |  |  | Uplink | High | 28324.92 | 2084581 |  | 27576.12 | 2072101 | 504 |  |  | 22492 | 2084635 |  | 3 |  | 4 | 1 (4) | 1024 |
| 100+50 | CC1 | 100 | 120 | 66 | Downlink | Low | 27550.08 | 2071667 |  | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 4 |  | 7 | 1 (4) | 22 |
| +100 |  |  |  |  | & | Mid | 27849.96 | 2076665 |  | 27655.56 | 2073425 | 102 |  |  | 22463 | 2076283 |  | 1 |  | 3 | 1 (4) | 218 |
|  |  |  |  |  | Uplink | High | 28151.16 | 2081685 |  | 27377.88 | 2068797 | 504 |  |  | 22480 | 2081179 |  | 11 |  | 1 | 0 (0) | 1010 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 120 | 32 | Downlink | Low | 27624.48 | 2072907 |  | 27601.44 | 2072523 | 0 | 120 |  | 22451 | 2072827 |  | 8 |  | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 27924.36 | 2077905 |  | 27754.44 | 2075073 | 102 |  |  | 22469 | 2078011 |  | 5 |  | 6 | 1 (4) | 224 |
|  |  |  |  |  | Uplink | High | 28225.56 | 2082925 |  | 27476.76 | 2070445 | 504 |  |  | 22486 | 2082907 |  | 3 |  | 1 | 1 (4) | 1018 |
|  | Channel spacing CC2-CC3=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 27698.88 | 2074147 |  | 27651.36 | 2073355 | 0 | 120 |  | 22454 | 2073691 |  | 0 |  | 0 | 1 (4) | 8 |
|  |  |  |  |  | & | Mid | 27998.76 | 2079145 |  | 27804.36 | 2075905 | 102 |  |  | 22472 | 2078875 |  | 9 |  | 7 | 1 (4) | 226 |
|  |  |  |  |  | Uplink | High | 28299.96 | 2084165 |  | 27526.68 | 2071277 | 504 |  |  | 22489 | 2083771 |  | 7 |  | 2 | 1 (4) | 1020 |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink | Low | 27550.08 | 2071667 |  | 27502.56 | 2070875 | 0 | 120 |  | 22446 | 2071387 |  | 4 |  | 7 | 1 (4) | 22 |
| +50 |  |  |  |  | & | Mid | 27849.96 | 2076665 |  | 27655.56 | 2073425 | 102 |  |  | 22463 | 2076283 |  | 1 |  | 3 | 1 (4) | 218 |
|  |  |  |  |  | Uplink | High | 28150.56 | 2081675 |  | 27377.28 | 2068787 | 504 |  |  | 22480 | 2081179 |  | 4 |  | 2 | 0 (0) | 1012 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 27650.04 | 2073333 |  | 27602.52 | 2072541 | 0 | 120 |  | 22451 | 2072827 |  | 11 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27949.92 | 2078331 |  | 27755.52 | 2075091 | 102 |  |  | 22469 | 2078011 |  | 8 |  | 5 | 1 (4) | 222 |
|  |  |  |  |  | Uplink | High | 28250.52 | 2083341 |  | 27477.24 | 2070453 | 504 |  |  | 22486 | 2082907 |  | 11 |  | 0 | 1 (4) | 1016 |
|  | Channel spacing CC2-CC3=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 50 | 120 | 32 | Downlink | Low | 27724.44 | 2074573 |  | 27701.4 | 2074189 | 0 | 120 |  | 22457 | 2074555 |  | 3 |  | 1 | 1 (4) | 10 |
|  |  |  |  |  | & | Mid | 28024.32 | 2079571 |  | 27854.4 | 2076739 | 102 |  |  | 22474 | 2079451 |  | 0 |  | 1 | 0 (0) | 206 |
|  |  |  |  |  | Uplink | High | 28324.92 | 2084581 |  | 27576.12 | 2072101 | 504 |  |  | 22492 | 2084635 |  | 3 |  | 4 | 1 (4) | 1024 |
| 100+100 +100 | CC1 CC2 CC3 | 100 100 100 | 120 | 66 66 66 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid, High range in Table 4.3.1.2.3.5.7-2 for CBW combination 100+100+100, CC1, CC2, CC3 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.5.15	CA_n261Q

FFS