3GPP TSG-RAN5 Meeting #91-e	R5-212694

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1871 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test frequencies for DC_(n)41AA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The CBW combinations for DC_(n)41AA are specified in TS 38.101-3 Table 5.3B.1.2-1 as: The E-UTRA CBWs are limited to CBW=20 MHz for BCS0 and BCS1 in Rel-15 and CBW=10MHz was introduced by BCS2 in Rel-16. The NR CBWs are limited to CBW=40,50,60,80 and 100 MHz for BCS0 and BCS1 in Rel-15 and NR CBW=10,20 and 30 MHz with E-UTRA CBW=20Mhz and NR CBW=20 and 30 MHz with E-UTRA CBW=10MHz was introduced in Rel-16. The Supported NR CBW vs SCS are specified in TS 38.101-1 Table 5.3.5-1 as: In TS 38.508-1 clause 4.3.1.4.2.41.1 test frequencies for E-UTRA NR CBW combinations with E-UTRA CBWs=5, 10, 15 and 20 MHz, and NR CBWs= 10, 15, 20, 40, 50, 60, 80, 90 and 100 MHz are specified. There is a need to align clause 4.3.1.4.2.41.1 by removing test frequencies for E-UTRA and NR CBW Combinations with E-UTRA CBWs 5 MHz and 15 MHz, and NR CBW= 15 MHz and 90 MHz. - The test frequency tables for CA_(n)41AA soes not show which channel spacing between E-UTRA CC and NR CC have been used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Table 4.3.1.4.2.41.1-1 SCS=15 kHz, NR CC at the band edge: - Removed all E-UTRA NR CBW combinations with E-UTRA CBW=5 MHz - Removed all E-UTRA NR CBW combinations with E-UTRA CBW=15 MHz - Removed all E-UTRA NR CBW combinations with NR CBW=15 MHz - Removed all E-UTRA NR CBW combinations with NR CBW=90 MHz - Removed CBW combo 10+10 Table 4.3.1.4.2.41.1-2 SCS=30 kHz, NR CC at the band edge: - Removed all E-UTRA NR CBW combinations with E-UTRA CBW=5 MHz - Removed all E-UTRA NR CBW combinations with E-UTRA CBW=15 MHz - Removed all E-UTRA NR CBW combinations with NR CBW=15 MHz - Removed all E-UTRA NR CBW combinations with NR CBW=90 MHz - Removed CBW combo 10+10 Table 4.3.1.4.2.41.1-3 SCS=60 kHz, NR CC at the band edge: - Removed all E-UTRA NR CBW combinations with E-UTRA CBW=5 MHz - Removed all E-UTRA NR CBW combinations with E-UTRA CBW=15 MHz - Removed all E-UTRA NR CBW combinations with NR CBW=15 MHz - Removed all E-UTRA NR CBW combinations with NR CBW=90 MHz - Removed CBW combo 10+10 - Added information of used channel spacing between E-UTRA and NR CCs in all tables for CA_(n)41AA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Invalid E-UTRA NR CBW combinations remains in TS 38.508-1. - Test frequency tables fro CA_(n)41AA does not show which channel spacing between E-UTRA CC and NR CC have been used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.4.2.41.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The existing E-UTRA NR CBW combinations belonging to DC_(n)41AA BCS2 introduced in Rel-16 have been kept in the CR. There are a number of additional E-UTRA NR CBW combinations for DC_(n)41AA BCS2 missing in clause 4.3.1.4.2.41.1 that need to be handled under work item NR_CADC_NR_LTE_DC_R16-UEConTest. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.4.2	Intra-band contiguous EN-DC configurations within FR1

###### 4.3.1.4.2.1 – 4.3.1.4.2.40	FFS

###### 4.3.1.4.2.41	Intra-band contiguous EN-DC configurations DC_(n)41

4.3.1.4.2.41.1	DC_(n)41AA

Table 4.3.1.4.2.41.1-1: EN-DC combination DC_(n)41AA, intra-band contiguous, SCS 15 kHz, 15 kHz NR raster, NR CC at the band edges

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset[RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 10MHz + NR: 20MHz | E-UTRA | 10 | 50 | Downlink | Low | 2521.200 | 39902 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2603.100 | 40721 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2664.900 | 41339 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1 = 15 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 20 | 106 | Downlink | Low | 2506.200 | 501240 | 2496.66 | 499332 | 0 | 15 | 6246 | 499710 | 6 | 0 | 0 (0) | 0 |
|  | CC1 |  |  | & | Mid | 2588.100 | 517620 | 2560.2 | 512040 | 102 |  | 6453 | 516270 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | Uplink | High | 2679.900 | 535980 | 2579.64 | 515928 | 504 |  | 6681 | 534510 | 2 | 0 | 1 (2) | 506 |
| E-UTRA: 10MHz + NR: 40MHz | E-UTRA | 10 | 50 | Downlink | Low | 2541.300 | 40103 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2613.000 | 40820 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2644.800 | 41138 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1 = 25.005 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 40 | 216 | Downlink | Low | 2516.295 | 503259 | 2496.855 | 499371 | 0 | 15 | 6249 | 499950 | 1 | 2 | 2 (4) | 6 |
|  | CC1 |  |  | & | Mid | 2587.995 | 517599 | 2550.195 | 510039 | 102 |  | 6426 | 514110 | 1 | 1 | 0 (0) | 103 |
|  |  |  |  | Uplink | High | 2669.805 | 533961 | 2559.645 | 511929 | 504 |  | 6633 | 530670 | 7 | 2 | 2 (4) | 510 |
| E-UTRA: 10MHz + NR: 50MHz | E-UTRA | 10 | 50 | Downlink | Low | 2551.200 | 40202 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2618.100 | 40871 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2634.900 | 41039 | - | - | - | - | - | - | - | - | - | - |
|  | NR | 50 | 270 | Downlink | Low | 2521.200 | 504240 | 2496.9 | 499380 | 0 | 15 | 6249 | 499950 | 10 | 1 | 2 (4) | 5 |
|  | CC1 |  |  | & | Mid | 2588.100 | 517620 | 2545.44 | 509088 | 102 |  | 6414 | 513150 | 10 | 0 | 0 (0) | 102 |
|  |  |  |  | Uplink | High | 2664.900 | 532980 | 2549.88 | 509976 | 504 |  | 6606 | 528510 | 10 | 0 | 0 (0) | 504 |
| E-UTRA: 20MHz + NR: 10MHz | E-UTRA | 20 | 100 | Downlink | Low | 2516.100 | 39851 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2598.000 | 40670 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2670.000 | 41390 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1 = 15 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 10 | 52 | Downlink | Low | 2501.100 | 500220 | 2496.42 | 499284 | 0 | 15 | 6246 | 499710 | 10 | 1 | 0 (0) | 1 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2559.96 | 511992 | 102 |  | 6450 | 516030 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  | Uplink | High | 2685.000 | 537000 | 2589.6 | 517920 | 504 |  | 6705 | 536430 | 2 | 0 | 0 (0) | 504 |
| E-UTRA: 20MHz + NR: 20MHz | E-UTRA | 20 | 100 | Downlink | Low | 2526.000 | 39950 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2603.100 | 40721 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2659.800 | 41288 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=19.995 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 20 | 106 | Downlink | Low | 2506.005 | 501201 | 2496.465 | 499293 | 0 | 15 | 6246 | 499710 | 7 | 1 | 0 (0) | 1 |
|  | CC1 |  |  | & | Mid | 2583.105 | 516621 | 2555.205 | 511041 | 102 |  | 6441 | 515310 | 7 | 2 | 2 (4) | 108 |
|  |  |  |  | Uplink | High | 2679.795 | 535959 | 2579.535 | 515907 | 504 |  | 6681 | 534510 | 9 | 0 | 1 (2) | 506 |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA | 20 | 100 | Downlink | Low | 2546.100 | 40151 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2613.000 | 40820 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2640.000 | 41090 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=30 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 40 | 216 | Downlink | Low | 2516.100 | 503220 | 2496.66 | 499332 | 0 | 15 | 6246 | 499710 | 6 | 0 | 0 (0) | 0 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2545.2 | 509040 | 102 |  | 6414 | 513150 | 2 | 0 | 1 (2) | 104 |
|  |  |  |  | Uplink | High | 2670.000 | 534000 | 2559.84 | 511968 | 504 |  | 6633 | 530670 | 6 | 1 | 2 (4) | 509 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA | 20 | 100 | Downlink | Low | 2556.000 | 40250 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2618.100 | 40871 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2629.800 | 40988 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=34.995 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 50 | 270 | Downlink | Low | 2521.005 | 504201 | 2496.705 | 499341 | 0 | 15 | 6246 | 499710 | 3 | 0 | 0 (0) | 0 |
|  | CC1 |  |  | & | Mid | 2583.105 | 516621 | 2540.445 | 508089 | 102 |  | 6402 | 512190 | 11 | 1 | 0 (0) | 103 |
|  |  |  |  | Uplink | High | 2664.795 | 532959 | 2549.775 | 509955 | 504 |  | 6606 | 528510 | 5 | 1 | 0 (0) | 505 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch. ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The nominal carrier spacing between the E-UTRA and the NR carriers is set in accordance to TS 38.101-3 [9], clause 5.4B.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.4.2.41.1-1A: EN-DC combination DC_(n)41AA, intra-band contiguous, SCS 15 kHz, 15 kHz NR raster, E-UTRA CC at the band edges

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA | 20 | 100 | Downlink | Low | 2506.200 | 39752 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2613.000 | 40820 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2679.900 | 41489 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=30 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 40 | 216 | Downlink | Low | 2536.200 | 507240 | 2516.76 | 503352 | 0 | 15 | 6297 | 503790 | 2 | 0 | 1 (2) | 2 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2545.2 | 509040 | 102 |  | 6414 | 513150 | 2 | 0 | 1 (2) | 104 |
|  |  |  |  | Uplink | High | 2649.900 | 529980 | 2539.74 | 507948 | 504 |  | 6582 | 526590 | 10 | 1 | 1 (2) | 512 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch. ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The nominal carrier spacing between the E-UTRA and the NR carriers is set in accordance to TS 38.101-3 [9], clause 5.4B.1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.4.2.41.1-2: EN-DC combination DC_(n)41AA, intra-band contiguous, SCS 30 kHz, 30 kHz NR raster, NR CC at the band edges

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 10MHz + NR: 20MHz | E-UTRA | 10 | 50 | Downlink | Low | 2521.200 | 39902 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2603.100 | 40721 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2664.900 | 41339 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=15 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 20 | 51 | Downlink | Low | 2506.200 | 501240 | 2497.02 | 499404 | 0 | 30 | 6252 | 500190 | 22 | 0 | 0 (0) | 0 |
|  | CC1 |  |  | & | Mid | 2588.100 | 517620 | 2542.2 | 508440 | 102 |  | 6456 | 516510 | 2 | 0 | 0 (0) | 204 |
|  |  |  |  | Uplink | High | 2679.900 | 535980 | 2489.28 | 497856 | 504 |  | 6687 | 534990 | 18 | 0 | 1 (1) | 1010 |
| E-UTRA: 10MHz + NR: 40MHz | E-UTRA | 10 | 50 | Downlink | Low | 2541.000 | 40100 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2613.000 | 40820 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2644.800 | 41138 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=24.99 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 40 | 106 | Downlink | Low | 2516.010 | 503202 | 2496.93 | 499386 | 0 | 30 | 6252 | 500190 | 4 | 0 | 1 (1) | 2 |
|  | CC1 |  |  | & | Mid | 2588.010 | 517602 | 2532.21 | 506442 | 102 |  | 6432 | 514590 | 4 | 0 | 1 (1) | 206 |
|  |  |  |  | Uplink | High | 2669.790 | 533958 | 2469.27 | 493854 | 504 |  | 6636 | 530910 | 16 | 0 | 0 (0) | 1008 |
| E-UTRA: 10MHz + NR: 50MHz | E-UTRA | 10 | 50 | Downlink | Low | 2551.200 | 40202 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2618.100 | 40871 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2634.900 | 41039 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=30 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 50 | 133 | Downlink | Low | 2521.200 | 504240 | 2497.26 | 499452 | 0 | 30 | 6252 | 500190 | 6 | 0 | 0 (0) | 0 |
|  | CC1 |  |  | & | Mid | 2588.100 | 517620 | 2527.44 | 505488 | 102 |  | 6420 | 513630 | 2 | 0 | 1 (1) | 206 |
|  |  |  |  | Uplink | High | 2664.900 | 532980 | 2459.52 | 491904 | 504 |  | 6612 | 528990 | 2 | 0 | 1 (1) | 1010 |
| E-UTRA: 10MHz + NR: 60MHz | E-UTRA | 10 | 50 | Downlink | Low | 2561.100 | 40301 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2622.900 | 40919 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2624.700 | 40937 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=35.01 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 60 | 162 | Downlink | Low | 2526.090 | 505218 | 2496.93 | 499386 | 0 | 30 | 6252 | 500190 | 4 | 0 | 1 (1) | 2 |
|  | CC1 |  |  | & | Mid | 2587.890 | 517578 | 2522.01 | 504402 | 102 |  | 6408 | 512670 | 20 | 0 | 2 (2) | 208 |
|  |  |  |  | Uplink | High | 2659.710 | 531942 | 2449.11 | 489822 | 504 |  | 6585 | 526830 | 0 | 0 | 0 (0) | 1008 |
| E-UTRA: 10MHz + NR: 80MHz | E-UTRA | 10 | 50 | Downlink | Low | 2581.200 | 40502 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2633.100 | 41021 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2604.900 | 40739 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=45 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 80 | 217 | Downlink | Low | 2536.200 | 507240 | 2497.14 | 499428 | 0 | 30 | 6252 | 500190 | 14 | 0 | 0 (0) | 0 |
|  | CC1 |  |  | & | Mid | 2588.100 | 517620 | 2512.32 | 502464 | 102 |  | 6384 | 510750 | 2 | 0 | 3 (3) | 210 |
|  |  |  |  | Uplink | High | 2649.900 | 529980 | 2429.4 | 485880 | 504 |  | 6537 | 522990 | 10 | 0 | 1 (1) | 1010 |
| E-UTRA: 10MHz + NR: 100MHz | E-UTRA | 10 | 50 | Downlink | Low | 2601.000 | 40700 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2643.000 | 41120 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2584.800 | 40538 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=54.99 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 100 | 273 | Downlink | Low | 2546.010 | 509202 | 2496.87 | 499374 | 0 | 30 | 6252 | 500190 | 8 | 0 | 1 (1) | 2 |
|  | CC1 |  |  | & | Mid | 2588.010 | 517602 | 2502.15 | 500430 | 102 |  | 6357 | 508590 | 8 | 0 | 1 (1) | 206 |
|  |  |  |  | Uplink | High | 2639.790 | 527958 | 2409.21 | 481842 | 504 |  | 6486 | 518910 | 20 | 0 | 0 (0) | 1008 |
| E-UTRA: 20MHz + NR: 10MHz | E-UTRA | 20 | 100 | Downlink | Low | 2516.100 | 39851 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2598.000 | 40670 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2670.000 | 41390 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=15 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 10 | 24 | Downlink | Low | 2501.100 | 500220 | 2496.78 | 499356 | 0 | 30 | 6252 | 500190 | 14 | 0 | 1 (1) | 2 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2541.96 | 508392 | 102 |  | 6456 | 516510 | 18 | 0 | 0 (0) | 204 |
|  |  |  |  | Uplink | High | 2685.000 | 537000 | 2499.24 | 499848 | 504 |  | 6711 | 536910 | 18 | 0 | 0 (0) | 1008 |
| E-UTRA: 20MHz + NR: 20MHz | E-UTRA | 20 | 100 | Downlink | Low | 2526.300 | 39953 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2603.100 | 40721 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2659.800 | 41288 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=20.01 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 20 | 51 | Downlink | Low | 2506.290 | 501258 | 2497.11 | 499422 | 0 | 30 | 6252 | 500190 | 16 | 0 | 0 (0) | 0 |
|  | CC1 |  |  | & | Mid | 2583.090 | 516618 | 2537.19 | 507438 | 102 |  | 6444 | 515550 | 16 | 0 | 0 (0) | 204 |
|  |  |  |  | Uplink | High | 2679.810 | 535962 | 2489.19 | 497838 | 504 |  | 6687 | 534990 | 0 | 0 | 2 (2) | 1012 |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA | 20 | 100 | Downlink | Low | 2546.100 | 40151 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2613.000 | 40820 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2640.000 | 41090 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=30 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 40 | 106 | Downlink | Low | 2516.100 | 503220 | 2497.02 | 499404 | 0 | 30 | 6252 | 500190 | 22 | 0 | 0 (0) | 0 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2527.2 | 505440 | 102 |  | 6420 | 513630 | 18 | 0 | 1 (1) | 206 |
|  |  |  |  | Uplink | High | 2670.000 | 534000 | 2469.48 | 493896 | 504 |  | 6636 | 530910 | 2 | 0 | 0 (0) | 1008 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA | 20 | 100 | Downlink | Low | 2556.300 | 40253 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2618.100 | 40871 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2629.800 | 40988 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=35.01 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 50 | 133 | Downlink | Low | 2521.290 | 504258 | 2497.35 | 499470 | 0 | 30 | 6252 | 500190 | 0 | 0 | 0 (0) | 0 |
|  | CC1 |  |  | & | Mid | 2583.090 | 516618 | 2522.43 | 504486 | 102 |  | 6408 | 512670 | 16 | 0 | 1 (1) | 206 |
|  |  |  |  | Uplink | High | 2664.810 | 532962 | 2459.43 | 491886 | 504 |  | 6612 | 528990 | 8 | 0 | 1 (1) | 1010 |
| E-UTRA: 20MHz + NR: 60MHz | E-UTRA | 20 | 100 | Downlink | Low | 2566.200 | 40352 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2622.900 | 40919 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2619.900 | 40889 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=39.99 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 60 | 162 | Downlink | Low | 2526.210 | 505242 | 2497.05 | 499410 | 0 | 30 | 6252 | 500190 | 20 | 0 | 0 (0) | 0 |
|  | CC1 |  |  | & | Mid | 2582.910 | 516582 | 2517.03 | 503406 | 102 |  | 6393 | 511470 | 0 | 0 | 0 (0) | 204 |
|  |  |  |  | Uplink | High | 2659.890 | 531978 | 2449.29 | 489858 | 504 |  | 6588 | 527070 | 20 | 0 | 2 (2) | 1012 |
| E-UTRA: 20MHz + NR: 80MHz | E-UTRA | 20 | 100 | Downlink | Low | 2586.300 | 40553 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2633.100 | 41021 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2599.800 | 40688 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=50.01 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 80 | 217 | Downlink | Low | 2536.290 | 507258 | 2497.23 | 499446 | 0 | 30 | 6252 | 500190 | 8 | 0 | 0 (0) | 0 |
|  | CC1 |  |  | & | Mid | 2583.090 | 516618 | 2507.31 | 501462 | 102 |  | 6369 | 509550 | 8 | 0 | 0 (0) | 204 |
|  |  |  |  | Uplink | High | 2649.810 | 529962 | 2429.31 | 485862 | 504 |  | 6537 | 522990 | 16 | 0 | 1 (1) | 1010 |
| E-UTRA: 20MHz + NR: 100MHz | E-UTRA | 20 | 100 | Downlink | Low | 2606.100 | 40751 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2643.000 | 41120 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2580.000 | 40490 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=60 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 100 | 273 | Downlink | Low | 2546.100 | 509220 | 2496.96 | 499392 | 0 | 30 | 6252 | 500190 | 2 | 0 | 1 (1) | 2 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2497.14 | 499428 | 102 |  | 6345 | 507630 | 22 | 0 | 1 (1) | 206 |
|  |  |  |  | Uplink | High | 2640.000 | 528000 | 2409.42 | 481884 | 504 |  | 6486 | 518910 | 6 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch. ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The nominal carrier spacing between the E-UTRA and the NR carriers is set in accordance to TS 38.101-3 [9], clause 5.4B.1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.4.2.41.1-2A: EN-DC combination DC_(n)41AA, intra-band contiguous, SCS 30 kHz, 30 kHz NR raster, E-UTRA CC at the band edges

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset[RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA | 20 | 100 | Downlink | Low | 2506.200 | 39752 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2613.000 | 40820 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2679.900 | 41489 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=30 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 40 | 106 | Downlink | Low | 2536.200 | 507240 | 2517.12 | 503424 | 0 | 30 | 6303 | 504270 | 18 | 0 | 1 (1) | 2 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2527.2 | 505440 | 102 |  | 6420 | 513630 | 18 | 0 | 1 (1) | 206 |
|  |  |  |  | Uplink | High | 2649.900 | 529980 | 2449.38 | 489876 | 504 |  | 6588 | 527070 | 14 | 0 | 2 (2) | 1012 |
| E-UTRA: 20MHz + NR: 60MHz | E-UTRA | 20 | 100 | Downlink | Low | 2506.200 | 39752 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2622.900 | 40919 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2679.900 | 41489 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=39.99 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 60 | 162 | Downlink | Low | 2546.190 | 509238 | 2517.03 | 503406 | 0 | 30 | 6303 | 504270 | 0 | 0 | 2 (2) | 4 |
|  | CC1 |  |  | & | Mid | 2582.910 | 516582 | 2517.03 | 503406 | 102 |  | 6393 | 511470 | 0 | 0 | 0 (0) | 204 |
|  |  |  |  | Uplink | High | 2639.910 | 527982 | 2429.31 | 485862 | 504 |  | 6537 | 522990 | 16 | 0 | 1 (1) | 1010 |
| E-UTRA: 20MHz + NR: 80MHz | E-UTRA | 20 | 100 | Downlink | Low | 2506.200 | 39752 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2633.100 | 41021 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2679.900 | 41489 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=50.01 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 80 | 217 | Downlink | Low | 2556.210 | 511242 | 2517.15 | 503430 | 0 | 30 | 6303 | 504270 | 16 | 0 | 1 (1) | 2 |
|  | CC1 |  |  | & | Mid | 2583.090 | 516618 | 2507.31 | 501462 | 102 |  | 6369 | 509550 | 8 | 0 | 0 (0) | 204 |
|  |  |  |  | Uplink | High | 2629.890 | 525978 | 2409.39 | 481878 | 504 |  | 6486 | 518910 | 8 | 0 | 0 (0) | 1008 |
| E-UTRA: 20MHz + NR: 100MHz | E-UTRA | 20 | 100 | Downlink | Low | 2506.200 | 39752 | - | - | - | - | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2643.000 | 41120 | - | - | - | - | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2679.900 | 41489 | - | - | - | - | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=60 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 100 | 273 | Downlink | Low | 2566.200 | 513240 | 2517.06 | 503412 | 0 | 30 | 6303 | 504270 | 22 | 0 | 1 (1) | 2 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2497.14 | 499428 | 102 |  | 6345 | 507630 | 22 | 0 | 1 (1) | 206 |
|  |  |  |  | Uplink | High | 2619.900 | 523980 | 2389.32 | 477864 | 504 |  | 6438 | 515070 | 18 | 0 | 2 (2) | 1012 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch. ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The nominal carrier spacing between the E-UTRA and the NR carriers is set in accordance to TS 38.101-3 [9], clause 5.4B.1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.4.2.41.1-3: EN-DC combination DC_(n)41AA, intra-band contiguous, SCS 60 kHz, 15 kHz NR raster, NR CC at the band edges without CORESET#0

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 1 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 10MHz + NR: 20MHz | E-UTRA | 10 | 50 | Downlink | Low | 2521.200 | 39902 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2603.100 | 40721 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2664.900 | 41339 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=15 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 20 | 24 | Downlink | Low | 2506.200 | 501240 | 2497.56 | 499512 | 0 | 15 | - | 499872 |
|  | CC1 |  |  | & | Mid | 2588.100 | 517620 | 2506.02 | 501204 | 102 |  | - | 516252 |
|  |  |  |  | Uplink | High | 2679.900 | 535980 | 2308.38 | 461676 | 504 |  | - | 534612 |
| E-UTRA: 10MHz + NR: 40MHz | E-UTRA | 10 | 50 | Downlink | Low | 2541.300 | 40103 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2613.000 | 40820 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2644.800 | 41138 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=25.005 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 40 | 51 | Downlink | Low | 2516.295 | 503259 | 2497.935 | 499587 | 0 | 15 | - | 499947 |
|  | CC1 |  |  | & | Mid | 2587.995 | 517599 | 2496.195 | 499239 | 102 |  | - | 514287 |
|  |  |  |  | Uplink | High | 2669.805 | 533961 | 2288.565 | 457713 | 504 |  | - | 530649 |
| E-UTRA: 10MHz + NR: 50MHz | E-UTRA | 10 | 50 | Downlink | Low | 2551.200 | 40202 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2618.100 | 40871 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2634.900 | 41039 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=30 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 50 | 65 | Downlink | Low | 2521.200 | 504240 | 2497.8 | 499560 | 0 | 15 | - | 499920 |
|  | CC1 |  |  | & | Mid | 2588.100 | 517620 | 2491.26 | 498252 | 102 |  | - | 513300 |
|  |  |  |  | Uplink | High | 2664.900 | 532980 | 2278.62 | 455724 | 504 |  | - | 528660 |
| E-UTRA: 10MHz + NR: 60MHz | E-UTRA | 10 | 50 | Downlink | Low | 2561.100 | 40301 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2622.900 | 40919 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2625.000 | 40940 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=34.995 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 60 | 79 | Downlink | Low | 2526.105 | 505221 | 2497.665 | 499533 | 0 | 15 | - | 499893 |
|  | CC1 |  |  | & | Mid | 2587.905 | 517581 | 2486.025 | 497205 | 102 |  | - | 512253 |
|  |  |  |  | Uplink | High | 2659.995 | 531999 | 2268.675 | 453735 | 504 |  | - | 526671 |
| E-UTRA: 10MHz + NR: 80MHz | E-UTRA | 10 | 50 | Downlink | Low | 2581.200 | 40502 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2633.100 | 41021 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2604.900 | 40739 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=45 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 80 | 107 | Downlink | Low | 2536.200 | 507240 | 2497.68 | 499536 | 0 | 15 | - | 499896 |
|  | CC1 |  |  | & | Mid | 2588.100 | 517620 | 2476.14 | 495228 | 102 |  | - | 510276 |
|  |  |  |  | Uplink | High | 2649.900 | 529980 | 2248.5 | 449700 | 504 |  | - | 522636 |
| E-UTRA: 10MHz + NR: 100MHz | E-UTRA | 10 | 50 | Downlink | Low | 2601.300 | 40703 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2643.000 | 41120 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2584.800 | 40538 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=55.005 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 100 | 135 | Downlink | Low | 2546.295 | 509259 | 2497.695 | 499539 | 0 | 15 | - | 499899 |
|  | CC1 |  |  | & | Mid | 2587.995 | 517599 | 2465.955 | 493191 | 102 |  | - | 508239 |
|  |  |  |  | Uplink | High | 2639.805 | 527961 | 2228.325 | 445665 | 504 |  | - | 518601 |
| E-UTRA: 20MHz + NR: 10MHz | E-UTRA | 20 | 100 | Downlink | Low | 2516.100 | 39851 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2598.000 | 40670 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2670.000 | 41390 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=15 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 10 | 11 | Downlink | Low | 2501.100 | 500220 | 2497.14 | 499428 | 0 | 15 | - | 499788 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2505.6 | 501120 | 102 |  | - | 516168 |
|  |  |  |  | Uplink | High | 2685.000 | 537000 | 2318.16 | 463632 | 504 |  | - | 536568 |
| E-UTRA: 20MHz + NR: 20MHz | E-UTRA | 20 | 100 | Downlink | Low | 2526.000 | 39950 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2603.100 | 40721 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2659.800 | 41288 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=19.995 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 20 | 24 | Downlink | Low | 2506.005 | 501201 | 2497.365 | 499473 | 0 | 15 | - | 499833 |
|  | CC1 |  |  | & | Mid | 2583.105 | 516621 | 2501.025 | 500205 | 102 |  | - | 515253 |
|  |  |  |  | Uplink | High | 2679.795 | 535959 | 2308.275 | 461655 | 504 |  | - | 534591 |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA | 20 | 100 | Downlink | Low | 2546.100 | 40151 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2613.000 | 40820 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2640.000 | 41090 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=30 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 40 | 51 | Downlink | Low | 2516.100 | 503220 | 2497.74 | 499548 | 0 | 15 | - | 499908 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2491.2 | 498240 | 102 |  | - | 513288 |
|  |  |  |  | Uplink | High | 2670.000 | 534000 | 2288.76 | 457752 | 504 |  | - | 530688 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA | 20 | 100 | Downlink | Low | 2556.000 | 40250 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2618.100 | 40871 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2629.800 | 40988 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=34.995 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 50 | 65 | Downlink | Low | 2521.005 | 504201 | 2497.605 | 499521 | 0 | 15 | - | 499881 |
|  | CC1 |  |  | & | Mid | 2583.105 | 516621 | 2486.265 | 497253 | 102 |  | - | 512301 |
|  |  |  |  | Uplink | High | 2664.795 | 532959 | 2278.515 | 455703 | 504 |  | - | 528639 |
| E-UTRA: 20MHz + NR: 60MHz | E-UTRA | 20 | 100 | Downlink | Low | 2566.200 | 40352 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2622.900 | 40919 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2619.900 | 40889 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=40.005 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 60 | 79 | Downlink | Low | 2526.195 | 505239 | 2497.755 | 499551 | 0 | 15 | - | 499911 |
|  | CC1 |  |  | & | Mid | 2582.895 | 516579 | 2481.015 | 496203 | 102 |  | - | 511251 |
|  |  |  |  | Uplink | High | 2659.905 | 531981 | 2268.585 | 453717 | 504 |  | - | 526653 |
| E-UTRA: 20MHz + NR: 80MHz | E-UTRA | 20 | 100 | Downlink | Low | 2586.000 | 40550 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2633.100 | 41021 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2599.800 | 40688 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=49.995 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 80 | 107 | Downlink | Low | 2536.005 | 507201 | 2497.485 | 499497 | 0 | 15 | - | 499857 |
|  | CC1 |  |  | & | Mid | 2583.105 | 516621 | 2471.145 | 494229 | 102 |  | - | 509277 |
|  |  |  |  | Uplink | High | 2649.795 | 529959 | 2248.395 | 449679 | 504 |  | - | 522615 |
| E-UTRA: 20MHz + NR: 100MHz | E-UTRA | 20 | 100 | Downlink | Low | 2606.100 | 40751 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2643.000 | 41120 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2580.000 | 40490 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=60 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 100 | 135 | Downlink | Low | 2546.100 | 509220 | 2497.5 | 499500 | 0 | 15 | - | 499860 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2460.96 | 492192 | 102 |  | - | 507240 |
|  |  |  |  | Uplink | High | 2640.000 | 528000 | 2228.52 | 445704 | 504 |  | - | 518640 |
| Note 1: The nominal carrier spacing between the E-UTRA and the NR carriers is set in accordance to TS 38.101-3 [9], clause 5.4B1. Note 2: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.4.2.41.1-3A: EN-DC combination DC_(n)41AA, intra-band contiguous, SCS 60 kHz, 15 kHz NR raster, E-UTRA CC at the band edges without CORESET#0

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 1 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA | 20 | 100 | Downlink | Low | 2506.200 | 39752 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2613.000 | 40820 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2679.900 | 41489 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=30 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 40 | 51 | Downlink | Low | 2536.200 | 507240 | 2517.84 | 503568 | 0 | 15 | - | 503928 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2491.2 | 498240 | 102 |  | - | 513288 |
|  |  |  |  | Uplink | High | 2649.900 | 529980 | 2268.66 | 453732 | 504 |  | - | 526668 |
| E-UTRA: 20MHz + NR: 60MHz | E-UTRA | 20 | 100 | Downlink | Low | 2506.200 | 39752 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2622.900 | 40919 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2679.900 | 41489 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=40.005 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 60 | 79 | Downlink | Low | 2546.205 | 509241 | 2517.765 | 503553 | 0 | 15 | - | 503913 |
|  | CC1 |  |  | & | Mid | 2582.895 | 516579 | 2481.015 | 496203 | 102 |  | - | 511251 |
|  |  |  |  | Uplink | High | 2639.895 | 527979 | 2248.575 | 449715 | 504 |  | - | 522651 |
| E-UTRA: 20MHz + NR: 80MHz | E-UTRA | 20 | 100 | Downlink | Low | 2506.200 | 39752 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2633.100 | 41021 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2679.900 | 41489 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=49.995 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 80 | 107 | Downlink | Low | 2556.195 | 511239 | 2517.675 | 503535 | 0 | 15 | - | 503895 |
|  | CC1 |  |  | & | Mid | 2583.105 | 516621 | 2471.145 | 494229 | 102 |  | - | 509277 |
|  |  |  |  | Uplink | High | 2629.905 | 525981 | 2228.505 | 445701 | 504 |  | - | 518637 |
| E-UTRA: 20MHz + NR: 100MHz | E-UTRA | 20 | 100 | Downlink | Low | 2506.200 | 39752 | - | - | - | - | - | - |
|  | CC1 |  |  | & | Mid | 2643.000 | 41120 | - | - | - | - | - | - |
|  |  |  |  | Uplink | High | 2679.900 | 41489 | - | - | - | - | - | - |
|  | Channel spacing E-UTRA CC1- NR CC1=60 MHz (Note 2) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 100 | 135 | Downlink | Low | 2566.200 | 513240 | 2517.6 | 503520 | 0 | 15 | - | 503880 |
|  | CC1 |  |  | & | Mid | 2583.000 | 516600 | 2460.96 | 492192 | 102 |  | - | 507240 |
|  |  |  |  | Uplink | High | 2619.900 | 523980 | 2208.42 | 441684 | 504 |  | - | 514620 |
| Note 1: The nominal carrier spacing between the E-UTRA and the NR carriers is set in accordance to TS 38.101-3 [9], clause 5.4B1. Note 2: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.4.2.42.to 4.3.1.4.2.70	FFS