3GPP TSG-RAN5 Meeting #91-e	R5-212746

# Electronic Meeting, 17th May – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0980 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of applicability for EN-DC Tx tests within FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas, CMCC, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | EN-DC Tx Anchor agnostic test cases applicability is caused confusion if EN-DC configuration with one or more LTE CC included. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated to EN-DC Tx Anchor agnostic test cases applicability with clear indication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test applicability will stay unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.2.3.2, 6.2B.3.3.2, 6.3B.1.3.2, 6.3B.3.3.2, 6.3B.4.3.2, 6.3B.8.1.3.2, 6.3B.8.2.3.2, 6.3B.8.3.3.2, 6.4B.1.3.2, 6.4B.2.3.1.2, 6.4B.2.3.2.2, 6.4B.2.3.3.2, 6.4B.2.3.4.2, 6.5B.1.3.2, 6.5B.2.3.1.2, 6.5B.2.3.2.2, 6.5B.2.3.3.2, 6.5B.4.3.2, 6.5B.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## {Start of changes}

#### 6.2B.2.3	UE Maximum Output Power reduction for Inter-Band EN-DC within FR1

6.2B.2.3.1	Test purpose

Same test purpose as in clause 6.2.2.1 in TS 38.521-1 [8] for the NR carrier.

6.2B.2.3.2	Test applicability

The requirements of this test apply to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.2B.2.3.3	Minimum conformance requirements

UE maximum output power requirement for E-UTRA single carrier and CA operation specified in subclauses 6.2.2 and 6.2.2A of TS 36.101 [10] and for NR single carrier and CA operation specified in subclause 6.2.1, 6.2A.1, and 6.2D.1 of TS 38.101-2 [9] apply.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.1.4.

<Unchanged Sections Skipped>

#### 6.2B.3.3	UE Additional Maximum Output Power reduction for Inter-Band EN-DC within FR1

6.2B.3.3.1	Test purpose

Same test purpose as in clause 6.2.3.1 in TS 38.521-1 [8] for the NR carrier.

6.2B.3.3.2	Test applicability

The requirements of this test apply to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.2B.3.3.3	Minimum conformance requirements

For inter-band EN-DC between E-UTRA and FR1 NR, UE additional maximum output power reduction specified in TS 36.101 [5] and TS 38.101-1 [2] apply for E-UTRA and NR respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.3.3.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied

<Unchanged Sections Skipped>

#### 6.3B.1.3	Minimum output power for inter-band EN-DC within FR1

6.3B.1.3.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.3B.1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.1.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

<Unchanged Sections Skipped>

#### 6.3B.3.3	Tx ON/OFF time mask for inter-band EN-DC within FR1

6.3B.3.3.1	Test purpose

Same test purpose as in clause 6.3.3.2.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.3.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.3B.3.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.3.2.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or LTE.LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

<Unchanged Sections Skipped>

#### 6.3B.4.3	PRACH Time Mask for inter-band EN-DC within FR1

6.3B.4.3.1	Test purpose

Same test purpose as in clause 6.3.3.4.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.4.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.3B.4.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.3.4.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

<Unchanged Sections Skipped>

##### 6.3B.8.1.3	Absolute power tolerance for inter-band EN-DC within FR1

6.3B.8.1.3.1	Test purpose

Same test purpose as in clause 6.3.4 in TS 38.521-1 [8] for the NR FR1 carrier(s).

6.3B.8.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.3B.8.1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

<Unchanged Sections Skipped>

##### 6.3B.8.2.3	Relative power tolerance for inter-band EN-DC within FR1

6.3B.8.2.3.1	Test purpose

Same test purpose as in clause 6.3.4 in TS 38.521-1 [8] for the NR FR1 carrier(s).

6.3B.8.2.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.3B.8.2.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

<Unchanged Sections Skipped>

##### 6.3B.8.3.3	Aggregate power tolerance for inter-band EN-DC within FR1

6.3B.8.3.3.1	Test purpose

Same test purpose as in clause 6.3.4 in TS 38.521-1 [8] for the NR FR1 carrier(s).

6.3B.8.3.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.3B.8.3.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

<Unchanged Sections Skipped>

#### 6.4B.1.3	Frequency error for Inter-band EN-DC within FR1

6.4B.1.3.1	Test purpose

Same test purpose as in clause 6.4.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.4B.1.3.3	Minimum conformance requirements

For inter-band EN-DC with uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.1 in TS 36.101 [5] and in clause 6.4.1 in TS 38.101-1 [2], respectively, with all component carriers active. If multiple component carriers are assigned to one E-UTRA band, the requirements in clauses 6.5.1A in TS 36.101 [5] apply for those component carriers, and if multiple component carriers are assigned to one NR band, the requirements in clauses 6.4A.1 in TS 38.101-1 [2] apply for those component carriers.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.4B.1.3.

<Unchanged Sections Skipped>

##### 6.4B.2.3.1	Error Vector Magnitude for inter-band EN-DC within FR1

6.4B.2.3.1.1	Test purpose

Same test purpose as in clause 6.4.2.1.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.3.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.4B.2.3.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.1.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

<Unchanged Sections Skipped>

##### 6.4B.2.3.2	Carrier Leakage for inter-band EN-DC within FR1

6.4B.2.3.2.1	Test purpose

Same test purpose as in clause 6.4.2.2.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.4B.2.3.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.2.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

<Unchanged Sections Skipped>

##### 6.4B.2.3.3	In-band Emissions for inter-band EN-DC within FR1

6.4B.2.3.3.1	Test purpose

Same test purpose as in clause 6.4.2.3.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.3.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.4B.2.3.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.3.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

<Unchanged Sections Skipped>

##### 6.4B.2.3.4	EVM Equalizer Flatness for inter-band EN-DC within FR1

6.4B.2.3.4.1	Test purpose

Same test purpose as in clause 6.4.2.4 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.3.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.4B.2.3.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.4.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

<Unchanged Sections Skipped>

#### 6.5B.1.3	Occupied bandwidth for Inter-Band EN-DC within FR1

6.5B.1.3.1	Test purpose

Same test purpose as in clause 6.5.1.1 in TS 38.521-1 [8] for the NR carrier.

6.5B.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.5B.1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.1.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.1.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

<Unchanged Sections Skipped>

##### 6.5B.2.3.1	Spectrum emissions mask for Inter-band EN-DC within FR1

6.5B.2.3.1.1	Test purpose

Same test purpose as in clause 6.5.2.2 in TS 38.521-1 [8] for the NR carrier.

6.5B.2.3.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.5B.2.3.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.2.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.3.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

<Unchanged Sections Skipped>

##### 6.5B.2.3.2	Additional Spectrum emissions mask for Inter-band EN-DC within FR1

6.5B.2.3.2.1	Test purpose

Same test purpose as in clause 6.5.2.3.1 in TS 38.521-1 [8] for the NR carrier.

6.5B.2.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.5B.2.3.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.3.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.3.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

<Unchanged Sections Skipped>

##### 6.5B.2.3.3	Adjacent channel leakage ratio for inter-band EN-DC within FR1

6.5B.2.3.3.1	Test purpose

Same test purpose as in clause 6.5.2.4.1.1 in TS 38.521-1 [8].

6.5B.2.3.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.5B.2.3.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.4.1.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.3.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied

<Unchanged Sections Skipped>

#### 6.5B.4.3	Additional Spurious Emissions for Inter-band EN-DC within FR1

6.5B.4.3.1	Test purpose

Same test purpose as in clause 6.5.3.3.1 in TS 38.521-1 [8] for the NR carrier.

6.5B.4.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.5B.4.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

<Unchanged Sections Skipped>

#### 6.5B.5.3	Inter-band EN-DC within FR1

6.5B.5.3.1	Test purpose

Same test purpose as in clause 6.5.4 in TS 38.521-1 [8] for the NR carrier.

6.5B.5.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC and one or more LTE UL CC(s).

6.5B.5.3.3	Minimum conformance requirements

The transmit intermodulation requirement specified in clauses 6.7.1 and 6.7.1A of TS 36.101 [5] and clauses 6.5.4 and 6.5A.4 of TS 38.101-1 [2] apply for each component carrier in E-UTRA bands and NR bands, respectively.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.5B.5.3.

## {End of changes}