3GPP TSG-RAN5 Meeting #91-e	R5-212758

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5316 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Annexes F1.3 and F3.3 for 7DL TCs 7.6.1A.10 7.6.2A.10 7.6.3A.10 and 7.7A.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tejet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Due to 7DL TCs 7.6.1A.10 7.6.2A.10 7.6.3A.10 and 7.7A.10 will be introduced to spec, annexes F.1.3 and F.3.3 should be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annexes F.1.3 and F.3.3 will be updated for 7DL TCs 7.6.1A.10 7.6.2A.10 7.6.3A.10 and 7.7A.10. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 7DL TCs 7.6.1A.10 7.6.2A.10 7.6.3A.10 and 7.7A.10 will still be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.3, F.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start Of Changes>

F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause |  | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 7.3 Reference sensitivity power level |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |  |  |
| <A lot of rows skipped here> |  |  |  |  |  |
| 7.6.1 In-band blocking |  | Blocking ±1.4 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB f ≤ 4.2GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1_1 In-band blocking with 4 Rx antenna ports |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1A.1 In-band blocking for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.1 for each CC |  | Same as 7.6.1 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.6.1A.2 In-band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.3 In-band blocking for CA (inter band DL CA without UL CA) |  | Blocking ±1.4 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.5 dB, 4.2GHz < f ≤ 6.0GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB f ≤ 6.0GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1A.4 In-band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.5 In-band blocking for 3DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.7 In-band blocking for 4DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.8 In-band blocking for 5DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.9 In-band blocking for 6DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.10 In-band blocking for 7DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1B In-band blocking for UL-MIMO |  | Blocking same as 7.6.1 Uplink power measurement same as 7.6.1, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.1D.1 In-band blocking for ProSe Direct Discovery |  | Blocking ±1.4 dB, f ≤ 3.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB f ≤ 4.2GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1D.2 In-band blocking for ProSe Direct Communication |  | Same as 7.6.1D.1 |  | Same as 7.6.1D.1 |  |
| 7.6.1E In-band blocking for UE category 0 |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1EB In-band blocking for UE Category 1bis |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1F In-band blocking for category NB1 and NB2 |  | Blocking ±1.4 dB, f ≤ 3.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB |  |
| 7.6.1G.1 In-band blocking for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Blocking ±2.5 dB, 4.2GHz < f ≤ 6.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.5dB 4.2GHz < f ≤ 6.0GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1G.3 In-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.6.1G.1 |  | Same as 7.6.1G.1 |  |
| 7.6.1G.2 In-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Blocking ±2.5 dB, 4.2GHz < f ≤ 6.0GHz for V2X sidelink Blocking ±1.4 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.5dB f ≤ 4.2GHz, 4.2GHz < f ≤ 6.0GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1G.3 In-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.6.1G.1 |  | Same as 7.6.1G.1 |  |
| 7.6.1EA In-band blocking for UE category M1 |  | Blocking ±1.4 dB, f ≤ 3.0GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB |  |
| 7.6.1EC In-band blocking for UE category M2 |  | Same as 7.6.1EA |  | Same as 7.6.1EA |  |
| 7.6.2 Out of-band blocking |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB Uplink power measurement ±0.7 dB Wanted signal 3.0GHz < f ≤ 4.2GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.5 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.3 dB Uplink power measurement ±1.0 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2_1 Out of-band blocking with 4 Rx antenna ports |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2A.1 Out-of-band blocking for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.2 for each CC |  | Same as 7.6.2 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.6.2A.2 Out-of-band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.3 Out-of-band blocking for CA (inter band DL CA without UL CA) |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB Uplink power measurement ±0.7 dB Wanted signal 3.0GHz < f ≤ 4.2GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.5 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.3 dB Uplink power measurement ±1.0 dB Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2A.4 Out-of-band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.5 Out-of-band blocking for 3DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.7 Out-of-band blocking for 4DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.8 Out-of-band blocking for 5DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.9 Out-of-band blocking for 6DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.10 Out-of-band blocking for 7DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2B Out-of-band blocking for UL-MIMO |  | Blocking same as 7.6.2 Uplink power measurement same as 7.6.2, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.2D.1 Out-of-band blocking for ProSe Direct Discovery |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB ±0.7 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2D.2 Out-of-band blocking for ProSe Direct Communication |  | Same as 7.6.2D.1 |  | Same as 7.6.2D.1 |  |
| 7.6.2E Out of-band blocking for UE category 0 |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2F Out of-band blocking for category NB1 and NB2 |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2G.1 Out-of-band blocking for V2X Communication/ Non-concurrent with E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2G.2 Out-of-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2G.3 Out-of-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2EA Out-of-band blocking for UE category M1 |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB Uplink power measurement ±0.7 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2EB Out-of-band blocking for UE category 1bis |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2EC Out-of-band blocking for UE category M2 |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.3 Narrow band blocking |  | Blocking ±1. 1 dB, f ≤ 3.0GHz ±1. 5 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Narrow band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.3_1 Narrow band blocking with 4 Rx antenna ports |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3A.1 Narrow band blocking for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.3 for each CC |  | Same as 7.6.3 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.6.3A.2 Narrow band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.3 Narrow band blocking for CA (inter band DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.4 Narrow band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.5 Narrow band blocking for 3DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.7 Narrow band blocking for 4DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.8 Narrow band blocking for 5DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.9 Narrow band blocking for 6DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.10 Narrow band blocking for 7DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3B Narrow band blocking for UL-MIMO |  | Blocking same as 7.6.3 Uplink power measurement same as 7.6.3, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.3D.1 Narrow band blocking for ProSe Direct Discovery |  | Blocking ±1.1 dB, f ≤ 3.0GHz |  | Narrow band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.3D.2 Narrow band blocking for ProSe Direct Communication |  | Same as 7.6.3D.1 |  | Same as 7.6.3D.1 |  |
| 7.6.3E Narrow band blocking for UE category 0 |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3EA Narrow band blocking for UE category M1 |  | Blocking ±1. 1 dB, f ≤ 3.0GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz |  | Narrow band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.3EB Narrow band blocking for UE category 1bis |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3EC Narrow band blocking for UE category M2 |  | Same as 7.6.3EA |  | Same as 7.6.3EA |  |
| 7.7 Spurious response |  | Same as 7.6.2 |  | Same as 7.6.2. |  |
| 7.7_1 Spurious response with 4 Rx antenna ports |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.7A.1 Spurious response for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.2 Spurious response for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.3 Spurious response for CA (inter band DL CA without UL CA) |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.7A.4 Spurious response for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.5 Spurious response for 3DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.7 Spurious response for 4DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.8 Spurious response for 5DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.9 Spurious response for 6DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.10 Spurious response for 7DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7B Spurious response for UL-MIMO |  | Same as 7.7 Uplink power measurement same as 7.6.2, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| <A lot of rows skipped here> |  |  |  |  |  |
| 7.9G.1 Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 30MHz ≤ f ≤ 4.0GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB 19GHz < f ≤ 26 GHz: ± 6.0 dB |  |  |  |
| Note 1: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. |  |  |  |  |  |

<Unchanged Sections Skipped here>

F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Test | Minimum Requirement in TS 36.101 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| 7.3 Reference sensitivity power level | Reference sensitivity power level: For 1.4MHz -102.2dBm -103.2dBm -105.2dBm -106.2dBm For 3MHz -99.2dBm -100.2dBm -102.2dBm For 5MHz -97dBm -98dBm -98.5dBm -99dBm -100dBm -96.5dBm Band 9 with Multi band For 10MHz -94dBm -95dBm -95.5dBm -96dBm -97dBm -93.5dBm Band 9 with Multi band For 15MHz -92.2dBm -93.2dBm -93.7dBm -94.2dBm -95.2dBm -91.7dBm Band 9 with Multi band For 20MHz -91dBm -92dBm -93dBm -94dBm -90.5dBm Band 9 with Multi band T-put limit = 95% of maximum for the Ref Meas channel | All cases: f ≤ 3.0GHz 0.7dB 3.0GHz < f ≤ 4.2GHz 1.0 dB | Formula: Reference sensitivity power level + TT T-put limit unchanged |
| <A lot of rows skipped here> |  |  |  |
| 7.6.1 In-band blocking | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.6.1_1 In-band blocking with 4 Rx antenna ports | Same as 7.6.1 | Same as 7.6.1 | Same as 7.6.1 |
| 7.6.1A.1 In-band blocking for CA (intra band contiguous DL CA and UL CA) | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | Same as 7.6.1 | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |
| 7.6.1A.2 In-band blocking for CA (intra band contiguous DL CA without UL CA) | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1A.3 In-band blocking for CA (inter band DL CA without UL CA) | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1A.4 In-band blocking for CA (intra-band non-contiguous DL CA without UL CA) | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1A.5 In-band blocking for 3DL CA | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1A.7 In-band blocking for 4DL CA | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1A.8 In-band blocking for 5DL CA | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1A.9 In-band blocking for 6DL CA | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1A.10 In-band blocking for 7DL CA | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1B In-band blocking for UL-MIMO | Same as 7.6.1 | Same as 7.6.1 | Same as 7.6.1 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.6.1D.1 In-band blocking for ProSe Direct Discovery | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.1D.2 In-band blocking for ProSe Direct Communication | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.6.1D.1 | Same as 7.6.1D.1 |
| 7.6.1E In-band blocking for UE category 0 | Same as 7.6.1 | Same as 7.6.1 | Same as 7.6.1 |
| 7.6.1F In-band blocking for category NB1 and NB2 | Wanted signal power: (REFSENS + 6dB) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.1EA In-band blocking for UE category M1 | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB | Same as 7.6.1 |
| 7.6.1EB In-band blocking for UE category 1bis | Same as 7.6.1 | Same as 7.6.1 | Same as 7.6.1 |
| 7.6.1EC In-band blocking for UE category M2 | Same as 7.6.1EA | Same as 7.6.1EA | Same as 7.6.1 |
| 7.6.1G.1 In-band blocking for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.1G.2 In-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB 4.2GHz < f ≤ 6GHz 0dB, -4.3dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.1G.3 In-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 7.6.1G.1 | Same as 7.6.1G.1 | Same as 7.6.1G.1 |
| 7.6.2 Out of-band blocking | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.6.2_1 Out of-band blocking with 4 Rx antenna ports | Same as 7.6.2 | Same as 7.6.2 | Same as 7.6.2 |
| 7.6.2A.1 Out-of-band blocking for CA (intra band contiguous DL CA and UL CA) | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | Same as 7.6.2 | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |
| 7.6.2A.2 Out-of-band blocking for CA (intra band contiguous DL CA without UL CA) | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2A.3 Out-of-band blocking for CA (inter band DL CA without UL CA) | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2A.4 Out-of-band blocking for CA (intra-band non-contiguous DL CA without UL CA) | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2A.5 Out-of-band blocking for 3DL CA | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2A.7 Out-of-band blocking for 4DL CA | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2A.8 Out-of-band blocking for 5DL CA | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2A.9 Out-of-band blocking for 6DL CA | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2A.10 Out-of-band blocking for 7DL CA | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2B Out-of-band blocking for UL-MIMO | Same as 7.6.2 | Same as 7.6.2 | Same as 7.6.2 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.6.2D.1 Out-of-band blocking for ProSe Direct Discovery | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.2D.2 Out-of-band blocking for ProSe Direct Communication | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.6.2D.1 | Same as 7.6.1D.1 |
| 7.6.2E Out of-band blocking for UE category 0 | Same as 7.6.2 | Same as 7.6.2 | Same as 7.6.2 |
| 7.6.2F Out of-band blocking for category NB1 and NB2 | Wanted signal power: (REFSENS + 6dB) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.2G.1 Out-of-band blocking for V2X Communication/ Non-concurrent with E-UTRA uplink transmissions | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.2G.2 Out-of-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.2G.3 Out-of-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.2EA Out-of-band blocking for UE category M1 | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB | Same as 7.6.2 |
| 7.6.2EB Out-of-band blocking for UE category 1bis | Same as 7.6.2 | Same as 7.6.2 | Same as 7.6.2 |
| 7.6.2EC Out-of-band blocking for UE category M2 | Same as 7.6.2EA | Same as 7.6.2EA | Same as 7.6.2 |
| 7.6.3 Narrow band blocking | Wanted signal power,: (REFSENS + BW dependent value) Interferer signal power: -55dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.6.3_1 Narrow band blocking with 4 Rx antenna ports | Same as 7.6.3 | Same as 7.6.3 | Same as 7.6.3 |
| 7.6.3A.1 Narrow band blocking for CA (intra band contiguous DL CA and UL CA) | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -55dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | Same as 7.6.3 | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |
| 7.6.3A.2 Narrow band blocking for CA (intra band contiguous DL CA without UL CA) | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3A.3 Narrow band blocking for CA (inter band DL CA without UL CA) | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3A.4 Narrow band blocking for CA (intra-band non-contiguous DL CA without UL CA) | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3A.5 Narrow band blocking for 3DL CA | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3A.7 Narrow band blocking for 4DL CA | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3A.8 Narrow band blocking for 5DL CA | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3A.9 Narrow band blocking for 6DL CA | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3A.10 Narrow band blocking for 7DL CA | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3B Narrow band blocking for UL-MIMO | Same as 7.6.3 | Same as 7.6.3 | Same as 7.6.3 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.6.3D.1 Narrow band blocking for ProSe Direct Discovery | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.3D.2 Narrow band blocking for ProSe Direct Communication | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.6.3D.1 | Same as 7.6.3D.1 |
| 7.6.3E Narrow band blocking for UE category 0 | Same as 7.6.3 | Same as 7.6.3 | Same as 7.6.3 |
| 7.6.3EA Narrow band blocking for UE category M1 | Wanted signal power,: (REFSENS + BW dependent value) Interferer signal power: -55dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB | Same as 7.6.3 |
| 7.6.3EB Narrow band blocking for UE category 1bis | Same as 7.6.3 | Same as 7.6.3 | Same as 7.6.3 |
| 7.6.3EC Narrow band blocking for UE category M2 | Same as 7.6.3EA | Same as 7.6.3EA | Same as 7.6.3 |
| 7.7 Spurious response | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.7_1 Spurious response with 4 Rx antenna ports | Same as 7.7 | Same as 7.7 | Same as 7.7 |
| 7.7A.1 Spurious response for CA (intra band contiguous DL CA and UL CA) | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | Same as 7.7 | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |
| 7.7A.2 Spurious response for CA (intra band contiguous DL CA without UL CA) | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7A.3 Spurious response for CA (inter band DL CA without UL CA) | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7A.4 Spurious response for CA (intra-band non-contiguous DL CA without UL CA) | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7A.5 Spurious response for 3DL CA | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7A.7 Spurious response for 4DL CA | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7A.8 Spurious response for 5DL CA | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7A.9 Spurious response for 6DL CA | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7A.10 Spurious response for 7DL CA | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7B Spurious response for UL-MIMO | Same as 7.7 | Same as 7.7 | Same as 7.7 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.7D.1 Spurious response for ProSe Direct Discovery | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.7D.2 Spurious response for ProSe Direct Communication | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.7D.1 | Same as 7.7D.1 |
| 7.7E Spurious response for UE category 0 | Same as 7.7 | Same as 7.7 | Same as 7.7 |
| 7.7EA Spurious response for UE category M1 | Same as 7.7 | Same as 7.7 | Same as 7.7 |
| 7.7EB Spurious response for UE category 1bis | Same as 7.7 | Same as 7.7 | Same as 7.7 |
| 7.7EC Spurious response for UE category M2 | Same as 7.7 | Same as 7.7 | Same as 7.7 |
| 7.7F Spurious response for category NB1 | Wanted signal power: (REFSENS + 6 dB) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.7G.1 Spurious response for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.7G.2 Spurious response for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.7G.3 Spurious response for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 7.7G.1 | Same as 7.7G.1 | Same as 7.7G.1 |
| <A lot of rows skipped here> |  |  |  |
| 7.9G.1 Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 30MHz f < 1GHz: -57dBm / 100kHz 1GHz f 12.75 GHz: -47dBm / 1MHz 12.75GHz f 26 GHz: -47dBm / 1MHz | 0 dB | Formula: Minimum Requirement + TT |

<End Of Changes>