3GPP TSG-RAN5 Meeting #91-e	R5-212775

# Online, 17th May 2021 – 28th  May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1887 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default contents for RRCReconfiguration-Video |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of default contents for RRCReconfiguration-Video |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added Table 4.8.1-1D for RRCReconfiguration-Video - Added Table 4.8.1-6 for RadioBearerConfig-Video |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure for IMS Video call will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.1-1D (new), 4.8.1-6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.8.1	Radio configurations

<Sections skipped here>

<Start>

#### –	RRCReconfiguration-Video

Table 4.8.1-1D: RRCReconfiguration-Video

| Derivation Path: Table 4.6.1-13 with condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-Video |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-DRB(0.1) | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RadioBearerConfig-Video

Table 4.8.1-6: RadioBearerConfig-Video

| Derivation Path: Table 4.6.3-132 and condition SRB2. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 2 entries |  |  |
| DRB-ToAddMod SEQUENCE { |  |  |  |
| cnAssociation[1] CHOICE { |  |  |  |
| sdap-Config[1] SEQUENCE { | SDAP-Config |  |  |
| defaultDRB[1] | false |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity[1] | j | j is allocated according to internal TTCN mapping |  |
| reestablishPDCP[1] | Not present |  |  |
| recoverPDCP[1] | Not present |  |  |
| pdcp-Config[1] | PDCP-Config with condition UM |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cnAssociation CHOICE[2] { |  |  |  |
| sdap-Config SEQUENCE[2] { | SDAP-Config |  |  |
| defaultDRB[2] | false |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity[2] | k | k is allocated according to internal TTCN mapping |  |
| reestablishPDCP[2] | Not present |  |  |
| recoverPDCP[2] | Not present |  |  |
| pdcp-Config[2] | PDCP-Config with condition UM |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<END>