3GPP TSG-RAN5 Meeting #91-e	R5-212853

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1896 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Note about n28 Test frequency Mid range and CBW 30 MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | High CBW and Mid Test frequency test point is specified in some test cases in the test spec, while there is no carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1, and the current Note does not specify which frequency to select for the testing of Highest CBW=30 MHz instead. To avoid adding a note in all RF test cases which is too easy to forget, NOTE about n28 Test frequency Mid range and CBW 30 MHz need to be updated to specify which frequency to select for the testing of Highest CBW=30 MHz instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NOTE about n28 Test frequency Mid range and CBW 30 MHz was updated to specify which frequency to select for the testing of Highest CBW=30 MHz instead. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec reader will have no idea which frequency to select for the testing of Highest CBW=30 MHz if Mid Test frequency is specified in the test spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.28 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

4.3.1.1.1.28	Reference test frequencies for NR operating band n28

Table 4.3.1.1.1.28-1: Test frequencies for NR operating band n28 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 760.5 | 152100 | 758.25 | 151650 | 0 | 15 | 1902 | 152190 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 780.5 | 156100 | 759.89 | 151978 | 102 |  | 1949 | 156010 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 800.5 | 160100 | 707.53 | 141506 | 504 |  | 2002 | 160130 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 632.53 | 126506 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 745.5 | 149100 | 742.17 | 148434 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 763 | 152600 | 758.32 | 151664 | 0 | 15 | 1903 | 152210 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 780.5 | 156100 | 757.46 | 151492 | 102 |  | 1943 | 155530 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 798 | 159600 | 702.6 | 140520 | 504 |  | 1989 | 159150 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 630.1 | 126020 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 743 | 148600 | 737.24 | 147448 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 765.5 | 153100 | 758.39 | 151678 | 0 | 15 | 1901 | 152170 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 780.5 | 156100 | 755.03 | 151006 | 102 |  | 1937 | 155050 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 795.5 | 159100 | 697.67 | 139534 | 504 |  | 1976 | 158170 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 627.67 | 125534 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 740.5 | 148100 | 732.31 | 146462 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 768 | 153600 | 758.46 | 151692 | 0 | 15 | 1902 | 152190 | 10 | 1 | 1 (2) | 3 |
| (Note 2) |  |  | Mid | 783 | 156600 | 755.1 | 151020 | 102 |  | 1938 | 155070 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 793 | 158600 | 692.74 | 138548 | 504 |  | 1966 | 157250 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 728 | 145600 | 627.74 | 125548 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 738 | 147600 | 727.38 | 145476 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 773 | 154600 | 758.6 | 151720 | 0 | 15 | 1901 | 152170 | 6 | 0 | 1 (2) | 2 |
| (Note 4) |  |  | High | 788 | 157600 | 682.88 | 136576 | 504 |  | 1940 | 155290 | 10 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 718 | 143600 | 703.6 | 140720 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 733 | 146600 | 717.52 | 143504 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use High range and CBW=30 MHz instead. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.28-2: Test frequencies for NR operating band n28 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 763 | 152600 | 758.68 | 151736 | 0 | 15 | 1909 | 152690 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 780.5 | 156100 | 739.46 | 147892 | 102 |  | 1949 | 156010 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 798 | 159600 | 612.24 | 122448 | 504 |  | 1995 | 159630 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 539.74 | 107948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 743 | 148600 | 736.52 | 147304 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 765.5 | 153100 | 758.66 | 151732 | 0 | 15 | 1907 | 152650 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 780.5 | 156100 | 736.94 | 147388 | 102 |  | 1943 | 155530 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 795.5 | 159100 | 607.22 | 121444 | 504 |  | 1982 | 158650 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 537.22 | 107444 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 740.5 | 148100 | 731.5 | 146300 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 768 | 153600 | 758.82 | 151764 | 0 | 15 | 1908 | 152670 | 14 | 0 | 2 (7) | 14 |
| (Note 2) |  |  | Mid | 783 | 156600 | 737.1 | 147420 | 102 |  | 1944 | 155550 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 793 | 158600 | 602.38 | 120476 | 504 |  | 1969 | 157490 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 728 | 145600 | 537.38 | 107476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 738 | 147600 | 726.66 | 145332 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 773 | 154600 | 758.96 | 151792 | 0 | 15 | 1907 | 152650 | 22 | 0 | 1 (6) | 12 |
| (Note 4) |  |  | High | 788 | 157600 | 592.52 | 118504 | 504 |  | 1943 | 155530 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 718 | 143600 | 703.96 | 140792 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 733 | 146600 | 716.8 | 143360 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use High range and CBW=30 MHz instead. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>