3GPP TSG-RAN5 Meeting #91-e	R5-212888

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5022 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction and reallocation of NB-IoT test case for WUS / DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. In the title of TCs wording " Wake Up Signalling" instead of " Wake Up Signal" is used; 2. WUS-Config-NB-r15 is missing numDRX-CyclesRelaxed-r15 IE definition. 3. This test case for NB-IoT WUS / DRX should be in 22.4 instead of 22.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Modify the title of test case and relocated into 22.4.28; 2. Add numDRX-CyclesRelaxed-r15 IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.11, 22.4.28(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1340 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

22.2.11	Void

<Skip Unchanged Sections>

22.4.28	NB-IoT / Wake-up Signal / DRX

22.4.28.1	Test Purpose (TP)

(1)

with { UE in RRC_IDLE state and configured for DRX and WUS }

ensure that {

when { network sends WUS followed by Paging message with a matched identity }

then { UE establishes RRC connection }

}

22.4.28.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 7.4

[TS 36.304, clause 7.4]

Paging with Wake Up Signal is only used in the cell in which the UE most recently entered RRC_IDLE triggered by:

-	reception of RRCEarlyDataComplete; or

-	reception of RRCConnectionRelease not including noLastCellUpdate; or

-	reception of RRCConnectionRelease including noLastCellUpdate and the UE was using WUS in this cell prior to this RRC connection attempt.

If the UE is in RRC_IDLE, the UE is not using GWUS according to clause 7.5 and the UE supports WUS and WUS configuration is provided in system information, the UE shall monitor WUS using the WUS parameters provided in System Information. When DRX is used and the UE detects WUS the UE shall monitor the following PO. When extended DRX is used and the UE detects WUS the UE shall monitor the following numPOs POs or until a paging message including the UE's NAS identity is received, whichever is earlier. If the UE does not detect WUS the UE is not required to monitor the following PO(s). If the UE missed a WUS occasion (e.g. due to cell reselection), it monitors every PO until the start of next WUS or until the PTW ends, whichever is earlier.

-	numPOs = Number of consecutive Paging Occasions (PO) mapped to one WUS provided in system information where (numPOs≥1).

The WUS configuration, provided in system information, includes time-offset between end of WUS and start of the first PO of the numPOs POs UE is required to monitor. The timeoffset in subframes, used to calculate the start of a subframe g0 (see TS 36.213 [6]), is defined as follows:

-	for UE using DRX, it is the signalled timeoffsetDRX;

-	for UE using eDRX, it is the signalled timeoffset-eDRX-Short if timeoffset-eDRX-Long is not broadcasted;

-	for UE using eDRX, it is the value determined according to Table 7.4-1 if timeoffset-eDRX-Long is broadcasted

Table 7.4-1: Determination of GAP between end of WUS and associated PO

|  |  | timeoffset-eDRX-Long |  |
| --- | --- | --- | --- |
|  |  | 1000ms | 2000ms |
| UE Reported wakeUpSignalMinGap-eDRX | 40ms or not reported | timeoffset-eDRX-Short | timeoffset-eDRX-Short |
|  | 240ms | timeoffset-eDRX-Short | timeoffset-eDRX-Short |
|  | 1000ms | timeoffset-eDRX-Long | timeoffset-eDRX-Long |
|  | 2000ms | timeoffset-eDRX-Short | timeoffset-eDRX-Long |

The timeoffset is used to determine the actual subframe g0 as follows (taking into consideration resultant SFN and/or H-SFN wrap-around of this computation):

g0 = PO – timeoffset, where PO is the Paging Occasion subframe as defined in clause 7.1

For UE using eDRX, the same timeoffset applies between the end of WUS and associated first PO of the numPOs POs for all the WUS occurrences for a PTW.

The timeoffset, g0, is used to calculate the start of the WUS as defined in TS 36.213 [6].

22.4.28.3	Test description

22.4.28.3.1	Pre-test conditions

System Simulator:

-	NCell 1 supports WUS.

UE:

-	The UE supports WUS.

Preamble:

-	The UE is in state Registered, Idle Mode (State 3-NB) on Ncell 1 according to [18].

22.4.28.3.2	Test procedure sequence

Table 22.4.28.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits WUS followed by Paging-NB message with a matched identity. | <-- | Paging-NB | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest-NB message? | --> | RRC: RRCConnectionRequest-NB | 1 | P |
| 3-5 | Steps 3 to 5 of the generic test procedure in TS 36.508 subclause 8.1.5A.2 are performed. | - | - | - | - |
| 6 | The SS transmits an RRCConnectionRelease-NB message. | <-- | RRCConnectionRelease-NB | - | - |

22.4.28.3.3	Specific message contents

Table 22.4.28.3.3-1: SystemInformationBlockType2-NB (preamble and all steps, table 22.4.28.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| RadioResourceConfigCommon-r13 ::= SEQUENCE { |  |  |  |
| wus-Config-r15 ::= SEQUENCE { |  |  |  |
| maxDurationFactor-r15 | one16th |  |  |
| numPOs-r15 | n1 |  |  |
| numDRX-CyclesRelaxed-r15 | n1 |  |  |
| timeOffsetDRX-r15 | ms80 |  |  |
| timeOffset-eDRX-Short-r15 | ms160 |  |  |
| timeOffset-eDRX-Long-r15 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |