3GPP TSG-RAN5 Meeting #91-e	R5-212904

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1189 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RMSI CORESET reference configuration in Annex A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For 30kHz SCS + TDD test configuration. only RMC CORESET reference configurations with 24RB bandwidth are defined (CCR.2.1 TDD and CCR.2.2 TDD). However, In several TCs (e.g. TC 6.5.2.1/6.5.6.1/6.5.6.2) DLBWP.0.2 is used hence the active BWP is only 24RB. On several test frequencies defined in 38.508-1 offset between active BWP and point A isn't multiple of 6RBs. Hence it is impossible to fully contain a 24 RB RMC CORESET in active BWP in certain test cases. Then UE will fail the test case. frequencyDomainResources Frequency domain resources for the CORESET. Each bit corresponds a group of 6 RBs, with grouping starting from the first RB group in the BWP. When at least one search space is configured with freqMonitorLocation-r16, only the first NRBG,set0size bits are valid (see TS 38.213 [13], clause 10.1). The first (left-most / most significant) bit corresponds to the first RB group in the BWP, and so on. A bit that is set to 1 indicates that this RB group belongs to the frequency domain resource of this CORESET. Bits corresponding to a group of RBs not fully contained in the bandwidth part within which the CORESET is configured are set to zero (see TS 38.211 [16], clause 7.3.2.2).. Actually similar issue also happens under 15kHz SCS + FDD test configuration. After discussion RAN4 has agreed to add a 18RB + 4AL RMC CORESET reference configuration. It is also neccessary to add a 18RB+4AL RMC CORESET for 30kHz SCS + TDD too. |
| Summary of change: |  | - A 18RB +4AL RMC CORESET reference configuration (CCR.2.3 TDD) is added for 30kHz SCS+TDD configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

Table A.1.3.2-2: Control Channel RMC for SCS = 30 kHz for TDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.2.1 TDD | CCR.2.2 TDD | CCR.2.3 TDD |  |
| Channel bandwidth | MHz | 40 | 40 | 40 |  |
| Subcarrier spacing for RMSI CORESET | kHz | 30 | 30 | 30 |  |
| Allocated resource blocks for CORESET3 |  | 24 | 24 | 18 |  |
| Number of transmitter antenna |  | 1 | 1 | 1 |  |
| Duration of CORESET | symbols | 2 | 2 | 2 |  |
| REG bundle size |  | 6 | 6 | 6 |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved |  |
| Interleave n_shift |  | 0 | 0 | 0 |  |
| Interleave size |  | 2 | 2 | 2 |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A |  |
| Aggregation level | CCE | 4 | 8 | 4 |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |

<End of modified section>