3GPP TSG-RAN5 Meeting #91-e	R5-212917

# Electronic, 17th - 28th May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0529 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction of AMPR and Additional spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed e?planations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Format in 6.2.3 is not correct - Some references are not correct in 6.2.3 and 6.5.3.3 - Minimum requirement of 6.5.3.3.3 is not aligned with core requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clean up format of sub-titles in 6.2.3 - Correcting references in 6.2.3 and 6.5.3.3 - Align the minimum requirements in 6.5.3.3.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2.3	UE maximum output power with additional requirements

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- Multiband relaxation is not considered in test requirements

- Measurment with modifiedMPRbehavior is FFS.

- Test description and Test requirements subsections for NS 202 and NS 203 are incomplete

6.2.3.1	Test purpose

Additional spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power.

6.2.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.2.3.3	Minimum conformance requirements

6.2.3.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE field freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements

To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in subclause 6.2.1.1.3. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of and the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in subclause 6.2.2.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_200 |  |  |  |  | N/A |
| NS_201 | 6.5.3.3.3 | n258 |  |  | 6.2.3.2 |
| NS_202 | 6.5.3.3.3 | n257, n258 |  |  | 6.2.3.3.3 |
| NS_203 | 6.5.3.3.3 | n258 |  |  | 6.2.3.3.4 |
| NOTE: NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |

Table 6.2.3.3.1-2: Mapping of Network Signalling label

| NR Band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | NS_200 | NS_202 | NS_202 | NS_203 |  |  |  |  |
| n258 | NS_200 | NS_2012 |  |  |  |  |  |  |
| n260 | NS_200 |  |  |  |  |  |  |  |
| n261 | NS_200 |  |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in sub-clause 6.3.2 of TS 38.331 [19]. NOTE 2: NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.3.1.

6.2.3.3.2	Void

6.2.3.3.2.1	Void

For power class 1, A-MPR for NS_201 shall be 9 dB.

Table 6.2.3.3.2.1-1: Void

6.2.3.3.2.2	Void

6.2.3.3.2.3	Void

6.2.3.3.3	A-MPR for NS_202

6.2.3.3.3.1	A-MPR for NS_202 for power class 1

For power class 1, A-MPR for NS_202 shall be 11.0 dB.

6.2.3.3.3.2	A-MPR for NS_202 for power class 2

For power class 2, A-MPR for NS_202 specified in clause 6.2.3.3.3.3 applies.

6.2.3.3.3.3	A-MPR for NS_202 for power class 3

For power class 3, A-MPR for NS_202 shall be 1.0 dB.

6.2.3.3.3.4	A-MPR for NS_202 for power class 4

For power class 4, A-MPR for NS_202 specified in clause 6.2.3.3.3.3 applies.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.3.3.

6.2.3.3.4	A-MPR for NS_203

6.2.3.3.4.1	A-MPR for NS_203 for power class 1

For power class 1, A-MPR for NS_203 shall be 3.0 dB if Offset frequency < BWchannel, 0.0 dB otherwise. 
The Offset frequency is defined as the frequency from 24.25 GHz to the lower edge of the channel bandwidth.

6.2.3.3.4.2	A-MPR for NS_203 for power class 2

For power class 2, A-MPR for NS_203 specified in clause 6.2.3.3.4.3 applies.

6.2.3.3.4.3	A-MPR for NS_203 for power class 3

For power class 3, A-MPR for NS_203 shall be 0 dB.

6.2.3.3.4.4	A-MPR for NS_203 for power class 4

For power class 4, A-MPR for NS_203 specified in clause 6.2.3.3.4.3 applies.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.3.4.

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the Subscriber Station (SS) to take with the UE to reach the correct measurement state.

Table 6.2.3.4.1-1: Void

Table 6.2.3.4.1-2: Test configuration table for NS_202

[TBD]

Table 6.2.3.4.1-3: Test configuration table for NS_203

[TBD]

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.1.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The DL and UL Reference Measurement channels are set according to Table 6.2.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.3.4.3

6.2.3.4.2	Test procedure

<Unchanged Text Skipped>

6.5.3.3	Additional Spurious emission

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- NS_202 & NS_203 A-MPR test case 6.2.3 is incomplete

6.5.3.3.1	Test purpose

Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5.3.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.3.3.3	Minimum conformance requirements

The additional spurious emission limits in Table 6.5.3.3.3-1 through Table 6.5.3.3.3-3 apply for all transmitter band configurations (RB) and channel bandwidths. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5.3.3.3-1: Void

When "NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3-2.

Table 6.5.3.3.3-2: Additional spurious emissions (NS_202) test limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |
| 23.6 GHz f 24.0 GHz | +1 dBm | 200 MHz |
| NOTE 1: This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |

When "NS_203" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3-3. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3-3: Additional spurious emissions (NS_203) test limits

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 23.6 f 24.0 | +1 | 200 MHz |

The normative reference for this requirement is TS 38.101-2 subclause 6.5.3.2.

6.5.3.3.4	Test description

6.5.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the Subscriber Station (SS) to take with the UE to reach the correct measurement state.

Table 6.5.3.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3 and PC4 Inner_Full_Region1 for PC1 |
| 2 |  | DFT-s-OFDM QPSK | Inner_1RB_Left for PC2, PC3 and PC4 Inner_Partial for PC1 (NOTE 3) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3 and PC4 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3 and PC4 or Inner_Partial_Right_Region1 for PC1. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5.3.3.4.1-1

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.3.3.4.3.

6.5.3.3.4.2	Test procedure

1.	Select any of the three Alignment Options (1, 2, or 3) from Tables J.2-1 through J.2-3 [3] to mount the DUT inside the QZ.

2. 	If the re-positioning concept is applied, position the device in DUT Orientation 1 if the maximum beam peak direction is within zenith angular range 0o≤≤90o for the alignment option selected in step 1; position the device in DUT Orientation 2 (either Options 1 or 2) if the maximum beam peak direction is within zenith angular range 90o<≤180o for DUT Orientation 1 for the alignment option selected in step 1. If the re-positioning concept is not applied, position the device in DUT Orientation 1.

3.	SS sends uplink scheduling information for each UL HARQ process via PDSCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.3.3.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

4.	Set the UE in the Inband Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 3) for the UE Tx beam selection to complete.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200msec for the UE to reach PUMAX. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 3) for the UE Tx beam selection to complete.6.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

7.	Measure the spurious emissions as per steps outlined below with an exception to the procedure in Annex K if the re-positioning concept is applied (NOTE 4). Step (a) is optional and applicable only if SNR (test requirement level in Table 6.5.3.3.5-2 through Table 6.5.3.3.5-3 minus offset value minus noise floor of the test system) ≥ 0 dB is guaranteed.

(a)	Perform coarse TRP measurements to identify spurious emission frequencies and corresponding power level according to the procedures in Annex K, using coarse TRP measurement grid selection criteria as per Table M.4.5-3 in Annex M. The measurement is completed in both polarizations  and  over frequency range and measurement bandwidth according to Table 6.5.3.3.5-2 through Table 6.5.3.3.5-3. Optionally, a larger and non-constant measurement bandwidth than that of Table 6.5.3.3.5-2 through Table 6.5.3.3.5-3 may be applied. The measurement period shall capture the active time slots. For each spurious emission frequency with coarse TRP identified to be less than an offset dB from the TRP limit according to Table 6.5.3.3.5-2 through Table 6.5.3.3.5-3, continue with fine TRP procedures according to step (b).

The offset value shall be the TRP measurement uncertainty at 95% confidence level including the effect of coarse grid measurement uncertainty element. Different coarse TRP grids and corresponding offset values may be used for different frequencies. The coarse TRP grid and offset values used shall be recorded in the test report.

(b)	Measure fine TRP measurements according to procedures in Annex K, using fine TRP measurement grid selection criteria as per Table M.4.5-3 in Annex M, for each of the spurious emission frequency identified in step (a). Apply a measurement bandwidth according to Table 6.5.3.3.5-2 through Table 6.5.3.3.5-3.

8.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The frequency range defined in Table 6.5.3.3.5-2 through Table 6.5.3.3.5-3 may be split into ranges. For each range a different test system, e.g. antenna and/or chamber, may be used. To pass the test case all verdicts of the frequency ranges must pass.

NOTE 2: 	The coarse TRP measurement grid and corresponding offset dB value referred in step 5(a) above, for some valid grids can be found in TR 38.903 section B.18.

NOTE 3:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

NOTE 4:	If the (in-band) beam peak is within 0o≤≤90o: perform first hemispherical TRP scan (0o≤≤90o) in DUT Orientation 1 and second hemispherical TRP scan (90o>≥0o) in DUT Orientation 2. If the (in-band) beam peak is within 90o<≤180o: perform first hemispherical TRP scan (0o≤≤90o) in DUT Orientation 2 and second hemispherical TRP scan (90o>≥0o) in DUT Orientation 1. The DUT with UBF activated needs to be re-positioned during the test.

<Unchanged Text Skipped>

6.5.3.3.5	Test requirement

This clause specifies the requirements for the specified NR band for Transmitter Additional Spurious emissions requirement with frequency range as indicated in Table 6.5.3.3.5-1, Table 6.5.3.3.5-2 and Table 6.5.3.3.5-3.

The maximum EIRP or TRP power of spurious emission for Transmitter Additional Spurious emissions, measured using RMS detector, shall not exceed the described value in Table 6.5.3.3.5-1, Table 6.5.3.3.5-2 and Table 6.5.3.3.5-3.

The Transmitter Additional Spurious emissions limits in Table 6.5.3.3.5-1, Table 6.5.3.3.5-2 and Table 6.5.3.3.5-3 apply for all transmitter band configurations (NRB) and channel bandwidths.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5.3.3.5-1: Void

Table 6.5.3.3.5-2: Additional spurious emissions (NS_202) test requirements

| Frequency Range | Maximum Level (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 7.25 GHz ≤ f ≤ 12.75 GHz | -10 | 100 MHz |  |
| 12.75 GHz ≤ f ≤ 23.45 GHz | -10 + 13 | 100 MHz | NOTE 1 |
| 23.45 GHz ≤ f ≤ 40.8 GHz | -10 + 13 | 100 MHz | NOTE 1 |
| 40.8 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 + 13 | 100 MHz | NOTE 1 |
| 23.6 GHz ≤ f ≤ 24.0 GHz | +1 +0.3 | 200 MHz | NOTE 2 |
| NOTE 1: 13 dB relaxation due to testability limit NOTE 2: 0.3 dB relaxation due to testability limit |  |  |  |

Table 6.5.3.3.5-3: Additional spurious emissions (NS_203) test requirements

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 23.6 ≤ f ≤ 24.0 | +1 + 0.3 | 200 MHz | NOTE 1 |
| NOTE 1: 0.3 dB relaxation due to testability limit |  |  |  |

<End of Changes >