3GPP TSG-RAN5 Meeting #91-e	R5-212922

# Electronic, 17th – 28th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1902 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of calculation method of NR sidelink test frequencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NR sidelink operation is specified for bands n38 and n47. Test frequencies need to be specified for these bands. As the S-SSB location on SL carrier is not the same as SSB on normal DL carrier, current calculation of test frequencies can’t be reused. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding calculation of test frequencies for V2X bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The method of calculation of test frequencies is not captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

Annex C (informative): Calculation of test frequencies

C.0	General

Test frequencies are defined in clause 4.3.1 with extensions for signalling test cases in clause 6.2.3. This annex gives a guideline to determine these test frequencies and the associated signalling parameters for a given NR band, NR CA or NR DC band combination.

Clause C.1 describes definitions and parameters used by the procedures to determine test frequencies, SS/PBCH Block (SSB) and CORESET#0 configuration parameters.

Clause C.2 describes how to calculate test frequencies for symmetric NR bands, asymmetric NR bands, NR CA and NR DC configurations.

Clause C.3.2 describes how to determine the SSB, CORESET#0 and signalling parameters for a PCell.

Clause C.3.3 describes how to determine the SSB and signalling parameters for a Cell without CORESET#0.

Clause C.5 describes how to calculate test frequencies for NR V2X bands.

C.1	Definitions and Parameters

Figure C.1-1 shows SSB and CORESET#0 and related parameters.

<Unchanged Text Skipped>

C.5	Determination of test frequencies and S-SSB for V2X bands

C.5.1	General

Figure C.5.1-1 shows carrier and S-SSB on V2X bands and related parameters.

Figure C.5.1-1: location of S-SSB within a channel

The parameters referenced in Figure C.5.1-1 are defined in Table C.5.1-1.

Table C.5.1-1: Definition of parameters for V2X bands

| Parameter | Description |
| --- | --- |
| FSL_PointA | Reference Point A frequency. |
| FSL_carrier | FSL_carrier is the centre frequency of a carrier corresponding to its NR-ARFCN value. |
| FSL_carrierLow | FSL_carrierLow is the centre frequency of lowest subcarrier of the carrier. FSL_carrierLow = FSL_carrier - 12 * SCSSL_Carrier * (NRB / 2) with NRB according to section 5.3E.1 of TS 38.101-1 for the channel bandwidth of the carrier. |
| ΔFSL_carrierBandwidth | ΔFSL_carrierBandwidth is the carrier’s channel bandwidth as provided in carrierBandwidth to the UE (sl-SCS-SpecificCarrierList-r16). |
| ΔFSL_OffsetToCarrier | ΔFSL_OffsetToCarrier is the frequency offset between Point A and the lower edge of the carrier. ΔFSL_OffsetToCarrier = offsetToCarrier * CRBsize, where CRBsize according to the subcarrier spacing of the carrier. offsetToCarrier is signalled to the UE (sl-SCS-SpecificCarrierList-r16). |
| FSL_SSref | Centre frequency of subcarrier with index 66 in the S-SS/PSBCH block, corresponding to NR-ARFCN value signalled to the UE by sl-AbsoluteFrequencySSB-r16. |

Additional parameters used in this annex are defined in Table C.5.1-2.

Table C.5.1-2: Definition of additional parameters used in section C.5.2

| SCSSL_Carrier | subcarrier spacing for the carrier (sl-SCS-SpecificCarrierList-r16): FR1: 15kHz, 30kHz or 60kHz according to TS 38.101-1 [7] Table 5.3.5-1 |
| --- | --- |
| CRBsize | Common Resource Block size = 12 * SCSSL_Carrier. |
| FSL_Low | Lowest frequency of the frequency range of the V2X band as defined in clause 5.2E.1 of TS 38.101-1 [7]. |
| FSL_High | Highest frequency of the frequency range of the V2X band as defined in clause 5.2E.1 of TS 38.101-1 [7]. |
| ΔFSL_Raster | Frequency raster of the band as defined in clause 5.4E.2.3 of TS 38.101-1 [7]. |
| BWSL | Bandwidth of V2X operation frequency range of the band. |
| CBWSL | UE V2X operation channel bandwidth (MHz) of the carrier according to section 5.3E.1 of TS 38.101-1 [7]. |
| BWSL_SSB | Bandwidth of the SSB. BWSL_SSB = 11 * SCSSL_Carrier * 20 |

C.5.2	Determination of test frequencies and S-SSB for V2X bands

The carrier test frequencies are determined considering the channel raster according to clause 5.4.2.3 in TS 38.101-1 [7] for FR1.

The complete S-SSB shall be within a bandwidth of the SL BWP. The subcarrier with index 0 in the S-SSB shall be aligned with a subcarrier with index 0 in an RB of the SL BWP.

1.	Determine sidelink carrier centre frequencies and the frequency of the carrier’s lowest subcarrier:

| FSL_LowRange = Ceil((FSL_Low + CBWSL/2) / ΔFSL_Raster) * ΔFSL_Raster | C.5.2-Eq1 |
| --- | --- |
| FSL_MidRange = Round((FSL_Low + BWSL/2) / ΔFSL_Raster_) * ΔFSL_Raster | C.5.2-Eq2 |
| FSL_HighRange = Floor((FSL_High – CBWSL/2) / ΔFSL_Raster) * ΔFSL_Raster | C.5.2-Eq3 |
| FSL_carrierLow = see formula for FSL_carrierLow in Table C.1-1 |  |

2.	Determine OffsetToCarrier

Select offsetToCarrier value for the carrier in accordance to Table C.5.2-1.

Table C.5.2-1: Sidelink offsetToCarrier default values for different frequency ranges

| Frequency range | Sidelink offsetToCarrier |
| --- | --- |
| Low range | 0 |
| Mid range | 504 |
| High range | 6 |
| Mid-Low range | 36 |
| Mid-High range | 114 |
| Note: Different values of offsetToCarrier have been selected for Low, Mid-Low, Mid, Mid-High and High ranges to achieve enhanced test coverage of the offsetToCarrier range of values. |  |

3.	Determine FSL_PointA:

FSL_PointA = FSL_carrierLow - offsetToCarrier * CRBsize

4.	Determine the centre frequencies of S-SSB for the lowest, mid and highest possible location of FSL_SSref:

FSL_SSref_Low = FcarrierLow + BWSL_SSB / 2

FSL_SSref_Mid = FcarrierLow + (Floor(NRB/2) – 5) * CRBsize + BWSSB / 2

FSL_SSref_High = FcarrierLow + (NRB – 11) * CRBsize + BWSSB / 2

5.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| sl-AbsoluteFrequencySSB-r16 | Set to FSL_SSref expressed in ARFCN as defined in TS 38.101-1 [15] clause 5.4E.2. |
| sl-AbsoluteFrequencyPointA-r16 | Set to FSL_PointA expressed in ARFCN as defined in TS 38.101-1 [15] clause 5.4E.2. |
| offsetToCarrier | Set to value calculated in step 2. The IE field offsetToCarrier is signalled in IE sl-SCS-SpecificCarrierList-r16. |

<End of Changes >