3GPP TSG-RAN5 Meeting #91-e	R5-212946

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1905 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating IEs for URLLC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC -UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | IEs are newly introduced by RAN2 to support URLLC. Those IEs neede to be introduced into 38.508-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The structure of new IEs are added along with other R16 IEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | URLLC test cases can’t be configured appropriately |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

–	ControlResourceSet

Table 4.6.3-28: ControlResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP |  |
| duration | 2 | SearchSpace duration of 2 symbols |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| nonInterleaved | null |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| tci-StatesPDCCH-ToReleaseList | Not present |  |  |
| tci-PresentInDCI | Not present |  |  |
| pdcch-DMRS-ScramblingID | Not present |  |  |
| rb-Offset-r16 | Not present |  |  |
| tci-PresentDCI-1-2-r16 | Not present |  |  |
| coresetPoolIndex-r16 | Not present |  |  |
| controlResourceSetId-v1610 | Not present |  |  |
| } |  |  |  |

<Unchanged Text Skipped>

–	CSI-MeasConfig

Table 4.6.3-38: CSI-MeasConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 1 entry |  |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource | entry 1 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceToReleaseList | Not present |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet | entry 1 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToReleaseList | Not present |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { | 1 entry |  |  |
| CSI-IM-Resource[1] | CSI-IM-Resource | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceToReleaseList | Not present |  |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet { | 1 entry |  |  |
| CSI-IM-ResourceSet[1] | CSI-IM-ResourceSet | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceSetToReleaseList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet { | 1 entry |  |  |
| CSI-SSB-ResourceSet[1] | CSI-SSB-ResourceSet | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetToReleaseList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 1 entry |  |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig | entry 1 |  |
| } |  |  |  |
| csi-ResourceConfigToReleaseList | Not present |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | 1 entry |  |  |
| CSI-ReportConfig[1] | CSI-ReportConfig | entry 1 |  |
| } |  |  |  |
| csi-ReportConfigToReleaseList | Not present |  |  |
| reportTriggerSize | 0 |  |  |
| aperiodicTriggerStateList CHOICE { |  |  |  |
| setup | CSI-AperiodicTriggerStateList |  |  |
| } |  |  |  |
| semiPersistentOnPUSCH-TriggerStateList | Not present |  |  |
| reportTriggerSizeDCI-0-2-r16 | Not present |  |  |
| } |  |  |  |

<Unchanged Text Skipped>

–	CSI-ReportConfig

Table 4.6.3-39: CSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| carrier | ServCellIndex |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId |  |  |
| nzp-CSI-RS-ResourcesForInterference | CSI-ResourceConfigId |  |  |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER { | 1 entry |  |  |
| INTEGER[1] | 14 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | NULL, |  | FR1 |
| cri-RI-LI-PMI-CQI | NULL |  | FR2 |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| cqi-FormatIndicator | widebandCQI |  |  |
| pmi-FormatIndicator | widebandPMI |  |  |
| csi-ReportingBand | Not present |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | CodebookConfig |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | n1 |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table2 |  | FR1 |
|  | table1 |  | FR2 |
| subbandSize | value2 |  |  |
| non-PMI-PortIndication | Not present |  |  |
| semiPersistentOnPUSCH-v1530 | Not present |  |  |
| semiPersistentOnPUSCH-v1610 | Not present |  |  |
| aperiodic-v1610 | Not present |  |  |
| reportQuantity-r16 | Not present |  |  |
| codebookConfig-r16 | Not present |  |  |
| } |  |  |  |

<Unchanged Text Skipped>

–	PDCCH-Config

Table 4.6.3-95: PDCCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList | Not present |  | EN-DC |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet | entry 1 |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace with condition USS | entry 1 |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| controlResourceSetToAddModListSizeExt-v1610 | Not present |  |  |
| controlResourceSetToReleaseListSizeExt-r16 | Not present |  |  |
| searchSpacesToAddModListExt-r16 | Not present |  |  |
| uplinkCancellation-r16 | Not present |  |  |
| monitoringCapabilityConfig-r16 | Not present |  |  |
| searchSpaceSwitchConfig-r16 | Not present |  |  |
| } |  |  |  |

<Unchanged Text Skipped>

–	PDCCH-ConfigCommon

Table 4.6.3-96: PDCCH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| controlResourceSetZero | ControlResourceSetZero | Initial BWP |  |
|  | Not present |  | SCell_Add InitialBWP_SIB |
| commonControlResourceSet | ControlResourceSet |  | EN-DC |
|  | Not present |  |  |
| searchSpaceZero | SearchSpaceZero | Initial BWP |  |
|  | Not present |  | SCell_Add InitialBWP_SIB |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| SearchSpace[2] | SearchSpace with condition SISS | entry 2 |  |
| } |  |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry |  | EN-DC |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| } |  |  |  |
| commonSearchSpaceList | Not present |  | SCell_Add |
| searchSpaceSIB1 | 0 |  |  |
|  | Not present |  | EN-DC, SCell_Add |
| searchSpaceOtherSystemInformation | SearchSpaceId with condition SISS |  |  |
|  | Not present |  | EN-DC, SCell_Add |
| pagingSearchSpace | 0 |  |  |
|  | Not present |  | EN-DC, SCell_Add |
| ra-SearchSpace | SearchSpaceId with condition CSS |  |  |
|  | Not present |  | SCell_Add |
| firstPDCCH-MonitoringOccasionOfPO | Not present |  |  |
| commonSearchSpaceListExt-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| SCell_Add | Add SCell |
| InitialBWP_SIB | Configured via DownlinkConfigCommonSIB |

<Unchanged Text Skipped>

–	PDSCH-Config

Table 4.6.3-100: PDSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPDSCH | 0 |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-DownlinkConfig |  |  |
| } |  |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeB | Not present |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 1 entry |  |  |
| TCI-State[1] | TCI-State | entry 1 |  |
| } |  |  |  |
| tci-StatesToReleaseList | Not present |  |  |
| vrb-ToPRB-Interleaver | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
|  | resourceAllocationType0 |  | Used_for_Type0 |
| pdsch-TimeDomainAllocationList | Not present |  |  |
| pdsch-AggregationFactor | Not present |  |  |
| rateMatchPatternToAddModList | Not present |  |  |
| rateMatchPatternToReleaseList | Not present |  |  |
| rateMatchPatternGroup1 | Not present |  |  |
| rateMatchPatternGroup2 | Not present |  |  |
| rbg-Size | config1 |  |  |
| mcs-Table | Not present | qam64 per default |  |
| maxNrofCodeWordsScheduledByDCI | Not present |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | wideband |  |  |
| } |  |  |  |
| } |  |  |  |
| zp-CSI-RS-ResourceToAddModList | Not present |  |  |
| zp-CSI-RS-ResourceToReleaseList | Not present |  |  |
| aperiodic-ZP-CSI-RS-ResourceSetsToAddModList | Not present |  |  |
| aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList | Not present |  |  |
| sp-ZP-CSI-RS-ResourceSetsToAddModList | Not present |  |  |
| sp-ZP-CSI-RS-ResourceSetsToReleaseList | Not present |  |  |
| p-ZP-CSI-RS-ResourceSet | Not present |  |  |
| maxMIMO-Layers-r16 | Not present |  |  |
| minimumSchedulingOffsetK0-r16 | Not present |  |  |
| antennaPortsFieldPresenceDCI-1-2-r16 | Not present |  |  |
| aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2-r16 | Not present |  |  |
| aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2-r16 | Not present |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16 | Not present |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16 | Not present |  |  |
| dmrs-SequenceInitializationDCI-1-2-r16 | Not present |  |  |
| harq-ProcessNumberSizeDCI-1-2-r16 | Not present |  |  |
| mcs-TableDCI-1-2-r16 | Not present |  |  |
| numberOfBitsForRV-DCI-1-2-r16 | Not present |  |  |
| pdsch-TimeDomainAllocationListDCI-1-2-r16 | Not present |  |  |
| prb-BundlingTypeDCI-1-2-r16 | Not present |  |  |
| priorityIndicatorDCI-1-2-r16 | Not present |  |  |
| rateMatchPatternGroup1DCI-1-2-r16 | Not present |  |  |
| rateMatchPatternGroup2DCI-1-2-r16 | Not present |  |  |
| resourceAllocationType1GranularityDCI-1-2-r16 | Not present |  |  |
| vrb-ToPRB-InterleaverDCI-1-2-r16 | Not present |  |  |
| referenceOfSLIVDCI-1-2-r16 | Not present |  |  |
| resourceAllocationDCI-1-2-r16 | Not present |  |  |
| priorityIndicatorDCI-1-1-r16 | Not present |  |  |
| dataScramblingIdentityPDSCH2-r16 | Not present |  |  |
| pdsch-TimeDomainAllocationList-r16 | Not present |  |  |
| repetitionSchemeConfig-r16 | Not present |  |  |
| repetitionSchemeConfig-v1630 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Used_for_Type0 | Used for RF performance test cases |

<Unchanged Text Skipped>

–	PDSCH-ServingCellConfig

Table 4.6.3-102: PDSCH-ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| codeBlockGroupTransmission | Not present |  |  |
| xOverhead | Not present |  |  |
| nrofHARQ-ProcessesForPDSCH | n16 |  |  |
|  | Not present | Default value: 8 HARQ processes | SIG |
| pucch-Cell | Not present |  |  |
| maxMIMO-Layers | Not present |  |  |
| processingType2Enabled | Not present |  |  |
| pdsch-CodeBlockGroupTransmissionList-r16 | Not present |  |  |
| } |  |  |  |

<Unchanged Text Skipped>

–	PUCCH-Config

Table 4.6.3-112: PUCCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config ::= SEQUENCE { |  |  |  |
| resourceSetToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet { | 2 entries |  |  |
| PUCCH-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| pucch-ResourceSetId | 0 |  |  |
| resourceList SEQUENCE (SIZE (0..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-ResourceId { | 8 entries |  |  |
| PUCCH-ResourceId[1] | 0 | entry 1 |  |
| PUCCH-ResourceId[2] | 1 | entry 2 |  |
| PUCCH-ResourceId[3] | 2 | entry 3 |  |
| PUCCH-ResourceId[4] | 3 | entry 4 |  |
| PUCCH-ResourceId[5] | 4 | entry 5 |  |
| PUCCH-ResourceId[6] | 5 | entry 6 |  |
| PUCCH-ResourceId[7] | 6 | entry 7 |  |
| PUCCH-ResourceId[8] | 7 | entry 8 |  |
| } |  |  |  |
| maxPayloadSize | Not present |  |  |
| } |  |  |  |
| PUCCH-ResourceSet[2] SEQUENCE { |  | entry 2 |  |
| pucch-ResourceSetId | 1 |  |  |
| resourceList SEQUENCE (SIZE (8..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-ResourceId { | 8 entries |  |  |
| PUCCH-ResourceId[1] | 8 | entry 1 |  |
| PUCCH-ResourceId[2] | 9 | entry 2 |  |
| PUCCH-ResourceId[3] | 10 | entry 3 |  |
| PUCCH-ResourceId[4] | 11 | entry 4 |  |
| PUCCH-ResourceId[5] | 12 | entry 5 |  |
| PUCCH-ResourceId[6] | 13 | entry 6 |  |
| PUCCH-ResourceId[7] | 14 | entry 7 |  |
| PUCCH-ResourceId[8] | 15 | entry 8 |  |
| } |  |  |  |
| maxPayloadSize | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceSetToReleaseList | Not present |  |  |
| resourceToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource { | 16 entries |  |  |
| PUCCH-Resource[1] SEQUENCE { |  | entry 1 |  |
| pucch-ResourceId | 0 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[2] SEQUENCE { |  | entry 2 |  |
| pucch-ResourceId | 1 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[3] SEQUENCE { |  | entry 3 |  |
| pucch-ResourceId | 2 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[4] SEQUENCE { |  | entry 4 |  |
| pucch-ResourceId | 3 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[5] SEQUENCE { |  | entry 5 |  |
| pucch-ResourceId | 4 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[6] SEQUENCE { |  | entry 6 |  |
| pucch-ResourceId | 5 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[7] SEQUENCE { |  | entry 7 |  |
| pucch-ResourceId | 6 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 12 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[8] SEQUENCE { |  | entry 8 |  |
| pucch-ResourceId | 7 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format1 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 14 |  |  |
| startingSymbolIndex | 0 |  |  |
| timeDomainOCC | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[9] SEQUENCE { |  | entry 9 |  |
| pucch-ResourceId | 8 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBs | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[10] SEQUENCE { |  | entry 10 |  |
| pucch-ResourceId | 9 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBs | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[11] SEQUENCE { |  | entry 11 |  |
| pucch-ResourceId | 10 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBs | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[12] SEQUENCE { |  | entry 12 |  |
| pucch-ResourceId | 11 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBs | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[13] SEQUENCE { |  | entry 13 |  |
| pucch-ResourceId | 12 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRB | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[14] SEQUENCE { |  | entry 14 |  |
| pucch-ResourceId | 13 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBsinitial | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[15] SEQUENCE { |  | entry 15 |  |
| pucch-ResourceId | 14 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRB | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 12 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[16] SEQUENCE { |  | entry 16 |  |
| pucch-ResourceId | 15 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format3 SEQUENCE { |  |  |  |
| nrofPRBs | 1 |  |  |
| nrofSymbols | 14 |  |  |
| startingSymbolIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceToReleaseList | Not present |  |  |
| format1CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| interslotFrequencyHopping | enabled |  |  |
| additionalDMRS | Not Present |  |  |
| maxCodeRate | Not Present |  |  |
| nrofSlots | Not present |  |  |
| pi2BPSK | Not present |  |  |
| simultaneousHARQ-ACK-CSI | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| format2 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| interslotFrequencyHopping | Not Present |  |  |
| additionalDMRS | Not Present |  |  |
| maxCodeRate | zeroDot25 |  |  |
| nrofSlots | Not present |  |  |
| pi2BPSK | Not present |  |  |
| simultaneousHARQ-ACK-CSI | True |  |  |
| } |  |  |  |
| } |  |  |  |
| format3 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| interslotFrequencyHopping | enabled |  |  |
| additionalDMRS | True |  |  |
| maxCodeRate | zeroDot25 |  |  |
| nrofSlots | Not present |  |  |
| pi2BPSK | Not present |  |  |
| simultaneousHARQ-ACK-CSI | true |  |  |
| } |  |  |  |
| } |  |  |  |
| format4 | Not present |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] | SchedulingRequestResourceConfig | entry 1 |  |
| } |  |  |  |
| schedulingRequestResourceToReleaseList | Not present |  |  |
| multi-CSI-PUCCH-ResourceList | Not present |  |  |
| dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER { | 8 entries |  |  |
| INTEGER[1] | 2 | entry 1 |  |
| INTEGER[2] | 3 | entry 2 |  |
| INTEGER[3] | 4 | entry 3 |  |
| INTEGER[4] | 5 | entry 4 |  |
| INTEGER[5] | 6 | entry 5 |  |
| INTEGER[6] | 7 | entry 6 |  |
| INTEGER[7] | 8 | entry 7 |  |
| INTEGER[8] | 9 | entry 8 |  |
| } |  |  |  |
| spatialRelationInfoToAddModList | Not present |  |  |
| spatialRelationInfoToReleaseList | Not present |  |  |
| pucch-PowerControl | PUCCH-PowerControl |  |  |
| resourceToAddModListExt-r16 | Not present |  |  |
| dl-DataToUL-ACK-r16 | Not present |  |  |
| ul-AccessConfigListDCI-1-1-r16 | Not present |  |  |
| subslotLengthForPUCCH-r16 | Not present |  |  |
| dl-DataToUL-ACK-DCI-1-2-r16 | Not present |  |  |
| numberOfBitsForPUCCH-ResourceIndicatorDCI-1-2-r16 | Not present |  |  |
| dmrs-UplinkTransformPrecodingPUCCH-r16 | Not present |  |  |
| spatialRelationInfoToAddModListSizeExt-v1610 | Not present |  |  |
| spatialRelationInfoToReleaseListSizeExt-v1610 | Not present |  |  |
| spatialRelationInfoToAddModListExt-v1610 | Not present |  |  |
| spatialRelationInfoToReleaseListExt-v1610 | Not present |  |  |
| resourceGroupToAddModList-r16 | Not present |  |  |
| resourceGroupToReleaseList-r16 | Not present |  |  |
| sps-PUCCH-AN-List-r16 | Not present |  |  |
| schedulingRequestResourceToAddModListExt-v1610 | Not present |  |  |
| } |  |  |  |

<Unchanged Text Skipped>

–	PUSCH-Config

Table 4.6.3-118: PUSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPUSCH | Not present |  |  |
| txConfig | Not Present |  | Short_DCI |
|  | codebook |  |  |
| dmrs-UplinkForPUSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-UplinkConfig |  |  |
| } |  |  |  |
| dmrs-UplinkForPUSCH-MappingTypeB | Not present |  |  |
| pusch-PowerControl | PUSCH-PowerControl |  |  |
| frequencyHopping | Not present |  |  |
| frequencyHoppingOffsetLists | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
| pusch-TimeDomainAllocationList | Not present |  |  |
| pusch-AggregationFactor | Not present |  |  |
| mcs-Table | Not present |  |  |
| mcs-TableTransformPrecoder | Not present |  |  |
| transformPrecoder | enabled |  | TRANSFORM_PRECODER_ENABLED |
|  | Not present | TRANSFORM_PRECODER_DISABLED |  |
| codebookSubset | Not present |  | Short_DCI |
|  | nonCoherent |  |  |
| maxRank | Not present |  | Short_DCI |
|  | 1 |  |  |
|  | 2 |  | 2TX_UL_MIMO |
| rbg-Size | Not present |  |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| betaOffsets CHOICE { |  |  |  |
| semiStatic | BetaOffsets |  |  |
| } |  |  |  |
| scaling | f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| tp-pi2BPSK | Not present |  |  |
| minimumSchedulingOffsetK2-r16 | Not present |  |  |
| ul-AccessConfigListDCI-0-1-r16 | Not present |  |  |
| harq-ProcessNumberSizeDCI-0-2-r16 | Not present |  |  |
| dmrs-SequenceInitializationDCI-0-2-r1 | Not present |  |  |
| numberOfBitsForRV-DCI-0-2-r16 | Not present |  |  |
| antennaPortsFieldPresenceDCI-0-2-r16 | Not present |  |  |
| dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2-r16 | Not present |  |  |
| dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2-r16 | Not present |  |  |
| frequencyHoppingDCI-0-2-r16 | Not present |  |  |
| frequencyHoppingOffsetListsDCI-0-2-r16 | Not present |  |  |
| codebookSubsetDCI-0-2-r16 | Not present |  |  |
| invalidSymbolPatternIndicatorDCI-0-2-r16 | Not present |  |  |
| maxRankDCI-0-2-r16 | Not present |  |  |
| mcs-TableDCI-0-2-r16 | Not present |  |  |
| mcs-TableTransformPrecoderDCI-0-2-r16 | Not present |  |  |
| priorityIndicatorDCI-0-2-r16 | Not present |  |  |
| pusch-RepTypeIndicatorDCI-0-2-r16 | Not present |  |  |
| resourceAllocationDCI-0-2-r16 | Not present |  |  |
| resourceAllocationType1GranularityDCI-0-2-r16 | Not present |  |  |
| uci-OnPUSCH-ListDCI-0-2-r16 | Not present |  |  |
| pusch-TimeDomainAllocationListDCI-0-2-r16 | Not present |  |  |
| pusch-TimeDomainAllocationListDCI-0-1-r16 | Not present |  |  |
| invalidSymbolPatternIndicatorDCI-0-1-r16 | Not present |  |  |
| priorityIndicatorDCI-0-1-r16 | Not present |  |  |
| pusch-RepTypeIndicatorDCI-0-1-r16 | Not present |  |  |
| frequencyHoppingDCI-0-1-r16 | Not present |  |  |
| uci-OnPUSCH-ListDCI-0-1-r16 | Not present |  |  |
| invalidSymbolPattern-r16 | Not present |  |  |
| pusch-PowerControl-v1610 | Not present |  |  |
| ul-FullPowerTransmission-r16 | Not present |  |  |
| pusch-TimeDomainAllocationListForMultiPUSCH-r16 | Not present |  |  |
| numberOfInvalidSymbolsForDL-UL-Switching-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (DFT-s-OFDM UL waveform is configured) |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |

<Unchanged Text Skipped>

–	PUSCH-ServingCellConfig

Table 4.6.3-121: PUSCH-ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| codeBlockGroupTransmission | Not present |  |  |
| rateMatching | Not present |  |  |
| xOverhead | Not present |  |  |
| maxMIMO-Layers | Not present |  |  |
| processingType2Enabled | Not present |  |  |
| maxMIMO-LayersDCI-0-2-r16 | Not present |  |  |
| } |  |  |  |

<Unchanged Text Skipped>

–	RLC-BearerConfig

Table 4.6.3-148: RLC-BearerConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRBn |  | DRBn |
|  | LogicalChannelIdentity with condition SRB1 |  | SRB1 |
|  | LogicalChannelIdentity with condition SRB2 |  | SRB2 |
|  | LogicalChannelIdentity with condition SRB3 |  | SRB3 |
| servedRadioBearer CHOICE { |  |  |  |
| srb-Identity | SRB-Identity with condition SRB1 |  | SRB1 |
|  | SRB-Identity with condition SRB2 |  | SRB2 |
|  | SRB-Identity with condition SRB3 |  | SRB3 |
| drb-Identity | DRB-Identity with condition DRBn |  | DRBn |
| } |  |  |  |
| servedRadioBearer | Not present |  | RESUME |
| reestablishRLC | Not present |  |  |
|  | true |  | Re-establish_RLC, RESUME |
| rlc-Config | RLC-Config using condition AM |  | AM |
|  | RLC-Config using condition UM. |  | UM |
|  | Not present | Use default parameters as per TS 38.331 [6] clause 9.2.1 | SRB1, SRB2, SRB3, RESUME |
| mac-LogicalChannelConfig | LogicalChannelConfig using condition HI |  | AM |
|  | LogicalChannelConfig using condition LO |  | UM |
|  | LogicalChannelConfig using condition SRBn | n= 1, 2, 3 for SRB1, SRB2, SRB3 resp. | SRB1, SRB2, SRB3, RESUME |
| rlc-Config-v1610 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| AM | RLC AM DRB |
| UM | RLC UM DRB |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| SRB3 | Establishment of SRB3 |
| DRBn | Establishment of DRBn |
| Re-establish_RLC | Re-establishment of RLC |
| RESUME | Used in RRCResume Message |

<Unchanged Text Skipped>

–	SRS-Config

Table 4.6.3-182: SRS-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToReleaseList | Not present |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet { | 1 entry |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceSetId | 0 |  |  |
| srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId { | 1 entry |  |  |
| SRS-ResourceId[1] | 0 | entry 1 |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| aperiodicSRS-ResourceTrigger | 1 |  |  |
| csi-RS | Not present |  |  |
| slotOffset | 7 |  | FR1 |
|  | 4 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| usage | codebook |  |  |
| alpha | Alpha |  |  |
| p0 | 0 |  |  |
| pathlossReferenceRS CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| srs-PowerControlAdjustmentStates | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-ResourceToReleaseList | Not present |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource { | 1 entry |  |  |
| SRS-Resource[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceId | 0 |  |  |
| nrofSRS-Ports | ports2 |  | 2TX_UL_MIMO |
|  | port1 |  |  |
| ptrs-PortIndex | Not present |  |  |
| transmissionComb CHOICE { |  |  |  |
| n2 SEQUENCE { |  |  |  |
| combOffset-n2 | 0 |  |  |
| cyclicShift-n2 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  |  |
| nrofSymbols | n1 |  |  |
| repetitionFactor | n1 |  |  |
| } |  |  |  |
| freqDomainPosition | 0 |  |  |
| freqDomainShift | 0 |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 0 |  |  |
|  | 63 |  | FR1_100MHz |
|  | 17 |  | FR2_100MHz |
| b-SRS | 0 |  |  |
| b-hop | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping | groupHopping |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId | 0 |  |  |
| spatialRelationInfo SEQUENCE { | SRS-SpatialRelationInfo |  |  |
| servingCellId | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| tpc-Accumulation | Not present |  |  |
| srs-RequestDCI-1-2-r16 | Not present |  |  |
| srs-RequestDCI-0-2-r16 | Not present |  |  |
| srs-ResourceSetToAddModListDCI-0-2-r16 | Not present |  |  |
| srs-ResourceSetToReleaseListDCI-0-2-r16 | Not present |  |  |
| srs-PosResourceSetToReleaseList-r16 | Not present |  |  |
| srs-PosResourceSetToAddModList-r16 | Not present |  |  |
| srs-PosResourceToReleaseList-r16 | Not present |  |  |
| srs-PosResourceToAddModList-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| FR1_100MHz | FR1 is used under the test. CBW is set to 100MHz. |
| FR2_100MHz | FR2 is used under the test. CBW is set to 100MHz. |

<End of Changes >