3GPP TSG-RAN5 Meeting #91-e	R5-212967

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5317 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 6.6.2.3A.1_3 ACLR for CA for HPUE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_TDD_HPUE_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TPs and the reference table in test step 6 need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Test points according to test case 6.2.3A.1_3 MPR for CA and HPUE; Updated test procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.3A.1_3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

6.6.2.3A.1_3	Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for HPUE

6.6.2.3A.1_3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for CA.

6.6.2.3A.1_3.2	Test applicability

This test applies to all types of E-UTRA UE Power Class 2 release 10 and forward that support intra band contiguous DL CA and UL CA.

6.6.2.3A.1_3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.1_3.4	Test description

6.6.2.3A.1_3.4.1	Initial condition

Use 6.6.2.3A.1.4.1 except replace Table 6.6.2.3A.1.4.1-1 with Table 6.6.2.3A.1_3.4.1-1.

Table 6.6.2.3A.1_3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  |  |  |  |  | Low and High range |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  |  |  | DL Allocation |  |  | CC MOD |  | UL Allocation |  |  |  |  |  |  |  |  |  |
| ID | PCC NRB |  | SCCs NRB |  | PCC & SCC RB allocation |  |  |  |  | NRB_ alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |  |  |  |  |
| 1 | 100 |  | 25 |  |  |  |  | QPSK |  | 6 |  | P_6@0 |  | S_0@0 |  |  |  |  |  |
| 2 | 100 |  | 25 |  |  |  |  | QPSK |  | 25 |  | P_25@0 |  | S_0@0 |  |  |  |  |  |
| 3 | 100 |  | 25 |  |  |  |  | QPSK |  | 125 |  | P_100@0 |  | S_25@0 |  |  |  |  |  |
| 4 | 100 |  | 25 |  |  |  |  | 16QAM |  | 6 |  | P_6@0 |  | S_0@0 |  |  |  |  |  |
| 5 | 100 |  | 25 |  |  |  |  | 16QAM |  | 25 |  | P_25@0 |  | S_0@0 |  |  |  |  |  |
| 6 | 100 |  | 25 |  |  |  |  | 16QAM |  | 125 |  | P_100@0 |  | S_25@0 |  |  |  |  |  |
| 7 | 100 |  | 25 |  |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@24 |  |  |  |  |  |
| 8 | 75 |  | 75 |  |  |  |  | QPSK |  | 6 |  | P_6@0 |  | S_0@0 |  |  |  |  |  |
| 9 | 75 |  | 75 |  |  |  |  | QPSK |  | 75 |  | P_75@0 |  | S_0@0 |  |  |  |  |  |
| 10 | 75 |  | 75 |  |  |  |  | QPSK |  | 150 |  | P_75@0 |  | S_75@0 |  |  |  |  |  |
| 11 | 75 |  | 75 |  |  |  |  | 16QAM |  | 16 |  | P_16@0 |  | S_0@0 |  |  |  |  |  |
| 12 | 75 |  | 75 |  |  |  |  | 16QAM |  | 75 |  | P_75@0 |  | S_0@0 |  |  |  |  |  |
| 13 | 75 |  | 75 |  |  |  |  | 16QAM |  | 150 |  | P_75@0 |  | S_75@0 |  |  |  |  |  |
| 14 | 75 |  | 75 |  |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@74 |  |  |  |  |  |
| 15 | 100 |  | 50 |  |  |  |  | QPSK |  | 6 |  | P_6@0 |  | S_0@0 |  |  |  |  |  |
| 16 | 100 |  | 50 |  |  |  |  | QPSK |  | 50 |  | P_50@0 |  | S_0@0 |  |  |  |  |  |
| 17 | 100 |  | 50 |  |  |  |  | QPSK |  | 150 |  | P_100@0 |  | S_50@0 |  |  |  |  |  |
| 18 | 100 |  | 50 |  |  |  |  | 16QAM |  | 6 |  | P_6@0 |  | S_0@0 |  |  |  |  |  |
| 19 | 100 |  | 50 |  |  |  |  | 16QAM |  | 50 |  | P_50@0 |  | S_0@0 |  |  |  |  |  |
| 20 | 100 |  | 50 |  |  |  |  | 16QAM |  | 150 |  | P_100@0 |  | S_50@0 |  |  |  |  |  |
| 21 | 100 |  | 50 |  |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@49 |  |  |  |  |  |
| 22 | 100 |  | 75 |  |  |  |  | QPSK |  | 6 |  | P_6@0 |  | S_0@0 |  |  |  |  |  |
| 23 | 100 |  | 75 |  |  |  |  | QPSK |  | 75 |  | P_75@0 |  | S_0@0 |  |  |  |  |  |
| 24 | 100 |  | 75 |  |  |  |  | QPSK |  | 175 |  | P_100@0 |  | S_75@0 |  |  |  |  |  |
| 25 | 100 |  | 75 |  |  |  |  | 16QAM |  | 16 |  | P_16@0 |  | S_0@0 |  |  |  |  |  |
| 26 | 100 |  | 75 |  |  |  |  | 16QAM |  | 75 |  | P_75@0 |  | S_0@0 |  |  |  |  |  |
| 27 | 100 |  | 75 |  |  |  |  | 16QAM |  | 175 |  | P_100@0 |  | S_75@0 |  |  |  |  |  |
| 28 | 100 |  | 75 |  |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@74 |  |  |  |  |  |
| 29 | 100 |  | 100 |  |  |  |  | QPSK |  | 6 |  | P_6@0 |  | S_0@0 |  |  |  |  |  |
| 30 | 100 |  | 100 |  |  |  |  | QPSK |  | 100 |  | P_100@0 |  | S_0@0 |  |  |  |  |  |
| 31 | 100 |  | 100 |  |  |  |  | QPSK |  | 200 |  | P_100@0 |  | S_100@0 |  |  |  |  |  |
| 32 | 100 |  | 100 |  |  |  |  | 16QAM |  | 18 |  | P_18@0 |  | S_0@0 |  |  |  |  |  |
| 33 | 100 |  | 100 |  |  |  |  | 16QAM |  | 100 |  | P_100@0 |  | S_0@0 |  |  |  |  |  |
| 34 | 100 |  | 100 |  |  |  |  | 16QAM |  | 200 |  | P_100@0 |  | S_100@0 |  |  |  |  |  |
| 35 | 100 |  | 100 |  |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@99 |  |  |  |  |  |
| 36 | 75 |  | 50 |  |  |  |  | QPSK |  | 6 |  | P_6@0 |  | S_0@0 |  |  |  |  |  |
| 37 | 75 |  | 50 |  |  |  |  | QPSK |  | 25 |  | P_25@0 |  | S_0@0 |  |  |  |  |  |
| 38 | 75 |  | 50 |  |  |  |  | QPSK |  | 125 |  | P_75@0 |  | S_50@0 |  |  |  |  |  |
| 39 | 75 |  | 50 |  |  |  |  | 16QAM |  | 6 |  | P_6@0 |  | S_0@0 |  |  |  |  |  |
| 40 | 75 |  | 50 |  |  |  |  | 16QAM |  | 25 |  | P_25@0 |  | S_0@0 |  |  |  |  |  |
| 41 | 75 |  | 50 |  |  |  |  | 16QAM |  | 125 |  | P_75@0 |  | S_50@0 |  |  |  |  |  |
| 42 | 75 |  | 50 |  |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@24 |  |  |  |  |  |
| 43 | 100 |  | 25 |  |  |  |  | 64QAM |  | 8 |  | P_8@0 |  | S_0@0 |  |  |  |  |  |
| 44 | 100 |  | 25 |  |  |  |  | 64QAM |  | 125 |  | P_100@0 |  | S_25@0 |  |  |  |  |  |
| 45 | 75 |  | 75 |  |  |  |  | 64QAM |  | 16 |  | P_16@0 |  | S_0@0 |  |  |  |  |  |
| 46 | 75 |  | 75 |  |  |  |  | 64QAM |  | 150 |  | P_75@0 |  | S_75@0 |  |  |  |  |  |
| 47 | 100 |  | 50 |  |  |  |  | 64QAM |  | 12 |  | P_12@0 |  | S_0@0 |  |  |  |  |  |
| 48 | 100 |  | 50 |  |  |  |  | 64QAM |  | 150 |  | P_100@0 |  | S_50@0 |  |  |  |  |  |
| 49 | 100 |  | 75 |  |  |  |  | 64QAM |  | 16 |  | P_16@0 |  | S_0@0 |  |  |  |  |  |
| 50 | 100 |  | 75 |  |  |  |  | 64QAM |  | 175 |  | P_100@0 |  | S_75@0 |  |  |  |  |  |
| 51 | 100 |  | 100 |  |  |  |  | 64QAM |  | 18 |  | P_18@0 |  | S_0@0 |  |  |  |  |  |
| 52 | 100 |  | 100 |  |  |  |  | 64QAM |  | 200 |  | P_100@0 |  | S_100@0 |  |  |  |  |  |
| 53 | 75 |  | 50 |  |  |  |  | 64QAM |  | 12 |  | P_12@0 |  | S_0@0 |  |  |  |  |  |
| 54 | 75 |  | 50 |  |  |  |  | 64QAM |  | 125 |  | P_75@0 |  | S_50@0 |  |  |  |  |  |
| NOTE 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.6.2.3A.1_3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.3A.1.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3A.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers of the UE in the CA configuration of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3A.1_3.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the rectangular filtered mean power for CA E-UTRA.

8.	Measure the rectangular filtered mean power of the first CA E-UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel, respectively.

9.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel, respectively.

10.	Calculate the ratio of the power between the values measured in step 7 over step 8 for CA E-UTRAACLR.

11.	Calculate the ratio of the power between the values measured in step 7 over step 9 for UTRAACLR1, UTRAACLR2.

6.6.2.3A.1_3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.3A.1_3.5	Test Requirements

6.6.2.3A.1_3.5.1	Test requirements of UTRA for CA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1 and UTRAACLR2, derived in step 11, shall be higher than the limits in table 6.6.2.3A.1.5.1-1.

Table 6.6.2.3A.1.5.1-1: UTRA UE ACLR for CA

|  | CA bandwidth class / UTRAACLR1/2 / measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| UTRAACLR1 | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + BWUTRA/2 / - BWChannel_CA / 2 - BWUTRA/2 |
| UTRAACLR2 | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + 3*BWUTRA/2 / - BWChannel_CA /2 – 3*BWUTRA/2 |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| UTRA 5MHz channel Measurement bandwidth (Note 1) | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth (Note 2) | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |

6.6.2.3A.1_3.5.2	Test requirements of CA E-UTRA

-	The measured UE mean power in the channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.1.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured CA E-UTRAACLR, derived in step 10, shall be higher than the limits in table 6.6.2.3A.1.5.2-1.

Table 6.6.2.3A.1_3.5.2-1: CA E-UTRA ACLR

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| CA E-UTRAACLR | 29.2 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

<End of modified section>