3GPP TSG-RAN5 Meeting #91-e

# Electronic Meeting, 17th – 28th May 2021

Source:      Samsung, Huawei

Title:         TP analysis on FR1 6.5A.3.2.1 Spurious emission for UE co-existence for CA (2UL CA)

Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection and agreement in [3] for FR1 SC NR spurious emission for UE co-existence, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for NR spurious emission for UE co-existence of 2UL inter-band CA case.

Discussion

In LTE specification [4], the test configuration for UL and DL inter-band CA is defined for QPSK and 16QAM as table below.

Table 6.6.3.2A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  | CA Configuration / NRB_agg (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  |  |  |  |  |  |  | UL Allocation (Note 2,3) |  |  |  |  |  |  |  |  |
|  |  |  | CA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC NRB |  |  |  |  |  | SCC NRB |  |  |  |  | CC MOD |  |  |  |  | PCC & SCC RB allocation |  |  |  |  |  | CC MOD |  |  |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |  |
|  |  |  | PCC |  |  |  |  |  |  |  |  |  |  | SCC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC |  | SCC |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Band |  |  |  |  |  | Range |  |  |  |  | Band |  |  |  |  | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  |  |  |  | Low |  |  |  |  | Y |  |  |  |  | Low |  |  |  |  | Highest NRB |  |  |  |  |  | Highest NRB |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  |  | 1@0 |  |  |  |  | 1@0 |  |
| 2 | X |  |  |  |  | High |  |  |  |  | Y |  |  |  |  | High |  |  |  |  | Highest NRB |  |  |  |  |  | Highest NRB |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  |  | 1@RBmax |  |  |  |  | 1@RBmax |  |
| Test Settings for CA_1A-3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  |  |  |  | Low |  |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |  |
| 2 |  | 1 |  |  |  |  | Low |  |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |  |
| 3 |  | 1 |  |  |  |  | High |  |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |  |
| 4 |  | 1 |  |  |  |  | High |  |  |  |  | 3 |  |  |  |  | Note 7 |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |  |
| Test Settings for CA_1A-7A Configuration(Note 12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |  |
| 2 |  | 1 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |  |
| Test Settings for CA_1A-8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |  |
| 2 |  | 1 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |  |
| Test Settings for CA_1A-18A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  | Low |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  | 1 |  |  |  |  | Low |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@74 |  |
| Test Settings for CA_1A-19A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |  |
| 2 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 25@0 |  |  | 25@0 |  |
| 3 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |  |
| 4 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 100@0 |  |  | 75@0 |  |
| Test Settings for CA_1A-21A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@24 |  |  | 1@24 |  |
| 2 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@24 |  |  | 1@12 |  |
| 3 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 25@0 |  |  | 25@0 |  |
| 4 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@74 |  |
| 5 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@37 |  |
| 6 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 100@0 |  |  | 75@0 |  |
| Test Settings for CA_1A-28A Configuration (Note 16) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Mid |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@49 |
| 2 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Low |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 3 |  |  | 1 |  |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | Mid |  |  |  |  |  |  | 100 |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@47 |
| 4 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Mid |  |  |  |  |  |  | 50 |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@49 |
| 5 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Low |  |  |  |  |  |  | 50 |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 6 |  |  | 1 |  |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | Mid |  |  |  |  |  |  | 50 |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@49 |  |  | 1@47 |
| Test Settings for CA_1A-42A Configuration (Note 13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 2 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@99 |
| 3 |  |  | 1 |  |  |  |  |  | High |  |  |  |  | 42 |  |  |  |  | Mid |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@24 |
| 4 |  |  | 1 |  |  |  |  |  | High |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@49 |  |  | 1@0 |
| Test Settings for CA_2A-4A Configuration (Note 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 3 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 4 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |  |
| 5 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 6 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 7 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 8 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 9 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| 10 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| Test Settings for CA_2A-7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| Test Settings for CA_3A-5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 3 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 4 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 5 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 6 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| Test Settings for CA_3A-7A Configuration (Note 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 3 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@75 |  |  |  | 1@99 |  |
| 4 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| 5 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| 6 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 7 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| Test Settings for CA_3A-8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 3 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 4 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 5 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 6 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 7 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 8 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 9 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| Test Settings for CA_3A-18A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@74 |  |
| Test Settings for CA_3A-19A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |  |
| 2 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@24 |  |  | 1@0 |  |
| 3 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 25@0 |  |  | 25@0 |  |
| 4 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |  |
| 5 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@0 |  |
| 6 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 100@0 |  |  | 75@0 |  |
| Test Settings for CA_3A-20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 3 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| 4 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 5 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 6 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| Test Settings for CA_3A-26A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 26 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 26 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 3 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 26 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 4 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 26 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |  |
| 5 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 26 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 6 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 26 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@74 |  |
| 7 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 26 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@74 |  |
| 8 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 26 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| Test Settings for CA_3A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | High |  |  |  |  | 28 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  | 50 |  |  |  |  |  | High |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 2 |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  | 50 |  |  |  |  |  | Low |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| Test Settings for CA_4A-5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| 3 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 4 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 5 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 6 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| Test Settings for CA_4A-7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 3 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |  |
| 4 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@99 |  |
| 5 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |  |
| 6 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| 7 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 8 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| 9 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 10 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 11 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 12 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| Test Settings for CA_5A-7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@99 |  |
| 2 |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@24 |  |  |  | 1@0 |  |
| 3 |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |  |
| Test Settings for CA_5A-12A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 12 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 2 |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 12 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| Test Settings for CA_5A-17A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 17 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| Test Settings for CA_7A-20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |  |
| 2 |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 3 |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |  |
| 4 |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 5 |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| Test Settings for CA_7A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@74 |  |
| 2 |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| Test Settings for CA_18A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Note 15 |  |  |  |  | 75 |  |  |  |  |  |  | 50 |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@2 |  |
| 2 |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Note 15 |  |  |  |  | 75 |  |  |  |  |  |  | 50 |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@74 |  |  |  | 1@47 |  |
| Test Settings for CA_39A-41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 39 |  |  |  |  | High |  |  |  |  | 41 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| Test Settings for CA_41A-42A Configuration (Note 14) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 41 |  |  |  |  | Note 8 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| 2 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | Note 11 |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@22 |  |  |  | 1@99 |  |
| 3 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@44 |  |  |  | 1@99 |  |
| 4 |  |  |  | 41 |  |  |  |  | Note 8 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 5 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| 6 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@44 |  |  |  | 1@99 |  |
| 7 |  |  |  | 41 |  |  |  |  | Note 10 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |  |
| 8 |  |  |  | 41 |  |  |  |  | Note 8 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| 9 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 10 |  |  |  | 41 |  |  |  |  | Low |  |  |  |  | 42 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table. Otherwise use the Default Test Settings test points. Note 3: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_1A-19A,X=1,Y=19. Note 4: For CA configurations CA_1A-18A, CA_1A-19A, CA_1A-21A , CA_3A-19A and CA_18A-28A each test point shall be tested. For all other CA configurations only test points with highest NRB_agg , according to UE supported BCS shall be tested. Note 5: Test Point ID 7 for CA_3A-7A is applicable only for a Rel-13 and forward UE. Note 6: Test Point ID 3 and Test Point ID 8 for CA_2A-4A are applicable only for a Rel-14 and forward UE. Note 7: Test Point ID 4 for CA_1A-3A has the centre carrier frequency 1849.9 MHz, i.e. DL E-ARFCN is 1649. Note 8: Test Point ID 1, 4 and 8 for CA_41A-42A have the centre carrier frequency of 2635 MHz in Band 41 (EARFCN=41040). Note 9: Test Point ID 2, 3, 5, 6 and 9 for CA_41A-42A has the centre carrier frequency of 2555 MHz in Band 41 (EARFCN=40240). Note 10: Test Point ID 7 for CA_41A-42A has the centre carrier frequency of 2674 MHz in Band 41 (EARFCN=41430). Note 11: Test Point ID 2 for CA_41A-42A has the centre carrier frequency of 3421 MHz in Band 42 (EARFCN=41800). Note 12: Test Point ID 2 for CA_1A-7A is applicable only for a Rel-15 and forward UE. Note 13: Test Point ID 4 for CA_1A-42A is applicable only for a Rel-15 and forward UE. Note 14: Test Point ID 10 for CA_41A-42A is applicable only for a Rel-15 and forward UE. Note 15: Test Point ID 1 and 2 for CA_18A-28A have the centre carrier frequency of 778 MHz in Band 28 (EARFCN=9410). Note 16: As an exception to Note 4, Test Point ID 3 for CA_1A-28A shall be tested for a UE supporting bandwidth combination set 0, even though it has not highest NRB_agg for this UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

In NR specification [5], the Test configuration for spurious emission for UE co-existence for single carrier mode is defined as table below.

Table 6.5.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1. |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest supported SCS per test channel BW, Highest supported SCS per test channel BW |  |
| Test Parameters |  |  |  |
| Ch BW | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 5 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 10 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 15 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 20 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 25 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 30 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 40 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 50 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 60 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 80 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 90 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| 100 MHz |  | CP-OFDM QPSK | Outer_Full |
|  |  |  | Edge_1RB_Left |
|  |  |  | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration. |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

Test Environment

In LTE, Spurious emission for UE coexistence of CA is tested under NC only. According to baseline endorsed in [1], it’s proposed to reuse NC for Spurious emission of inter-band UL CA.

Proposal 1: Define test environment as NC for spurious emission for UE coexistence in FR1 for UL inter-band CA.

Test Subcarrier Spacing

In [3] the smallest SCSis selected for Single Carrier Spurious emission for UE coexistence verification. It is proposed to reuse the same way for inter-band UL CA.

Proposal 2: Select smallest SCS for Spurious emission for UE coexistence in FR1 for inter-band UL CA.

Test Frequencies

In LTE, Spurious emission for UE coexistence of inter-band CA is tested for band combination specific condition. Hence it proposed to reuse the LTE method for NR inter-band UL CA.

Proposal 3: Reuse the band combination specific condition for spurious emission for UE coexistence of UL inter-band CA regarding frequency range under testing.

Test Channel Bandwidth

In LTE, Spurious emission for UE coexistence of inter-band CA is tested for highest BW for both PCC and SCC. Hence it proposed to reuse the LTE method for NR inter-band UL CA.

Proposal 4: Test with highest BW for spurious emission for UE coexistence of UL inter-band CA.

Test Modulations and waveforms

For single carrier Spurious emission for UE coexistence TC presented in [3] only CP-OFDM with OPSK is selected for testing. For CA case it’s proposed to reuse this configuration.

Proposal 5: Test Spurious emission for UE coexistence for CA with CP-OFDM QPSK.

RB allocation

Considering RB allocation defined in LTE CA and NR SC for Spurious emission, the Edge_1RB_Left and Edge_1RB_Right are proposed for NR CA spurious emissions for UE co-existence test.

Proposal 6: Test Spurious emission for UE coexistence of UL inter-band CA with Edge_1RB_Left and Edge_1RB_Right

Proposal 7: detail analysis for CA_n3-n78 and CA_n8-n78 can be found in TpAnalysisSpur(n3A-n78A)_v3.zip and TpAnalysisSpur(n8A-n78A)_v3.zip. Detail analysis for CA_n28-n41 can be found in TpAnalysisSpur(n28A-n41A).zip.

Number of test points

| Band combination | Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3-n78 | 1 | 1 | 1 | 1 | 1 | 3 |
| CA_n8-n78 | 1 | 1 | 1 | 2 | 2 | 4 |
| CA_n28-n41 | 1 | 1 | 1 | 1 | 4 | 4 |

Conclusion

For inter-band UL CA Spurious emission for UE coeixstence requirement, the test condition is proposed as below:

Proposal 1: Define test environment as NC for spurious emission for UE coexistence in FR1 for UL inter-band CA.

Proposal 2: Select smallest SCS for Spurious emission for UE coexistence in FR1 for inter-band UL CA.

Proposal 3: Reuse the band combination specific condition for spurious emission for UE coexistence of UL inter-band CA regarding frequency range under testing.

Proposal 4: Test with highest BW for spurious emission for UE coexistence of UL inter-band CA.

Proposal 5: Test Spurious emission for UE coexistence for CA with CP-OFDM QPSK.

Proposal 6: Test Spurious emission for UE coexistence of UL inter-band CA with Edge_1RB_Left and Edge_1RB_Right

Proposal 7: detail analysis for CA_n3-n78 and CA_n8-n78 can be found in TpAnalysisSpur(n3A-n78A)_v3.zip and TpAnalysisSpur(n8A-n78A)_v3.zip. Detail analysis for CA_n28-n41 can be found in TpAnalysisSpur(n28A-n41A).zip.

Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-182019, "Discussion of NR FR1 Test Point for TX Spurious Emission test cases "

[4] 3GPP TS 36.521-1 v16.8.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 v17.0.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 17).

[6] 3GPP TS 38.101-1 v16.7.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16).