Source: 	Ericsson, CAICT, Huawei

Title: 	Discussion on test point selection for UTRA ACLR test case in FR1

Introduction

A default baseline for test point selection for TRx test cases was presented in [1] at RAN5#1 5G NR Adhoc meeting, where some proposals for environmental conditions, test frequencies, channel bandwidths, subcarrier spacing and RB allocation for FR1 were based on LTE. Finally, it was stated that each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for UTRA Adjacent Channel Leakage Ratio (ACLR) test in RF1.

Discussion

General

Adjacent Channel Leakage Ratio requirements for FR1 is split into NR ACLR and UTRA ACLR. The actual requirement is different for NR ACLR and UTRA ACLR based in the offset from the transmitting NR channel and the measurement filter used in the adjacent channel. Basically, there is no reason not to use the same configuration for the two cases. However, UTRA ACLR is only for certain network signalling values with NR bands that have UTRA services deployed and UE shall meet the corresponding AMPR requirement while performing UTRA ACLR test. Therefore, the test point for NR ACLR shall be aligned with the corresponding AMPR test points.

Test Environment, Test Frequencies, Test Channel Bandwidth, Modulations, and Uplink Configuration

Based on the assumption in chapter 2.1, test environment, test subcarrier spacing, test frequencies, test channel bandwidth, modulations, and uplink configuration of the corresponding AMPR test applies to ULTR ACLR test.

Proposal 1: Select the test environment, test subcarrier spacing, test frequencies, test channel bandwidth, modulations, and uplink configuration of the corresponding AMPR test for UTRA ACLR in FR1.

Exceptions for UTRA ACLR UL-MIMO testing

The minimum requirement extracted from TS 38.101-1 clause 6.5D.2:

For UE supporting UL MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters is defined as the sum of the emissions from both UEtransmit antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclasuse 6.5.2 apply. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the requirements in clause 6.5.2 shall apply. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.5.2 apply.

Same out of band emission requirements as specified for single transmission apply for UE supporting UL-MIMO. According to [2], CP-OFDM is mandatory in case UL-MIMO transmission with two layer data, DFT-s-OFDM test points are not valid for UL-MIMO. Therefore DFT-s-OFDM is precluded from the test points for UL-MIMO testing.

Proposal 2: Test points with CP-OFDM selected in proposal 1 can be reused by UTRA ACLR UL-MIMO testing. The other test parameters are the same as single transmission case.

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 UTRA ACLR test case definition:

Proposal 1: Select test environment, test subcarrier spacing, test frequencies, test channel bandwidth, modulations, and uplink configuration of the corresponding AMPR test for UTRA ACLR measurement in FR1.

Proposal 2: Test points with CP-OFDM selected in proposal 1 can be reused by UTRA ACLR UL-MIMO testing. The other test parameters are the same as single transmission case.

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019