Title: 	Discussion on test point selection of Configured Transmitted Power in FR1

Source: 	Huawei, HiSilicon, Ericsson

# Introduction

In this contribution, we discuss some points in deciding the test points of Configured transmitted power in FR1.

Firstly the single transmission case is analysed, and then exceptions for UL-MIMO case based on the discrepancy of the waveform is provided.

# Discussion

As a reference, the initial test conditions for RX Reference Sensitivity for LTE is listed (Table 6.2.5.4.1-1 from 36.521) below:

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for Configured UE transmitted Output Power test case |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD | TDD |
| 1.4MHz |  |  | QPSK | 5 | 5 |
| 3MHz |  |  | QPSK | 4 | 4 |
| 5MHz |  |  | QPSK | 8 | 8 |
| 10MHz |  |  | QPSK | 12 | 12 |
| 15MHz |  |  | QPSK | 16 | 16 |
| 20MHz |  |  | QPSK | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For the uplink RB allocation the RBstart shall be RB #0. |  |  |  |  |  |

  - Test Environment

Based on the baseline test environment proposed in [1], test environment for Configured transmitted power for FR1 NR shall follow what is defined as in LTE.

Proposal 1: Propose Test Environment as Normal, TL/VL, TL/VH, TH/VL, TH/VH, for Configured transmitted power in FR1.

  - Test Frequencies

In LTE only Mid range test frequency is tested in Configured transmitted power. In NR the same approach can be adopted.

Proposal 2: Only Mid frequency range is tested for Configured transmitted power.

  - Test Channel Bandwidth

In LTE, Configured transmitted power is tested for Lowest, 5MHz, and Highest channel bandwidths. This assumption seems reasonable to test different behaviours of the UE transmitter related to the different channel bandwidth.

In NR similar approach can be adopted with 5MHz replaced by Mid channel bandwidth defined for each operating band.

Proposal 3: The Lowest, Mid and Highest Channel Bandwidth shall be tested for Configured transmitted power.

  - Test Subcarrier Spacing (SCS)

As discussed in previous test point analysis papers, the key element in choosing subcarrier spacing for testing is to achieve maximum spectrum utilization.  The conclusion is that Lowest SCS will provide maximum spectrum utilization.

Proposal 4: The Lowest subcarrier spacing shall be tested for Configured transmitted power.

  - Modulations and RB allocations

In LTE, Configured transmitted power is tested only for the combinations of modulation and RB allocation for which no MPR applies. The strategy is align with the purpose of this test case, which is to verify the UE behaves properly when P-max is the leading factor of determining the transmitted power.

In NR similar approach can be adopted, and the combinations of DFT-s-OFDM PI/2 BPSK+Inner Full allocation and DFT-s-OFDM QPSK+ Inner Full allocation shall be tested.

Proposal 5: The Configured transmitted power shall be tested with DFT-s-OFDM PI/2 BPSK+Inner Full allocation and DFT-s-OFDM QPSK+ Inner Full allocation

## 2.6 Exceptions for UL-MIMO test

The requirements of Configured transmitted power for UL-MIMO are in a quite similar way of single antenna test case in 6.2.4, therefore the test points selection in proposal 1~4 can be reused without any change.

According to [5], CP-OFDM is mandatory in case UL-MIMO transmission with two layer data and shall be selected for UL-MIMO testing. In single antenna test case, only combinations of modulation and RB allocation for which no MPR applies are tested. However the MPR requirements for CP-OFDM waveform are all above 0dB. Therefore for Configured transmitted power for UL-MIMO the modulation with the smallest MPR requirement is selected, which is CP-OFDM QPSK+Inner Full allocation for both PC2 and PC3.

Proposal 6: The Configured transmitted power for MIMO shall be tested with CP-OFDM QPSK + Inner Full allocation. The other test parameters are the same as test for single-antenna port.

For Rel-16 UEs supporting uplink full power transmission (ULFPTx) for UL MIMO, the requirements specified in TS 38.101-1 clause 6.5D.2 shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode. Table 6.2D.1-3 includes both DFT-s-OFDM and CP-OFDM.

Proposal 7: Test points for Rel-16 ULFPTx UE can be the same as single transmission case without exception.

  - Number of Test Points

AS a comparison, the total test points for Configured transmitted power testing is tabulated for LTE and NR FR1 below in Table 2.6-1 and 2.6-2

LTE:

In LTE Configured transmitted power, the maximum number of test points is shown in Table 2.6-1:

Table 2.6-1 Maximum Number of Test Steps for LTE reference sensitivity

| Environment conditions | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- |
| 5 | 3 | 1 | 1 | 15 |

Number of Ch BW to test: Lowest, 5 MHz, and Highest

Number of Frequencies to test: Mid

Number of RB allocations: different combinations of RB allocations and modulations according LTE MPR

NR FR1

Table 2.6-2 Maximum Number of Test Steps for NR reference sensitivity

| Environment conditions | Number SCS | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 5 | 1 | 3 | 1 | 2 | 30 |

Number of SCS to test: Lowest

Number of ChBw to test: Lowest, Mid and Highest

Number of Frequencies to test: Mid

Number of RB allocations: DFT-s-OFDM PI/2 BPSK+Inner full allocation and DFT-s-OFDM QPSK+ Inner full allocation

Table 2.6-3 Total test points of Configured transmitted power for UL-MIMO

| Environment conditions | Number SCS | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 5 | 1 | 3 | 1 | 1 | 15 |

Number of SCS to test: Lowest

Number of ChBw to test: Lowest, Mid and Highest

Number of Frequencies to test: Mid

Number of RB allocations: CP-OFDM QPSK + Inner Full allocation

# Conclusion

In this document, we discussed several points in deciding the test points of Configured transmitted power in FR1. It is proposed that RAN5 endorse following proposals.

Proposal 1: Propose Test Environment as Normal, TL/VL, TL/VH, TH/VL, TH/VH, for Configured transmitted power in FR1.

Proposal 2: Only Mid frequency range is tested for Configured transmitted power.

Proposal 3: The Lowest, Mid and Highest Channel Bandwidth shall be tested for Configured transmitted power.

Proposal 4: The Lowest subcarrier spacing shall be tested for Configured transmitted power.

Proposal 5: The Configured transmitted power shall be tested with DFT-s-OFDM PI/2 BPSK+Inner allocation and DFT-s-OFDM QPSK+ Inner allocation

Proposal 6: The Configured transmitted power for MIMO shall be tested with CP-OFDM QPSK + Inner Full allocation. The other test parameters are the same as test for single-antenna port.

Proposal 7: Test points for Rel-16 ULFPTx UE can be the same as single transmission case without exception.

# Reference

[1]		R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2]	TS38.101-1 v16.6.0, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 StandaloneUser Equipment (UE) radio access capabilities”

[3] R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019