3GPP TSG-RAN5 Meeting #91-e	R5-213057

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1912 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test frequencies for CA_n48(2A) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Dish Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CA_n48(2A) have been introduced in Rel-16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test frequencies for CA_n48(2A) without UL CA for SCS=15kHz and SCS=30kHz, and for the applicable Sub-Block CBW combinations as per TS 38.101-1 Table 5.5A.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing of CA_n48(2A) not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.5.48 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.5	NR intra-band non-contiguous CA configurations in FR1

###### 4.3.1.1.5.1 – 4.3.1.1.5.47	FFS

###### 4.3.1.1.5.48	CA_n48(2A)

Editor’s note:	Test frequencies for CA_n48(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.48-1: NR Intra-Band non-contiguous CA configuration CA_n48(2A) without UL CA, SCS=15 kHz, Max Wgap

| SB CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n48(2A); n48A (10MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 10+10 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 10+40 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 10+50 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (15MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 15+10 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 15+20 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 15+40 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 15+50 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (20MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 20+10 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 20+15 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 20+20 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 20+40 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 20+50 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (40MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 40+10 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 40+15 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 40+20 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 40+40 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 40+50 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (50MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 50+10 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 50+15 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 50+20 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 50+40 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 50+50 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |

Table 4.3.1.1.5.48-2: NR Intra-Band non-contiguous CA configuration CA_n48(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n48(2A); n48A (10MHz) + n48A (10-100MHz) |  |  |  |  |  |  |
| 10+10 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 10+40 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 10+50 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 10+60 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 10+80 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 10+90 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 10+100 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (15MHz) + n48A (10-100MHz) |  |  |  |  |  |  |
| 15+10 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 15+20 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 15+40 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 15+50 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 15+60 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 15+80 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 15+90 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 15+100 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (20MHz) + n48A (10-100MHz) |  |  |  |  |  |  |
| 20+10 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 20+15 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 20+20 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 20+40 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 20+50 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 20+60 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 20+80 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 20+90 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 20+100 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (40MHz) + n48A (10-100MHz) |  |  |  |  |  |  |
| 40+10 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 40+15 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 40+20 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 40+40 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 40+50 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 40+60 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 40+80 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 40+90 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 40+100 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (50MHz) + n48A (10-90MHz) |  |  |  |  |  |  |
| 50+10 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 50+15 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 50+20 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 50+40 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 50+50 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 50+60 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 50+80 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 50+90 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| CA_n48(2A); n48A (60MHz) + n48A (10-80MHz) |  |  |  |  |  |  |
| 60+10 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 60+15 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 60+20 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 60+40 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 60+50 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 60+60 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 60+80 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| CA_n48(2A); n48A (80MHz) + n48A (10-60MHz) |  |  |  |  |  |  |
| 80+10 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 80+15 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 80+20 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 80+40 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 80+50 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 80+60 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| CA_n48(2A); n48A (90MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 90+10 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 90+15 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 90+20 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 90+40 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 90+50 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| CA_n48(2A); n48A (100MHz) + n48A (10-40MHz) |  |  |  |  |  |  |
| 100+10 | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 100+15 | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 100+20 | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 100+40 | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |

###### 4.3.1.1.5.49 – 4.3.1.1.5.65	FFS

###### 4.3.1.1.5.66	CA_n66(2A)