3GPP TSG-RAN WG5 Meeting #91-e	R5-213090

# Electronic Meeting, 17th - 28th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1276 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of NR FR1 General ON-OFF time mask test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It was brought up in RAN5-81 discussion paper R5-187480 that the current ON/OFF time mask test case is not effective in detecting non-compliant UEs with regard to the time mask requirement. One identified improvement is to ensure the UE is at max power during ON power measurement which will help in detecting non compliant ON->OFF transients in the UE. The same topic is also being discussed in FR2. At RAN5-89-e a proposal to use relative+aggregate power control in the ON/OFF time mask to test at max power was endorsed (R5-206785) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed test procedure from absolute power control to relative power control Changed test requirement for ON power to be same as in MOP test. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-compliant UEs will pass the test leading to performance degradation in networks |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.3.3.2	General ON/OFF time mask

##### 6.3.3.2.1	Test purpose

To verify that the general ON/OFF time mask meets the requirements given in 6.3.3.2.5.

The transmit power time mask for transmit ON/OFF defines the transient period(s) allowed between transmit OFF power as defined in sub-clause 6.3.2 and transmit ON power symbols (transmit ON/OFF)

Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

##### 6.3.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

##### 6.3.3.2.3	Minimum conformance requirements

The general ON/OFF time mask defines the observation period between transmit OFF and ON power and between transmit ON and OFF power for each SCS. ON/OFF scenarios include : the beginning or end of DTX, measurement gap, contiguous, and non-contiguous transmission, etc

The OFF power measurement period is defined in a duration of at least one slot excluding any transient periods. The ON power is defined as the mean power over one slot excluding any transient period.

Figure 6.3.3.2.3-1: General ON/OFF time mask for NR UL transmission in FR1

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3.3.2.

##### 6.3.3.2.4	Test description

6.3.3.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in table 6.3.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for minimum output power | Modulation | RB allocation (NOTE 1) |
| 1 | test case | CP-OFDM QPSK | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3.3.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3.3.2.4.3.

6.3.3.2.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.3.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The UL assignment is  such that the UE transmits on slots 8 for15kHz SCS, on slots 8 and 18 for 30kHz SCS and on slots 17 and 37 for 60kHz SCS.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the UE transmission OFF power during the slot prior to the PUSCH transmission, excluding a transient period of 10 µs in the end of the slot.

4.	Measure the output power of the UE PUSCH transmission during one slot.

5.	Measure the UE transmission OFF power during the slot following the PUSCH transmission, excluding a transient period of 10 µs at the beginning of the slot.

6.3.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exceptions.

Table 6.3.3.2.4.3-1: PUSCH-ConfigCommon

| Derivation Path: TS 38.508-1[5], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -100 |  |  |
| } |  |  |  |

Table 6.3.3.2.4.3-2: Void

Table 6.3.3.2.4.3-3: TDD-UL-DL-Config

| Derivation Path: TS 38.508-1[5], Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms5 |  | FR1 |
|  | ms10 |  | FR1_15kHz |
| nrofDownlinkSlots | 6 |  | FR1_15kHz |
|  | 6 |  | FR1_30kHz |
|  | 14 |  | FR1_60kHz |
| nrofDownlinkSymbols | 10 |  | FR1_15kHz |
|  | 6 |  | FR1_30kHz |
|  | 12 |  | FR1_60kHz |
| nrofUplinkSlots | 3 |  | FR1_15kHz,FR1_30kHz |
|  | 4 |  | FR1_60kHz |
| nrofUplinkSymbols | 4 |  | FR1_30kHz |
|  | 2 |  | FR1_15kHz, |
|  | 8 |  | FR1_60kHz |
| } |  |  |  |
| pattern2 | Not present |  |  |
| } |  |  |  |

Table 6.3.3.2.4.3-4: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1[5], Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| k2 | 4 |  | FR1_15kHz,FR1_30kHz |
|  | 6 |  | FR1_60kHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| k2 | 2 | K2+ Δ=4 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1_15kHz |
|  | 6 | K2+ Δ=9 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1_30kHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Values are chosen so that first slot of a TDD-UL-DL slot configuration period can be used for the Random Access Response and the last slot (of the same or another period) for the corresponding Msg3. |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1_15kHz | FR1 is used under the test. SCS is set to 15kHz. |
| FR1_30kHz | FR1 is used under the test. SCS is set to 30kHz. |
| FR1_60kHz | FR1 is used under the test. SCS is set to 60kHz. |

Table 6.3.3.2.4.3-5: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 18 |  | SCS_15kHz |
|  | 21 |  | SCS_30kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for SS/PBCH block |
| SCS_30kHz | SCS=30kHz for SS/PBCH block |

##### 6.3.3.2.5	Test requirement

The requirement for the power measured in steps 2, 3 and 4 of the test procedure shall not exceed the values specified in Table 6.3.3.2.5-1.

Table 6.3.3.2.5-1: General ON/OFF time mask

|  | SCS | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |  | 30 MHz |  | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power |  |  | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  |  | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 |  | 28.815 |  | 38.895 | 48.615 | 58.35 | 78.15 | 88.23 | 98.31 |
| Transmit ON power |  |  |  |  |  |  |  | Same as Table 6.2.1.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3.3.2.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3.3.2.5-2: Test Tolerance for OFF power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

Table 6.3.3.2.5-3: Test Tolerance for ON power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

#### 6.3.3.3	Transmit power time mask for slot and short or subslot boundaries

No test case details are specified. Current test procedures for time masks are based on power measurement in relatively long period compared with transient period. For time masks between 2 active time slots with different power level, the test procedure can’t provide enough resolution to identify non-conformant UEs. Therefore the minimum requirement is not testable.