3GPP TSG-RAN WG5 Meeting #91-e	R5-213127

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1463 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 15.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per IR.92v10 and onwards, The CS version of Communication Forwarding on Not Reachable (CFNRc) implies diversion when the user is not registered in the CS core or cannot be reached. To mimic this behaviour, it is recommended that an UE activates both the CFNRc (CDIV using condition not-reachable) and the Communication Forwarding on Not Logged-in (CFNL) (CDIV using condition not-registered) to the same target. Based on above reference, to have compatibility with CS domain, for CFNL to be activated, Simservs document shall include CFNRc as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added rule for Communication Diversion on Not reachable in xml body of 200 OK. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case as CFNL can not be activated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

## 15.10	Communication Forwarding on Not logged-in

### 15.10.1	Definition

Test to verify that the UE activates and deactivates IMS Multimedia Telephony Communication Forwarding for the case when user is not registered to IMS service. This process is described in 3GPP TS 24.604 [106].

### 15.10.2	Conformance requirement

Generic requirements for Communication Forwarding can be found from Annexes F.1 and F.4.

[TS 24.604, clause 4.2.1.7]:

The Communication Forwarding on Not Logged-in (CFNL) service enables a served user to redirect incoming communications which are addressed to the served user's address, to another user (forwarded-to address) in case the served user is not registered (logged-in). The CFNL service may operate on all communications, or just those associated with specified basic services.

As a service provider option, a subscription option can be provided to enable the served user to receive an indication that the CFNL service has been activated. This indication shall be provided when the served user logs out according to procedures described in RFC 3261

The maximum number of diversions permitted for each communication is a service provider option. The service provider shall define the upper limit of diversions. When counting the number of diversions, all types of diversion are included.

Reference(s)

3GPP TS 24.604 [106]

### 15.10.3	Test purpose

1)	To verify that the UE can request activation of Communication Forwarding (when the called user is not logged in) with a correctly composed HTTP PUT request; and

2)	To verify that the UE can request deactivation of Communication Forwarding; and

3)	To verify that the UE can authenticate its HTTP requests by including a correctly composed Authorization header with credentials of the user to the request. The UE may either include the Authorization header to its initial request or when sending the request again after receiving 401 response from SS.

### 15.10.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE is configured with the name of the XCAP root directory on the XCAP server and the user's directory name. If needed the UE is also configured with the HTTP Digest password to be used for XCAP. UE has activated an IPCAN bearer (e.g. PDP context or EPS bearer) with SS.

SS is configured with the HTTP Digest password for XCAP or shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE.

If the UE uses GAA as XCAP authentication scheme, GAA bootstrapping exchange has been performed according to annex C.29.2.

Test procedure

The generic test procedure according to annex C.29.1 is applied:

At step 1 activation of Communication Forwarding on Not logged-in;

At step 5b, SS delivers a simservs document as specified in TS 24.604 [106] cl 4.9, including a non-empty rule set. Specifically, the SS includes the following:

<?xml version="1.0" encoding="UTF-8"?>

<simservs

xmlns="http://uri.etsi.org/ngn/params/xml/simservs/xcap"

xmlns:cp="urn:ietf:params:xml:ns:common-policy"

xmlns:ocp="urn:oma:xml:xdm:common-policy">

<communication-diversion active="true">

<cp:ruleset>

<cp:rule id="rule1">

<cp:conditions>

<not-registered/>

<rule-deactivated/>

</cp:conditions>

<cp:actions>

<forward-to>

<target>

px_XCAP_TargetUri

</target>

<notify-caller>true</notify-caller>

</forward-to>

</cp:actions>

</cp:rule>

<cp:rule id="rule2">

<cp:conditions>

<not-reachable/>

<rule-deactivated/>

</cp:conditions>

<cp:actions>

<forward-to>

<target>

px_XCAP_TargetUri

</target>

<notify-caller>true</notify-caller>

</forward-to>

</cp:actions>

</cp:rule>

</cp:ruleset>

</communication-diversion>

</simservs>

At step 7 deactivation of Communication Forwarding on Not logged-in is respectively triggered at the UE.

### 15.10.5	Test requirements

1. SS shall check that the UE can authenticate itself correctly with the authentication scheme that the UE supports:

-	HTTP Digest authentication (see Annex C.29.1 step 2 NOTE 1) or

-	GAA based authentication as specified in TS 33.222 [121] and TS 24.109 [119] (see Annex C.29.2).

2. SS shall check that after Annex C.29.1 step 6 the simservs document stored in the SS contains the following pieces of information supplied by the UE:

-	<communication-diversion> element with "active" attribute set as "true" or with “active” attribute not present.

-	within <cp:ruleset> one <cp:rule> element for communication forwarding as follows:

-	<cp:conditions> element containing a <not-registered> element and not containing a <rule-deactivated> element

-	<cp:actions> element containing <forward-to> element containing <target> element

-	value of target address to be px_XCAP_TargetUri

3. SS shall check that after step 9 the simservs document stored in the SS contains the following pieces of information supplied by the UE:

-	<communication-diversion> element with "active" attribute being set "false"

or

-	<communication-diversion> element with "active" attribute being set "true" or with “active” attribute not present.

-	within <cp:ruleset> one <cp:rule>  element found at step 2 for Communication Diversion as follows:

-	<cp:conditions> element containing a <rule-deactivated> element

<End of modifed section>