3GPP TSG-RAN5 Meeting # 91-e	R5-213176

# Electronic Meeting,  17th May - 28th May

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1936 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex C: Clarifications to maximum and minimum offsetRBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | OffsetRBs,max and OffsetRBs,min in current specifications could be understood as absolute max or absolute min in column Offset (RBs) in Tables 13-1 to 13-10 in TS 38.213 for the given band and {SCSSSB, SCSCarrier} combination of the carrier. However, these maximum and minimum values are limited to a set of table indexes with number of RBs equal to the minimum value of in the table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Required clarifications added to the definition of OffsetRBs,max and OffsetRBs,min in table C.1-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Calculation of test frequencies procedure could be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This clarifition does not imply any change to test frequencies defined in current version of the specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

Annex C (informative): Calculation of test frequencies

# C.0	General

Test frequencies are defined in clause 4.3.1 with extensions for signalling test cases in clause 6.2.3. This annex gives a guideline to determine these test frequencies and the associated signalling parameters for a given NR band, NR CA or NR DC band combination.

Clause C.1 describes definitions and parameters used by the procedures to determine test frequencies, SS/PBCH Block (SSB) and CORESET#0 configuration parameters.

Clause C.2 describes how to calculate test frequencies for symmetric NR bands, asymmetric NR bands, NR CA and NR DC configurations.

Clause C.3.2 describes how to determine the SSB, CORESET#0 and signalling parameters for a PCell.

Clause C.3.3 describes how to determine the SSB and signalling parameters for a Cell without CORESET#0.

# C.1	Definitions and Parameters

Figure C.1-1 shows SSB and CORESET#0 and related parameters.

Figure C.1-1: location of SSB and CORESET#0 within a channel

The parameters referenced in figure C.1-1 are defined in Table C.1-1.

Table C.1-1: Definition of parameters in Figure C.1-1 used in Annex C.

| Parameter | Description |
| --- | --- |
| FPointA | Reference Point A frequency. |
| Fcarrier | Fcarrier is the centre frequency of a carrier corresponding to its NR-ARFCN value. |
| FcarrierLow | FcarrierLow is the centre frequency of lowest subcarrier of the carrier. FcarrierLow = Fcarrier - 12 * SCSCarrier * (NRB / 2) with NRB according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8] for the channel bandwidth of the carrier. |
| ΔFcarrierBandwidth | ΔFcarrierBandwidth is the carrier’s channel bandwidth as provided in carrierBandwidth to the UE (SCS-SpecificCarrier). |
| ΔFoffsetToCarrier | ΔFoffsetToCarrier is the frequency offset between Point A and the lower edge of the carrier. FoffsetToCarrier = offsetToCarrier * PRB size, where PRB size according to the subcarrier spacing of the carrier. offsetToCarrier is signalled to the UE (SCS-SpecificCarrier). |
| FSSref | Centre frequency of SSB. For a cell selectable as PCell the FSSref corresponds to a valid GSCN value according to clause 5.4.3.1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| ΔFOffset(RBs) | ΔFOffset(RBs) = 12* Offset(RBs) * subCarrierSpacingCommon, where Offset(RBs) is given in tables 13-1 to 13-10 of TS 38.213 [22]. |
| ΔFkSSB | ΔFKSSB = kSSB * subcarrier spacing of SSB (SCSSSB). |
| ΔFOffsetSSB-CORESET0 | Frequency offset between the lowest subcarrier of the SSB and the lowest subcarrier of CORESET#0. ΔFOffsetSSB-CORESET0 = ΔFOffset(RBs) + ΔFkSSB. |
| ΔFOffsetCORESET-0-Carrier | Frequency offset, FOffsetCORESET#0-Carrier, between the lowest subcarrier of CORESET#0 and the lowest subcarrier of the carrier. |
| ΔFOffsetSSB-Carrier | Frequency offset between the lowest subcarrier of the SSB and the lowest subcarrier of the carrier. |
| FCORESET0Low | Centre frequency of subcarrier 0 of CORESET#0. |
| FOffsetToPointA | Frequency of the lowest subcarrier of the lowest resource block, which has the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon and overlaps with the SS/PBCH block used by the UE for initial cell selection, expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2 (TS 38.211 [29] clause 4.4.4.2). |
| ΔFOffsetToPointA | Frequency offset between FOffsetToPointA and point A. ΔFOffsetToPointA = offsetToPointA * {15 kHz for FR1; 60 kHz for FR2} (TS 38.211 [29] clause 4.4.4.2). |

Additional parameters used in this annex are defined in Table C.1-2.

Table C.1-2: Definition of additional parameters used in Annex C.

| kSSB | as defined in TS 38.211 [29] clause 7.4.3.1 |
| --- | --- |
| SCSCarrier | subcarrier spacing for the carrier (SCS-SpecificCarrier): FR1: 15kHz, 30kHz or 60kHz according to TS 38.101-1 [7] Table 5.3.5-1 FR2: 60kHz or 120kHz according to TS 38.101-2 [8] Table 5.3.5-1 |
| SCSSSB | SS/PBCH block subcarrier spacing FR1: 15kHz or 30kHz according to TS 38.101-1 [7] Table 5.4.3.3-1 FR2: 120kHz or 240kHz according to TS 38.101-2 [8] Table 5.4.3.3-1 NOTE: According to the tables in clause 13 of TS 38.213 [22] not all combinations of SCSSSB and SCSCarrier are applicable. |
| SCScommon | Subcarrier spacing for SIB1, Msg.2/4 for initial access, paging and broadcast SI-messages. Provided to the UE in the MIB in IE subCarrierSpacingCommon. |
| PRBsize | Physical Resource Block size of the carrier = 12 * SCSCarrier. |
| CRBsize | Common Resource Block size = 12 * SCScommon. |
| FDL_Low, FUL_Low | Lowest frequency of the downlink and uplink frequency range of the band as defined in clause 5.2 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| FDL_High, FUL_High | Highest frequency of the downlink and uplink frequency range of the band as defined in clause 5.2 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| ΔFRaster | Frequency raster of the band as defined in clause 5.4.2.3 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| BWDL | Bandwidth of downlink frequency range of the band. |
| BWUL | Bandwidth of uplink frequency range of the band. |
| CBWDL | Downlink channel bandwidth (MHz) of the carrier according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| CBWUL | Uplink channel bandwidth (MHz) of the carrier according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| FTx-Rx_separation | Default Tx – Rx carrier centre frequency separation of the band as defined in clause 5.4.4 of TS 38.101-1 [7]. For TDD bands FTx-Rx_separation = 0. |
| ΔFTx-Rx_separation | ΔFTx-Rx = ¦ (BWDL – BWUL)/2 ¦ is the deviation to the default Tx-Rx carrier centre frequency separation (FTx-Rx_separation) for FDD FR1 bands supporting asymmetric channel bandwidths as defined in clause 5.3.6 of TS 38.101-1 [7]. |
| BWSSB | Bandwidth of the SSB. BWSSB = 12 * SCSSSB * 20 |
| ΔGSCN, GSCNMIN, GSCNMAX | GSCN step size, GSCN minimum and GSCN maximum values for the NR band according to table 5.4.3.3-1 of TS 38.101-1 [7] and TS 38.101-2 [8] |
| OffsetRBs | Offset (RBs) according to the applicable table 13-1 to 13-10 in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier. |
| OffsetRBs,max | Maximum value for Offset (RBs) according to the applicable table 13-1 to 13-1o in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier limited to the table indexes with number of RBs equal to the minimum value of in the table. |
| OffsetRBs,min | Minimum value for Offset (RBs) according to the applicable table 13-1 to 13-1o in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier limited to the table indexes with number of RBs equal to the minimum value of in the table. |

<< End of change>>