3GPP TSG-RAN5 Meeting #91-e	R5-213183

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1205 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extension of Annex I for EIS spherical coverage points |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Annex I of TS 38.533 only contains information about how to find the Rx Beam Peak, but not how to find points fulfilling EIS SC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extended Annex I to refer to the RF test specification to find EIS SC Added the information for the Rx BPS for enhanced IFF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no information in TS 38.533 how to obtain the test direction needed for Setup 2, 3 and 4 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | I |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

Annex I (normative):
RRM OTA procedures

Annex I defines the RRM OTA procedures for the permitted testing methodologies defined in [32].

# I.0	Test applicability per permitted test method

The applicability of each permitted test method for the different RRM Angle of Arrival (AoA) setups as defined in Section A.9 is defined in Table I.0-1.

Table I.0-1 AoA Test Setup applicability per permitted test method

| AoA Test Setup | No DUT antenna configuration declaration | DUT antenna configuration declaration |  |  |
| --- | --- | --- | --- | --- |
|  |  | Configuration 1 (one antenna panel with D ≤ 5 cm active at any one time) | Configuration 2 (More than one antenna panel D ≤ 5 cm without phase coherency between panels active at any one time) | Configuration 3 (Any phase coherent antenna panel of any size) |
| Setup 1 | IFF, Enhanced IFF | DFF, Simplified DFF, IFF, Enhanced IFF | DFF, Simplified DFF, IFF, Enhanced IFF | IFF, Enhanced IFF |
| Setup 2a | IFF, Enhanced IFF | DFF, Simplified DFF, IFF, Enhanced IFF | DFF, Simplified DFF, IFF, Enhanced IFF | IFF, Enhanced IFF |
| Setup 2b | IFF, Enhanced IFF | DFF, Simplified DFF, IFF, Enhanced IFF | DFF, Simplified DFF, IFF, Enhanced IFF | IFF, Enhanced IFF |
| Setup 3 | IFF, Enhanced IFF | DFF, IFF, Enhanced IFF, IFF+DFF | DFF, IFF, Enhanced IFF, IFF+DFF | IFF, Enhanced IFF |
| Setup 4 | IFF, Enhanced IFF | DFF, IFF, Enhanced IFF, IFF+DFF | DFF, IFF, Enhanced IFF, IFF+DFF | IFF, Enhanced IFF |
| NOTE: D = DUT radiating aperture declared by UE vendor. |  |  |  |  |

# I.1	Direct far field (DFF)

## I.1.1	RX beam peak direction search

The Rx beam peak direction search can be found using any of the following options:

-	Same measurement procedure as in clause Annex K.1.2 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

## I.1.2	Search for directions corresponding to the EIS spherical coverage percentile

The test directions corresponding to the EIS spherical coverage percentile can be found using any of the following options:

-	EIS spherical coverage scan procedure as described in Annex K.1.6 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

# I.2	Direct far field (DFF) simplification

## I.2.1	RX beam peak direction search

The Rx beam peak direction search can be found using any of the following options:

-	Same measurement procedure as in clause Annex K.2.2 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

## I.2.2	Search for directions corresponding to the EIS spherical coverage percentile

The test directions corresponding to the EIS spherical coverage percentile can be found using any of the following options:

-	EIS spherical coverage scan procedure as described in Annex K.1.6 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

# I.3	Indirect far field (IFF)

## I.3.1	RX beam peak direction search

The Rx beam peak direction search can be found using any of the following options:

-	Same measurement procedure as in clause Annex K.3.2 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

## I.3.2	Search for directions corresponding to the EIS spherical coverage percentile

The test directions corresponding to the EIS spherical coverage percentile can be found using any of the following options:

-	EIS spherical coverage scan procedure as described in Annex K.1.6 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

# I.3A	Enhanced indirect far field (Enhanced IFF)

## I.3A.1	RX beam peak direction search

Same as in clause I.3.1 for IFF.

## I.3A.2	Search for directions corresponding to the EIS spherical coverage percentile

Same as in clause I.3.2 for IFF.

< End of changes >