3GPP TSG-RAN5 Meeting #91-e	R5-213196

# Electronic Meeting, 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0159 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test cases 7.29 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to RFC4028 section 7-2: "If the 2xx response did not contain a Session-Expires header field, there is no session expiration. In this case, no refreshes need to be sent. A 2xx without a Session-Expires can come for both initial and subsequent session refresh requests. This means that the session timer can be 'turned-off' in mid dialog by receiving a response without a Session-Expires header field." NG.114 clause 2.2.9 does not have any related requirements either. In TP3 and TP4 it is expected that the UE sends the update which is not in lie with the RFC as the UE may or may not send a refresh. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Incorrect TP3 and TP4 and related test steps have been removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.29 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.29	Session Timer / MO Call / Remote end does not support Session Timer / 5GS

7.29.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and being configured to use Session Timer and preconditions and to be the refresher }

ensure that {

when { UE is being made to initiate the voice call }

then { UE sends INVITE for voice call }

}

(2)

with { UE having sent INVITE and continuing with the call setup }

ensure that {

when { UE receives 200 OK for INVITE without timer tag and Session-Expires }

then { UE sends ACK }

}

7.29.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.2A.1.1]

A UE supporting RFC 4028 [58], when it receives a 422 (Session Interval Too Small) to an INVITE request where the response contains a Min-SE header field, shall retry the request in accordance with RFC 4028 [58] subclause 7.4.

[TS 24.229 clause 5.2.7.2]

When the P-CSCF receives from the UE an INVITE request, the P-CSCF may require the periodic refreshment of the session to avoid hung states in the P-CSCF. If the P-CSCF requires the session to be refreshed, then the P-CSCF shall apply the procedures described in RFC 4028 [58] clause 8.

NOTE 1:	Requesting the session to be refreshed requires support by at least one of the UEs. This functionality cannot automatically be granted, i.e. at least one of the involved UEs needs to support it.

[TS 24.229 clause 5.2.7.3]

When the P-CSCF receives an INVITE request destined for the UE the P-CSCF may require the periodic refreshment of the session to avoid hung states in the P-CSCF. If the P-CSCF requires the session to be refreshed, then the P-CSCF shall apply the procedures described in RFC 4028 [58] clause 8.

NOTE 1:	Requesting the session to be refreshed requires support by at least one of the UEs. This functionality cannot automatically be granted, i.e. at least one of the involved UEs needs to support it in order to make it work.

[TS 24.229 clause 5.4.5.3]

If the S-CSCF requested the session to be refreshed periodically, and the S-CSCF got the indication that the session will be refreshed, when the session timer expires, the S-CSCF shall delete all the stored information related to the dialog.

7.29.3	Test description

7.29.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	UE is configured to use Session Timer and preconditions.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

7.29.3.2	Test procedure sequence

Table 7.29.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to attempt an IMS voice call. | - | - |  |  |
| 2-7 | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - |  |  |
| 8 | UE sends INVITE indicating support for Session Timer, with either the Session-Expires value set to 1800 or no Session-Expires header. | --> | INVITE | 1 | P |
| 9-17 | Steps 2-10 of Annex A.4.1 happen. | - | - |  |  |
| 18 | SS sends 200 OK for INVITE, without timer tag in Supported and Require headers and without Session-Expires header. | <-- | 200 OK |  |  |
| 19 | UE sends ACK. | --> | ACK | 2 | P |
| 20-23 | Void |  |  |  |  |
| 24-25 | SS releases the call. (Steps 1-2 of Annex A.8) | - | - |  |  |

7.29.3.3	Specific message contents

Table 7.29.3.3-1: INVITE (step 8, table 7.29.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.2.1, Conditions A1, A3, A4, A26 and A28 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Session-Expires |  | (if present) |  | RFC 4028 [37] |
| delta-seconds |  | 1800 |  |  |
| refresher |  | uac (if present) |  |  |

Table 7.29.3.3-2: 183 Session Progress (step 10, table 7.29.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.2.3, Condition A1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Allow |  | INVITE, UPDATE, PRACK, ACK, OPTIONS, CANCEL, BYE |  | RFC 4028 [37] |

Table 7.29.3.3-3: 200 OK (step 18, table 7.29.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.3.1, Conditions A1 and A10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Allow |  | INVITE, UPDATE, PRACK, ACK, OPTIONS, CANCEL, BYE |  | RFC 4028 [37] |

Table 7.29.3.3-4: Void