3GPP TSG-RAN5 Meeting #91-e	R5-213201

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1942 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update maximum number of simultaneous configured cells for FR1 and FR2 in OTA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In OTA, simultaneous multiple cells can be configured FR1 or FR2 or E-UTRA. Therefore, maximum number of cells in OTA can be defined. In FR1, OTA calibration procedure is not defined. However, this does not restrict to defined multiple cells being configured and also, multiple cells can be active if measurements are not needed. Hence, 4 Cells maximum can be allowed. However, in case of measurements needed ( in IDLE or CONNECTED) for a test case, test case author needs ensure only one cell is active( even though multiple cells are configured) at a given point of time. In EUTRA OTA, similar to FR1, even though OTA Calibration is not needed, this does not restrict to configure maximum of 2 cells simulateneously. In FR2, Maximum 4 cells can be configured and simulateneously active if t the measurement are not needed. For example, Test cases like 8.2.4.2.1.x(EN-DC) or 8.1.4.18.x(NR-CA) are designed with maximum 4 cells. Since OTA calibration procedure and uncertianities are defined, therefore, test case author can carefully design with 4 active cells depending upon scenario of the test as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Maximum numbers of NR Cells in OTA set to 4 for both FR1 and FR2. Set to 2 in case of E-UTRA OTA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OTA environment will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

## 6.1	Requirements of test equipment

### 6.1.1	Requirements common for conducted and OTA tests

The requirements of test equipment specified in this subclause apply to Signalling test cases defined in TS 38.523-1 [12], in addition to the common requirements of test equipment specified in clause 4.2 of this specification.

Test equipment shall be able to simulate cells of Radio Access Technologies NR and E-UTRA, The number of cells to be simulated simultaneously by the test equipment shall not exceed the resources specified in Table 6.1-1

Table 6.1-1: Maximum resources in terms of number / configuration of cells to be simulated simultaneously in a test setup

| Simulation of |  | Max. number of cells (NR) |  |  |  |  |  | Max. number of cells (E-UTRA) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Conducted |  | OTA(FR1) |  | OTA(FR2) |  | Conducted |  | OTA |
| NR single-mode networks (FDD or TDD) |  |  | 4 cells |  | 4 cells |  | 4 cells |  | n/a |  | n/a |
| NR dual-mode networks (FDD and TDD) |  |  | 4 cells |  | 4 cells |  | 4 cells |  | n/a |  | n/a |
| NR networks involving Carrier Aggregation |  |  | 4 cells |  | 4 cells |  | 4 cells |  | n/a |  | n/a |
| NR dual connectivity (NR-DC) |  |  | 4 cells |  | 4 cells |  | 4 cells |  | n/a |  | n/a |
| NR dual connectivity (EN-DC) |  |  | 4 cells |  | 4 cells |  | 4 cells |  | 2 cells |  | 2 cells |
| NR dual connectivity (EN-DC) involving Carrier Aggregation |  |  | 4 cells |  | 4 cells |  | 4 cells |  | 2 cells |  | 2 cells |
| Mixed E-UTRA / NR networks |  |  | 4 cells |  | 4 cells |  | 4 cells |  | 2 cells |  | 2 cells |
|  | Note 1: No differentiation between cell configuration types (as defined in clause 6.3.1) here, because these types are only relevant to specific test cases and their TTCN-3 implementation. Note 2: Only network scenarios specified in clauses 4.4.1 and 6.3.2.1 have been covered. Note 3: In case of Carrier Aggregation, each cell can act as a SpCell, an SCell, or a standalone cell (not used as a CA component carrier). Note 4: In order to support test case requirements for conducted and OTA test methods, the number of active cells at any given time should be minimised in order to ensure maximum re use of SS Tx/Rx resources. |  |  |  |  |  |  |  |  |  |

Exceptions to the requirements outlined above are possible but need special evidence to be provided explicitly in the test case prose and should be allowed only if the test case purpose cannot be met otherwise.

Due to limited power level range for FR2 OTA test methods, when defining test cases requirements, care shall be taken to ensure that the number of active cells is minimised as this has an impact to have distinguishable power level difference. Cells that are used in initial parts of test cases and are no longer required for the rest of the procedure shall be clearly defined as Non-suitable "Off" cell to facilitate re use of SS Tx/Rx resources

<End of Change>