3GPP TSG-RAN5 Meeting #91-e                      	R5-213238

# Electronic Meeting, 17th May – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1948 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test frequencies for n261 for RRM Inter-freq adjacent cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test frequencies for Inter-frequency adjacent cells for band n261 need to be added to 7.2.3.2.5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adjacent inter-frequency cell added to tables 7.2.3.2.5-1 and 7.2.3.2.5-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test frequencies specified for Inter-frequency adjacent cells for band n261 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.508-1 CR 1949. |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

//Start of changes

7.2.3.2.5	Reference test frequencies for NR operating band n261

Table 7.2.3.2.5-1: Test frequencies for NR operating band n261 (SCS 120 kHz, ΔFRaster 120 kHz SSB SCS=120kHz, kSSB=0 and Offset(RBs)=0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 66 | Downlink & Uplink | Mid | 27929.28 | 2077987 | 27881.76 | 2077195 | 0 | 120 | 22467 | 2077435 | 0 | 0 | 0 (0) | 0 |
| 100 | 66 | Downlink & Uplink | Adjacent inter-frequency cell | 28032.96 | 2079715 | 27985.44 | 2078923 | 0 | 120 | 22473 | 2079163 | 0 | 0 | 0 (0) | 0 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.2.3.2.5-2: Test frequencies for NR operating band n261 (SCS 120 kHz, ΔFRaster 120 kHz, SSB SCS=240kHz, kSSB=0 and Offset(RBs)=0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 66 | Downlink & Uplink | Mid | 27932.16 | 2078035 | 27884.64 | 2077243 | 0 | 240 | 22468 | 2077723 | 0 | 0 | 2 (0) | 0 |
| 100 | 66 | Downlink & Uplink | Adjacent inter-frequency cell | 28035.84 | 2079763 | 27988.32 | 2078971 | 0 | 240 | 22474 | 2079451 | 0 | 0 | 0 (0) | 0 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-10 in TS 38.213 [22]. The value of CORESET#0 Index is signalled controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |