3GPP TSG-RAN5 Meeting #91-e	R5-213256

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0016 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new MCVideo test case 6.1.1.12 On-network / On-demand Pre-arranged Group Call / Transmission Control State Transitions / Client Originated (CO) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of a new test case |  |  |  |  |  |  |  |  |
| Summary of change: |  | Created new MCVideo test case 6.1.1.12 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MCVideo client will not be properly tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <START OF CHANGES>

6.1.1.12	On-network / On-demand Pre-arranged Group Call / Transmission Control State Transitions / Client Originated (CO)

6.1.1.12.1	Test Purpose (TP)

(1)

with { the UE (MCVideo Client) registered and authorised for MCVideo Service }

ensure that {

when { the MCVideo User requests the establishment of an MCVideo On-demand Pre-arranged Group Call forcing Automatic Commencement Mode and implicit Transmission Control }

then { the UE (MCVideo Client) requests an On-demand Automatic Commencement Mode Pre-arranged Group Call establishment with implicit Transmission Control by sending a SIP INVITE message, and, after indication from the SS (MCVideo Server) that the call was established, provides transmission granted notification to the MCVideo User }

}

(2)

with { the UE (MCVideo Client) having established a MCVideo On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { the MCVideo User engages in communication with the invited MCVideo User(s) and requests to terminate the ongoing MCVideo Group Call }

then { the UE (MCVi Media Reception Notification, deo Client) respects the Transmission Control imposed by the SS(MCVideo Server) (Transmission Granted, Media Reception Notification, Transmission Revoked, Queue Position Request, Queue Position Info, Transmission Control ACK, Transmission End Request, Transmission End Response, Transmission Request, Transmission Rejected, Transmission Cancel Request Notify }

}

(3)

with { the UE (MCVideo Client) having an ongoing On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { the MCVideo User requests to end the call }

then { the UE (MCVideo Client sends SIP BYE message, and leaves the MCVideo Session }

}

6.1.1.12.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.281, clauses 6.2.3.1.2, 6.2.4.1, 9.2.1.2.1.1, TS 24.581, clauses 6.2.4.2.2, 6.2.4.3.2, 6.2.4.4.2, 6.2.4.4.5, 6.2.4.4.6, 6.2.4.5.5, 6.2.4.5.6, 6.2.4.5.7, 6.2.4.6.4, 6.2.4.8.2, 6.2.4.9.2, 6.2.4.9.4, 6.2.4.9.6. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-15 requirements.

[TS 24.281, clause 6.2.3.1.2]

When performing the automatic commencement mode procedures, the MCVideo client shall follow the procedures in subclause 6.2.3.1.1 with the following clarification:

-	The MCVideo client may include a P-Answer-State header field with the value "Confirmed" as specified in IETF RFC 4964 [30] in the SIP 200 (OK) response.

[TS 24.281, clause 6.2.4.1]

Upon receiving a request from an MCVideo user to leave an MCVideo session established using on-demand session signalling, the MCVideo client:

1)	shall interact with the media plane as specified in 3GPP TS 24.581 [5];

2)	shall generate a SIP BYE request according to 3GPP TS 24.229 [11];

3)	shall set the Request-URI to the MCVideo session identity to leave; and

4)	shall send a SIP BYE request towards MCVideo server according to 3GPP TS 24.229 [11].

Upon receiving a SIP 200 (OK) response to the SIP BYE request, the MCVideo client shall interact with the media plane as specified in 3GPP TS 24.581 [5].

[TS 24.281, clause 9.2.1.2.1.1]

Upon receiving a request from an MCVideo user to establish an MCVideo prearranged group session the MCVideo client shall generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [11], with the clarifications given below.

The MCVideo client:

1)	if the MCVideo user has requested the origination of an MCVideo emergency group call or is originating an MCVideo prearranged group call and the MCVideo emergency state is already set, the MCVideo client shall comply with the procedures in subclause 6.2.8.1.1;

2)	if the MCVideo user has requested the origination of an MCVideo imminent peril group call, the MCVideo client shall comply with the procedures in subclause 6.2.8.1.9;

3)	if the MCVideo user has requested the origination of a broadcast group call, the MCVideo client shall comply with the procedures in subclause 6.2.8.2;

4)	shall include the g.3gpp.mcvideo media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcvideo" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [22];

5)	shall include an Accept-Contact header field containing the g.3gpp.mcvideo media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [20];

6)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcvideo" (coded as specified in 3GPP TS 24.229 [11]), in a P-Preferred-Service header field according to IETF RFC 6050 [14] in the SIP INVITE request;

7)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcvideo" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [20];

8)	should include the "timer" option tag in the Supported header field;

9)	should include the Session-Expires header field according to IETF RFC 4028 [23]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

10)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCVideo function serving the MCVideo user;

NOTE 1:	The MCVideo client is configured with public service identity identifying the participating MCVideo function serving the MCVideo user.

11)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [11];

12)	if the MCVideo emergency state is already set or the MCVideo client emergency group state for this group is set to "MVEG 2: in-progress", the MCVideo client shall include the Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.2;

13)	if the MCVideo client imminent peril group state for this group is set to "MVIG 2: in-progress" or "MVIG 4: confirm-pending" shall include the Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.12;

14)	shall contain in an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with:

a)	the <session-type> element set to a value of "prearranged";

b)	the <mcvideo-request-uri> element set to the group identity;

c)	the <mcvideo-client-id> element set to the MCVideo client ID of the originating MCVideo client; and

NOTE 2:	The MCVideo client does not include the MCVideo ID of the originating MCVideo user in the body, as this will be inserted into the body of the SIP INVITE request that is sent from the originating participating MCVideo function.

d)	if the group identity can be determined to be a TGI and if the MCVideo client can associate the TGI with a MCVideo group ID, the <associated-group-id> element set to the MCVideo group ID;

NOTE 3:	The text "can associate the TGI with a MCVideo group ID" means that the MCVideo client is able to determine that there is a constituent group of the temporary group that it is a member of.

NOTE 4:	The MCVideo client is informed about temporary groups and regouping of MCVideo groups that the user is a member of as specified in 3GPP TS 24.481 [24].

NOTE 5:	If the MCVideo user selected a TGI where there are several MCVideo groups where the MCVideo user is a member, the MCVideo client selects one of those MCVideo groups.

15)	shall include an SDP offer according to 3GPP TS 24.229 [11] with the clarifications given in subclause 6.2.1;

16)	if an implicit transmission request is required, shall indicate this as specified in subclause 6.4; and

17)	shall send the SIP INVITE request towards the MCVideo server according to 3GPP TS 24.229 [11].

On receiving a SIP 2xx response to the SIP INVITE request, the MCVideo client:

1)	shall interact with the user plane as specified in 3GPP TS 24.581 [5]

2)	if the MCVideo emergency group call state is set to "MVEGC 2: emergency-call-requested" or "MVEGC 3: emergency-call-granted" or the MCVideo imminent peril group call state is set to "MVIGC 2: imminent-peril-call-requested" or "MVIGC 3: imminent-peril-call-granted", the MCVideo client shall perform the actions specified in subclause 6.2.8.1.4; and

3)	may subscribe to the conference event package as specified in subclause 9.1.3.1.

On receiving a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request:

1)	if the MCVideo emergency group call state is set to "MVEGC 2: emergency-call-requested" or "MVEGC 3: emergency-call-granted"; or

2)	if the MCVideo imminent peril group call state is set to "MVIGC 2: imminent-peril-call-requested" or "MVIGC 3: imminent-peril-call-granted";

the MCVideo client shall perform the actions specified in subclause 6.2.8.1.5.

On receiving a SIP INFO request where the Request-URI contains an MCVideo session ID identifying an ongoing group session, the MCVideo client shall follow the actions specified in subclause 6.2.8.1.13

[TS 24.581, clause 6.2.4.2.2]

When a call is initiated as described in 3GPP TS 24.281 [2], the transmission participant:

1.	shall create an instance of the 'Transmission participant state transition diagram for basic transmission control operation';

2.	if the originating transmission participant receives a transmission control message before it receives the SIP 200 (OK) response, shall store the transmission control message;

NOTE:	The originating transmission participant might receive a transmission control message before the SIP 200 (OK) response when initiating, joining or rejoining a call because of processing delays of the SIP 200 (OK) response in the SIP core.

3.	if the established MCVideo call is a chat group call and the SIP INVITE request is not an implicit Transmission request, shall enter the 'U: has no permission to transmit' state;

4.	if for the established MCVideo call the SIP INVITE request is an implicit Transmission request:

a.	shall start timer T100 (Transmission Request) and initialise counter C100 (Transmission Request) to 1;

b.	shall enter the 'U: pending request to transmit' state; and

c.	if the transmission participant has received and stored a transmission control message before the reception of the SIP 200 (OK) response, shall act as if the transmission control message was received in the 'U: pending request to transmit' state after entering the 'U: pending request to transmit' state; and

5.	if the established MCVideo call is a broadcast group call, shall enter the 'U: has permission to transmit' state.

When the transmission participant is rejoining an ongoing MCVideo call as described in 3GPP TS 24.281 [2] the transmission participant shall enter the 'U: has no permission to transmit' state.

[TS 24.581, clause 6.2.4.3.2]

Upon receiving an indication from the user to request permission to send media, the transmission participant:

1.	void

2.	shall send the Transmission Request message toward the transmission control server; The Transmission Request message:

a.	if a different priority than the normal priority is required, shall include the Transmission Priority field with the priority not higher than negotiated with the transmission control server as specified in subclause 14.3.3; and

b.	if the Transmission request is a broadcast group call, system call, emergency call or an imminent peril call, shall include a Transmission Indicator field indicating the relevant call types;

3.	shall start timer T100 (Transmission Request) and initialise counter C100 (Transmission Request) to 1; and

4.	shall enter the 'U: pending request to transmit' state.

[TS 24.581, clause 6.2.4.4.2]

Upon receiving a Transmission rejected message, the transmission participant:

1.	if the first bit in the subtype of the Transmission rejected message is set to '1' (Acknowledgment is required) as described in subclause 9.2.2.1, shall send a Transmission control Ack message. The Transmission control Ack message:

a.	shall include the Message Type field set to '1' (Transmission rejected); and

b.	shall include the Source field set to '0' (the transmission participant is the source);

2.	shall provide Transmission rejected notification to the user;

3.	may display the Transmission rejected reason to the user using information in the Reject Cause field;

4.	shall stop timer T100 ( Transmission Request); and

5.	shall enter the 'U: has no permission to transmit' state.

[TS 24.581, clause 6.2.4.4.5]

Upon receiving a Queue Position Info message, the transmission participant:

1.	if the first bit in the subtype of the Queue Position Info message is set to '1' (Acknowledgment is required) as described in subclause 9.2.2.1, shall send a Transmission control Ack message. The Transmission control Ack message:

a.	shall include the Message Type field set to '5' (Queue Position Info); and

b.	shall include the Source field set to '0' (the transmission participant is the source);

2.	shall provide Transmission request queued notification to the MCVideo user;

3.	may provide the queue position and priority to the MCVideo user; and

4.	shall enter the 'U: queued transmission' state.

[TS 24.581, clause 6.2.4.4.6]

Upon receiving a Transmission Granted message from the transmission control server, the transmission participant:

1.	if the first bit in the subtype of the Transmission Granted message is set to '1' (Acknowledgment is required) as described in subclause 9.2.2.1, shall send a Transmission control Ack message. The Transmission control Ack message:

a.	shall include the Message Type field set to '0' (Transmission Granted); and

b.	shall include the Source field set to '0' (the transmission participant is the source);

2.	shall store the SSRC of granted transmission participant received in the Transmission Granted message and use it in the RTP media packets until the transmission is relased;

3.	shall provide Transmission granted notification to the user, if not already done;

4.	shall stop timer T100 (Transmission Request); and

5.	shall enter the 'U: has permission to transmit' state.

[TS 24.581, clause 6.2.4.5.5]

Upon receiving a Transmission Revoked message, the transmission participant:

1.	shall inform the user that the permission to send RTP media is being revoked;

2.	may give information to the user about the reason for revoking the permission to send media:

3.	shall request the media in the MCVideo client discard any remaining buffered RTP media packets and to stop forwarding of encoded video to the MCVideo server; and

4.	if the revoke reason is:

a.	terminate the RTP stream, shall enter the 'U: pending end of transmission' state:

i.	shall send a Transmission end request message towards the transmission control server; and

ii.	shall start timer T101 (Transmission End Request) and initialize counter C101 (Transmission End Request) to 1; or

b.	queue the transmission, shall enter the 'U: queued transmission' state:

i.	shall send a Queue Position Request message towards the transmission control server; and

ii.	shall start timer T102 (Transmission Queue Position Request) and initialize counter C102 (Queue Position Request) to 1.

[TS 24.581, clause 6.2.4.5.6]

Upon receiving a Media Reception notification message, the transmission participant:

1.	shall inform the user about the media reception by another user; and

2.	shall remain in the 'U: has permission to transmit' state.

[TS 24.581, clause 6.2.4.5.7]

Upon receiving a Transmission End Request message from transmission control server, the transmission participant:

1.	shall inform the user that the permission to send RTP media is being revoked;

2.	may give information to the user about the reason for terminating the permission to send media;

3.	shall request the media in the MCVideo client to discard any remaining buffered RTP media packets and to stop forwarding of encoded video to the MCVideo server; and

4.  shall send  Transmission End Response message to the transmission control server.

5.	if the session is not a broadcast group call or if the A-bit in the Transmission Indicator field is set to '1' (Normal call), shall enter the 'U: has no permission to transmit' state; and

6.	if the session was initiated as a broadcast group call:

a.	shall indicate to the MCVideo client the media transmission is completed; and

b	shall enter the 'Call releasing' state.

[TS 24.581, clause 6.2.4.6.4]

Upon receiving a Transmission end response message, the transmission participant:

1.	if the first bit in the subtype of the Transmission end response message to '1' (Acknowledgment is required) as described in subclause 9.2.2.1, shall send a Transmission control Ack message. The Transmission control Ack message:

a.	shall include the Message Type field set to '1' (Transmission end response); and

b.	shall include the Source field set to '0' (the transmission participant is the source);

2.	may provide a Transmission end notification to the MCVideo user;

3.	if the Transmission Indicator field is included and the B-bit set to '1' (Broadcast group call), shall provide a notification to the user indicating the type of call;

4.	shall stop timer T101 (Transmission End Request);

5.	if the session is not a broadcast group call or if the A-bit in the Transmission Indicator field is set to '1' (Normal call), shall enter the 'U: has no permission to transmit' state; and

6.	if the session was initiated as a broadcast group call:

a.	shall indicate to the MCVideo client the media transmission is completed; and

b	shall enter the 'Call releasing' state.

[TS 24.581, clause 6.2.4.8.2]

Upon receiving an MCVideo call release step 2 request from the application and signalling, the transmission participant:

1.	shall release all resources including any running timers associated with the MCVideo call; and

2.	shall enter the 'Start-stop' state and terminate the current instance of the 'Transmission control state machine – basic'.

[TS 24.581, clause 6.2.4.9.2]

Upon receiving a Queue Position Info message, the transmission participant:

1.	if the first bit in the subtype of the Queue Position Info message is set to '1' (Acknowledgment is required) as described in subclause 9.2.2.1, shall send a Transmission control Ack message. The Transmission control Ack message:

a.	shall include the Message Type field set to '5' (Queue Position Info); and

b.	shall include the Source field set to '0' (the transmission participant is the source);

2.	if the message indicates that the request has been queued or if a request for the queue position was sent, the transmission participant:

a.	may provide the queue position and priority (if available) to the MCVideo user;

3.	shall stop the timer T102 (Transmission Queue Position Request), if running; and

4.	shall remain in the 'U: queued transmission' state.

[TS 24.581, clause 6.2.4.9.4]

Upon receipt of an indication from the MCVideo client to cancel the media transmit request from the queue, the transmission participant:

1.	shall send the Transmission end request message to the transmission control server. The Transmission end request message, if the session is a broadcast call and if the session was established as a normal call, shall include the Transmission Indicator with the A-bit set to '1' (Normal call);

2.	shall start timer T101 (Transmission End Request) and initialize counter C101 (Transmission End Request) to 1; and

3.	shall enter the 'U: pending end of transmission' state.

[TS 24.581, clause 6.2.4.9.6]

Upon receiving a Transmission cancel request notify message, the transmission participant:

1.	if the first bit in the subtype of the Transmission cancel request notify message is set to '1' (Acknowledgment is required) as described in subclause 9.2.2.1, shall send a Transmission control Ack message. The Transmission control Ack message:

a.	shall include the Message Type field set to '10' (Transmission cancel request notify); and

b.	shall include the Source field set to '0' (the transmission participant is the source);

2.	shall enter in the 'U: has no permission to transmit' state.

6.1.1.12.3	Test description

6.1.1.12.3.1	Pre-test conditions

System Simulator:

-	SS (MCVideo Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

UE:

-	UE (MCVideo Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCVideo UE registration as specified in TS 36.579-1 [2], subclause 5.4.2A.

-	The MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2A.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCVideo Client Application has been activated and User has registered-in as the MCVideo User with the Server as active user at the Client.

6.1.1.12.3.2	Test procedure sequence

Table 6.1.1.12.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCVideo User request the establishment of an MCVideo On-demand Pre-arranged Group Call using, Automatic Commencement Mode, with request for implicit Transmission Control. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCVIDEO call establishment; steps are described in TS 36.579-1 [2], subclause 5.4.3A 'Generic Test Procedure for MCVideo CO communication in E-UTRA'. The test sequence below shows only the MCVideo relevant messages exchanged. | - | - | - | - |
| 2 | Check: Does the UE (MCVideo Client) send an initial SIP INVITE requesting the establishment of an MCVideo On-demand Pre-arranged Group Call, Automatic Commencement Mode, with implicit Transmission Control? | --> | SIP INVITE | 1 | P |
| 3 | The SS (MCVideo Server) responds to the UE with a SIP 100 (Trying) message. | <-- | SIP 100 (Trying) | - | - |
| 4 | The SS (MCVideo Server) sends a SIP 200 (OK) message to the UE (MCVideo Client), indicating that it has accepted the SIP INVITE request from the UE. | <-- | SIP 200 (OK) | - | - |
| 5 | Check: Does the UE (MCVideo Client) send an acknowledgement to the SS (MCVideo Server)? | --> | SIP ACK | 1 | P |
| 6 | The SS (MCVideo Server) sends a Transmission Granted message to the UE (MCVideo Client) | <-- | Transmission Granted | - | - |
| 6a | Check: Does the UE (MCVideo Client) provide transmission granted notification to the MCVideo User? (NOTE1) | - | - | 2 | P |
| 7 | The SS (MCVideo Server) sends information about the media reception of another user. | <-- | Media Reception Notification | - | - |
| 7a | Check: Does the UE (MCVideo Client) inform the MCVideo User about the media reception from another user? (NOTE1) | - | - | 2 | P |
| 8 | The SS (MCVideo Server) is revoking permission to transmit with revoke reason queue the transmission | <-- | Transmission Revoked | - | - |
| 8a | Check: Does the UE (MCVideo Client) inform the MCVideo User that the permission to send RTP media is being revoked? (NOTE1) | - | - | 2 | P |
| 9 | Check: Does the UE (MCVideo Client) ask for queue information.? (NOTE1) | --> | Queue Position Request | 2 | P |
| 10 | The SS (MCVideo Server) sends the queue information to the Client, with the first bit set to ‘1’. | <-- | Queue Position Info | - | - |
| 11 | Check: Does the UE (MCVideo Client) acknowledges receipt of information about the queue position? | --> | Transmission Control Ack | 2 | P |
| 12- | Make the MCVideo User request to end the transmission request and be removed from the queue. (NOTE1) | - | - | - | - |
| 12a | Check: Does the UE (MCVideo Client) requests to cancel the media transmit request from the queue? | --> | Transmission End Request | 2 | P |
| 13 | The SS (MCVideo Server) responds to end transmission. | <-- | Transmission End Response |  |  |
| 14 | Make the MCVideo User request permission to send a video transmission. (NOTE 1) | - | - | - | - |
| 14a | Check: Does the UE (MCVideo Client) request permission to send a video transmission? | --> | Transmission Request | 2 | P |
| 15 | The SS (MCVideo Server) rejects the request. | <-- | Transmission Rejected | - | - |
| 15a | Check: Does the UE (MCVideo Client) inform the MCVideo User that the request to transmit was rejected? (NOTE 1) | - | - | 2 | P |
| 16 | Make the MCVideo User request permission to send a video transmission. (NOTE 1) | - | - | - | - |
| 16a | Check: Does the UE (MCVideo Client) request permission to send a video transmission? | --> | Transmission Request | 2 | P |
| 17 | The SS (MCVideo Server) has queued the UE (MCVideo Client). (NOTE 1) | <-- | Queue Position Info | - | - |
| 17a | Does the UE (MCVideo Client) provide Transmission request queued notification to the MCVideo User? (NOTE 1) | - | - | - | - |
| 18 | The SS (MCVideo Server) removes the UE (MCVideo Client) from the queue. | <-- | Transmission Cancel Request Notify | - | - |
| 19 | Make the MCVideo User request permission to send a video transmission. (NOTE 1) | - | - | - | - |
| 19a | Check: Does the UE (MCVideo Client) request permission to send a video transmission? | --> | Transmission Request | 2 | P |
| 20 | The SS (MCVideo Server) provides transmission granted notification to the MCVideo User. | <-- | Transmission Granted |  |  |
| 20a | Check: Does the UE (MCVideo Client) provide transmission granted notification to the MCVideo User? (NOTE1) | - | - | 2 | P |
| 21 | The SS (MCVideo Server) sends the Transmission end request. | <-- | Transmission End Request | - | - |
| 21a | Check Does the UE (MCVideo Client) inform the MCVideo User that the permission to send RTP media is being revoked? (NOTE 1) | - | - | 2 | P |
| 22 | Check: Does the UE (MCVideo Client) respond to the Transmission end request? | --> | Transmission End Response | 2 | P |
| 23 | Make the MCVideo User request to end the session. (NOTE 1) |  |  |  |  |
| 23a | Check: Does the UE (MCVideo Client) send a SIP BYE message to end the On-demand Pre-arranged Group Call? | --> | SIP BYE | 3 | P |
| 24 | The SS (MCVideo Server) sends a SIP 200 (OK) message to the UE (MCVideo Client) to indicate acceptance to end the Group Call. | <-- | SIP 200 (OK) | - | - |
| - | EXCEPTION: SS (MCVideo Server) releases the E-UTRA connection. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.1.12.3.3	Specific message contents

Table 6.1.1.12.3.3-1: SIP INVITE (Step 2, Table 6.1.1.12.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1, condition MCVIDEO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.1.1.1.3.3-2 |  |  |  |

Table 6.1.1.12.3.3-2: MCVideo-Info in SIP INVITE (Table 6.1.1.12.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-2, condition GROUP-CALL

Table 6.1.1.12.3.3-3: SIP 200 (OK) (Step 4, Table 6.1.1.12.3.2-1)

| Derivation Path: RFC 3261 [22], Table 5.5.2.17.1.2-1, Conditions MCVIDEO, INVITE-RSP, GROUP CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact |  |  |  | REGISTER-RSP |
| feature-param | "+g.3gpp.mcvideo" MCVIDEO |  |  |  |
| expires | "600000" |  |  |  |
| Contact |  |  |  | SUBSCRIBE-RSP |
| addr-spec |  |  |  |  |
| user-info and host | px_MCVideo_PublicServiceId_A MCVIDEO |  |  |  |
| Contact |  |  |  | INVITE-RSP |
| addr-spec |  |  |  |  |
| user-info and host | px_MCVideo_Client_B_ID |  |  | MCVIDEO |
|  | “audio” |  |  | MCPTT OR MCVIDEO |
|  | “video” |  |  | MCVIDEO |
| feature-param |  |  |  |  |
| Call-ID | same value as received in the request |  |  |  |
| callid |  |  |  | INVITE-RSP |
| CSeq | "timer" |  |  |  |
| value |  |  |  | INVITE-RSP |
| Session-Expires |  |  |  | INVITE-RSP |
| generic-param | "tdialog" |  |  |  |
| refresher | "norefersub" |  |  |  |
| Supported | "explicitsub" |  |  |  |
| uri-parameters |  |  | RFC 5621 [58] | INVITE-RSP |
| Content-Type | length of message-body |  |  |  |
| media-type |  |  | RFC 3261 [22] | INVITE-RSP |
| Message-body | "application/sdp" |  | RFC 4566 [27] |  |
| MIME-part-header | SDP message as described in Table 5.5.3.1.2-2 |  |  | MCVIDEO |
| MIME-part-body |  | MCPTT/MCVideo/MCData Info |  |  |
|  | "application/vnd.3gpp.mcvideo-info+xml" MCVIDEO |  |  |  |
| MIME body part | MCVideo-Info as described inTable 6.1.1.12.3.3-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |

Table 6.1.1.12.3.3-5: Transmission Granted (Steps 6, 20, Table 6.1.1.12.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.11.2.1-1, condition ON-NETWORK |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Transmission Indicator |  | Length is 16 bits, A-P. |  |  |
| Transmission Indicator | "1000000000000000" | A = Normal call | TS 24.581 [27], clause 9.2.5.1 |  |

Table: 6.1.1.12.3.3-6 Queue Position Info (Steps 10, 17, Table 6.1.1.12.3.2-1)

| Derivation Path: TS 24.581 [88] Table 9.2.12-1, Condition ON-NETWORK |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Subtype | “00101” | Server client | TS 24.581 [88] 9.2.2.1-1 |  |

Table 6.1.1.12.3.3-7: Transmission Control Ack (Step 11, Table 6.1.1.12.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.11.3.5-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message Type |  |  |  |  |
| Message Type | "00010000" | Transmission Control Ack message for Transmission Granted message which requested acknowledgment. |  |  |

Table 6.1.1.12.3.3-8: SIP 200 (OK) (Step 24, Table 6.1.1.12.3.2-1)

| Derivation Path: RFC 3261 [22], Table 5.5.2.17.1.2-1, Conditions MCVIDEO, GROUP CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME-part-body | MCVideo-Info Not present |  | TS 24.281 [86] clause F.1 | MCVIDEO |

Table 6.1.1.13.2.2-9: SIP BYE (Step 23a, Table 6.1.1.12.3.2-1)

Derivation Path: 36.579-1 [2], Table 5.5.2.2.1-1, condition MO-CALL

### <END OF CHANGES>