3GPP TSG-RAN5 Meeting #91-e	R5-213260

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0149 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Functional Alias Generic Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There were no existing generic procedures for the functional alias Rel-15 feature |  |  |  |  |  |  |  |  |
| Summary of change: |  | Created generic procedures 5.3.24 and 5.3.25 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect MCPTT test case implementation and test case verification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.24 and 5.3.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <START OF CHANGES

### 5.3.24	Generic Test Procedure for MCPTT functional alias status determination and subscription

5.3.24.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3.24.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.24.3	Procedure

Table 5.3.24.3-1: MCPTT functional alias status determination and subscription

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.3 'Generic Test Procedure for MCPTT CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) send a SIP SUBSCRIBE requesting the status of any existing functional aliases? | --> | SIP SUBSCRIBE | - | P |
| 3 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 4 | The SS (MCPTT server) sends a SIP NOTIFY with functional alias information | <-- | SIP NOTIFY | - | - |
| 5 | Check: Does the UE (MCPTT Client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| - | EXCEPTION: The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. | - | - | - | - |

5.3.24.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3.24.4-1: SIP SUBSCRIBE (step 2, Table 5.3.24.3-1)

| Derivation Path: Table 5.5.2.14-1 with condition MCPTT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| value | "4294967295" | to receive the current status and later notification | TS 24.379 [9] clause 9A.2.1.3 |  |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcptt-info+xml" |  |  |  |
| Message-body |  |  | TS 24.379 [9] clause 9A.2.1.3 |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  |  |
| Content-ID | any value | Unique URL identifying the MCPTT Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.24.4-2 |  |  |  |

Table 5.3.24.4-2: MCPTT-Info in SIP SUBSCRIBE (Table 5.3.24.4-1)

| Derivation Path: Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.2.1.3 |  |

Table 5.3.24.4-3: SIP 200 (OK) (step 3, Table 5.3.24.3-1)

Derivation Path: Table 5.5.2.17.1.2-1 with condition SUBSCRIBE-RSP

Table 5.3.24.4-4: SIP NOTIFY (step 4, Table 5.3.24.3-1)

| Derivation Path: Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF | TS 24.379 [9] clause 9A.2.2.2.5 |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/pidf+xml" |  |  |  |
| MIME-part-body | PIDF as described in Table 5.3.24.4-5 |  |  |  |

Table 5.3.24.4-5: PIDF in SIP NOTIFY (Table 5.3.24.4-4)

| Derivation Path: Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | TS 24.379 [9] clause 9A.3.1v |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.3.1 |  |
| tuple |  |  | TS 24.379 [9] clause 9A.3.1 |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| functionalAlias |  | Having no functionalAliasID and ststus parameters under functionalAlias equates to no activated functional alias |  |  |
| functionalAliasID | not present | Functional Alias ID |  |  |
| user | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status | not present |  |  |  |
| affiliation | not present |  |  |  |
| group | not present |  |  |  |
| status | not present |  |  |  |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

### 5.3.25	Generic Test Procedure for MCPTT functional alias status change

5.3.25.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3.25.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.25.3	Procedure

Table 5.3.25.3-1: MCPTT functional alias status change

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.3 'Generic Test Procedure for MCPTT CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) send a SIP PUBLISH requesting the status change of a functional alias? | --> | SIP PUBLISH | - | P |
| 3 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 4 | The SS (MCPTT server) sends a SIP NOTIFY with functional alias information | <-- | SIP NOTIFY | - | - |
| 5 | Check: Does the UE (MCPTT Client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| - | EXCEPTION: The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. | - | - | - | - |

5.3.25.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3.25.4-1: SIP PUBLISH (step 2, Table 5.3.25.3-1)

| Derivation Path: Table 5.5.2.11-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info | TS 24.379 [9] clause 9A.2.1.2 |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  |  |
| Content-ID | any value |  |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.25.4-2 |  |  |  |
| MIME body part |  | PIDF | TS 24.379 [9] clause 9A.2.1.2 |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/pidf+xml" |  |  |  |
| MIME-part-body | PIDF as described in Table 5.3.25.4-3 |  |  |  |
| MIME body part |  | MIKEY |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  |  |  |
| MIME-part-body | MIKEY message as described in Table 5.5.9.1-1 | MIKEY message, containing the CSK |  |  |

Table 5.3.25.4-2: MCPTT-Info in SIP PUBLISH (Table 5.3.25.4-1)

| Derivation Path: Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.2.1.2 |  |

Table 5.3.25.4-3: PIDF in SIP PUBLISH (Table 5.3.25.4-1)

| Derivation Path: Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | TS 24.379 [9] clause 9A.3.1 |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.3.1 |  |
| tuple |  |  | TS 24.379 [9] clause 9A.3.1 |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| functionalAlias |  |  |  |  |
| functionalAliasID | px_MCPTT_User_A_Alias | Functional Alias ID |  |  |
| user | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| affiliation | not present |  |  |  |
| group | not present |  |  |  |
| status | not present |  |  |  |
| p-id-fa | Any allowed value | a globally unique value set to an identifier of a SIP PUBLISH request | TS 24.379 [9] clause 9A.2.1.2 |  |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

Table 5.3.25.4-4: SIP 200 (OK) (step 3, Table 5.3.25.3-1)

Derivation Path: Table 5.5.2.17.1.2-1 with condition PUBLISH-RSP

Table 5.3.25.4-5: SIP NOTIFY (step 4, Table 5.3.25.3-1)

| Derivation Path: Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  | ] |  |
| MIME body part |  | PIDF | TS 24.379 [9] clause 9A.2.2.2.5 |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/pidf+xml" |  |  |  |
| MIME-part-body | PIDF as described in Table 5.3.25.4-6 |  |  |  |

Table 5.3.25.4-6: PIDF in SIP NOTIFY (Table 5.3.25.4-5)

| Derivation Path: Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| functionalAlias |  |  |  |  |
| functionalAliasID | px_MCPTT_User_A_Alias | Functional Alias ID |  |  |
| status | "activated" |  |  |  |
| affiliation | not present |  |  |  |
| group | not present |  |  |  |
| status | not present |  |  |  |

EXTRA

Table 5.3.24.4-5: PIDF in SIP NOTIFY (Table 5.3.24.4-4)

| Derivation Path: Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | TS 24.379 [9] clause 9A.3.1v |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.3.1 |  |
| tuple |  |  | TS 24.379 [9] clause 9A.3.1 |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| functionalAlias |  | Having no functionalAliasID and ststus parameters under functionalAlias equates to no activated functional alias |  |  |
| functionalAliasID | px_MCPTT_User_A_Alias | Functional Alias ID |  |  |
| user | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status | "activated" |  |  |  |
| affiliation | not present |  |  |  |
| group | not present |  |  |  |
| status | not present |  |  |  |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

### <END OF CHANGES>