3GPP TSG-RAN5 Meeting #91-e	R5-213270

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0004 | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCData Test Case 6.2.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A Rel-15 test case was needed for testing the functionality to obtain a list of deferred communication |  |  |  |  |  |  |  |  |
| Summary of change: |  | Created the new MCData test case 6.2.13 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The device feature will not be tested properly if it attempts to obtain a list of deferred communication |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <START OF CHANGES>

### 6.2.13	On-network / File Distribution (FD) / Accessing list of deferred data group communications / Client Originated (CO)

6.2.13.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a Group Standalone FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" and the UE (MCDATA Client) is unaware of the URL of the Media Storage Function }

then { UE (MCDATA Client) sends a SIP MESSAGE to find the URL of the Media Storage Function and responds to a SIP MESSAGE that contains the URL of the Media Storage Function with a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a Group Standalone FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" and the UE (MCDATA Client) is aware of the URL of the Media Storage Function }

then { UE (MCDATA Client) uploads the file to the Media Storage Function via an HTTP POST message and then sends the URL of the file location to the recipient via a SIP MESSAGE message }

}

(3)

with { UE (MCDATA Client) having sent the URL of the file location to the recipient }

ensure that {

when { UE (MCDATA Client) receives a FD notification via a SIP MESSAGE message }

then { UE (MCDATA Client) responds to the SIP MESSAGE message with a SIP 200 (OK) message and delivers the notification to the MCDATA User }

}

(4)

with { UE (MCDATA Client) having received a notification that a sent message was deferred by the recipient }

ensure that {

when { MCData User requests to access the list of deferred group communication }

then { UE (MCDATA Client) sends a SIP MESSAGE message requesting to access the list of deferred communication and responds to a received SIP MESSAGE message with a SIP 200 (OK message and delivers the notification to the MCDATA User }

}

6.2.13.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.1.3.2, 10.2.2.1, 10.2.4.2.1, 12.2.1.2, 11.3.2.1, 11.3.2.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.1.3.2]

To discover the absolute URI of the media storage function, the MCData client shall generate a SIP MESSAGE request towards the participating MCData function, in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element containing the value "msf-disc-req";

4)	if the upload of a file is for a group standalone FD request, shall include in an application/vnd.3gpp.mcdata-info+xml MIME body, the <mcdata-calling-group-id> element set to the required MCData group identity; and

NOTE 1:	The absence of a group identity in the <mcdata-calling-group-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body implies that the MCData client intends to upload a file for a one-to-one FD request. In this case, the participating MCData function identifies the MCData ID of the user from the binding between the public user identity and the MCData ID.

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of a "SIP MESSAGE request for absolute URI discovery response", the MCData client:

1)	shall store the absolute URI found in the <mcdata-controller-psi> element;

2)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5]; and

3)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 10.2.2.1]

If the media storage client is not aware of the absolute URI of the media storage function, the media storage client shall request the MCData client to discover the absolute URI associated with the media storage function by following the procedures in subclause 10.2.1.3.

The media storage client shall send HTTP requests over a TLS connection as specified for the HTTP client in the UE in annex A of 3GPP TS 24.482 [24].

NOTE 1:	The HTTP client encodes the MCData ID in the bearer access token of the Authorization header field of an HTTP request as specified in 3GPP TS 24.482 [24].

NOTE 2:	The HTTP client always sends the HTTP requests to an HTTP proxy. Annex A of 3GPP TS 24.482 [24] indicates how the HTTP proxy forwards the HTTP request to the HTTP server.

To upload a file to media storage function, the media storage client:

1)	shall generate an HTTP POST request as specified in IETF RFC 7230 [22] and IETF RFC 7231 [23];

2)	shall set the Request-URI to the absolute URI identifying the resource on a media storage function;

3)	shall set the Host header field to a hostname identifying the media storage function;

4)	shall set the Content-Type header field to multipart/mixed and with a boundary delimiter parameter set to any chosen value;

5)	if the file upload is for one-to-one file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "one-to-one-fd"; and

b)	the <mcdata-calling-user-id> element set to the originating MCData ID;

6)	if the file upload is for group file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "group-fd";

b)	the <mcdata-request-uri> element set to the MCData group identity; and

c)	the <mcdata-calling-user-id> element set to the originating MCData ID;

7)	if end-to-end security is required for a one-to-one communication, the MCData client protects the binary data representing the file and prefixes the protected binary data with security parameters as described in 3GPP TS 33.180 [26];

8)	if

i)	end-to-end security is not required for a one-to-one communication, or

ii)	the file upload is for group file distribution;

shall include the binary data representing the file with Content-Type field set to application/octet-stream and Content-Length field set to the file size; and

9)	shall send the HTTP POST request towards the media storage function.

On receipt of a HTTP 201 Created containing a Location header field with a URL identifying the location of the resource where the file has been stored on the media storage function, then the media storage client shall store this information.

[TS 24.282, clause 10.2.4.2.1]

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	if a one-to-one standalone FD message is to be sent shall insert in the SIP MESSAGE request:

a)	an application/resource-lists+xml MIME body with the MCData ID of the target MCData user, according to rules and procedures of IETF RFC 4826 [9]; and

b)	an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element set to a value of "one-to-one-fd";

…

4)	shall generate a standalone FD message as specified in subclause 6.2.2.2; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

[TS 24.282, clause 11.3.2.1]

Upon receiving a request from the MCData user to access the list of deferred data group communications, the MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall generate DEFERRED DATA REQUEST message as specified in subclause 15.1.11.1;

3)	shall include in the SIP request, the DEFERRED DATA GROUP COMM message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1; and

4)	shall send the SIP MESSAGE request towards the participating MCData function according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 11.3.2.2]

Upon receipt of a "SIP MESSAGE response for the list of deferred group communications request", the MCData client:

1)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

2)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5];

3)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body:

a)	if the application/vnd.3gpp.mcdata-signalling MIME body contains DEFERRED DATA RESPONSE message as specified in subclause 15.1.12:

i)	for each payload, if payload type is set to "FILEURL", shall store the payload data; and

4)	shall present to MCData user, the list of file URLs which were deferred.

6.2.13.3	Test description

6.2.13.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

-	A binary file is available for the UE (MCDATA Client) to upload.

-	The binary file is smaller than the <mcdata-on-network-max-data-size-for-FD> element in the MCData group document for the MCData group ID as specified in 3GPP TS 24.481 [11].

-	The binary file is smaller than the <mcdata-max-data-in-single-request> element of the <entry> element of the MCData group document as specified in 3GPP TS 24.481 [11].

-	The binary file is smaller than the <mcdata-on-network-max-data-size-auto-recv> element of the MCData service configuration document as specified in 3GPP TS 24.484 [14].

Preamble:

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	Timer TDU2 (FD non-mandatory download timer) is configured to be 15 seconds.

-	The <mcdata-allow-transmit-data-in-this-group> element of an <actions> element is present in the MCData group document with the value "true" as specified in 3GPP TS 24.481 [11].

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.13.3.2	Test procedure sequence

Table 6.2.13.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCDATA User request to send a binary file that is larger than <max-data-size-auto-recv-bytes> via a Group Standalone FD message with a non-mandatory download and with a disposition request of "FILE DOWNLOAD COMPLETED UPDATE". (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCDATA call establishment are described in TS 36.579-1 [2], subclause 5.4.3B 'Generic Test Procedure for MCData CO communication in E-UTRA'. The test sequence below shows only the MCDATA relevant messages exchanged. | - | - | - | - |
| - | EXCEPTION: Steps 2a1 through 2a4 describe behaviour that depends on UE implementation. Steps 2a1 through 2a4 are only valid if the US (MCDATA Client) is not aware of the URI of the media storage function. | - | - | - | - |
| 2a1 | Check: Does the UE (MCDATA Client) send a SIP MESSAGE message to discover the absolute URI of the media storage function? | --> | SIP MESSAGE | 1 | P |
| 2a2 | SS (MCDATA Server) responds with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 2a3 | SS (MCDATA Server) responds with a SIP MESSAGE message with the absolute URI found in the <mcdata-controller-psi> element | <-- | SIP MESSAGE | - | - |
| 2a4 | Check: Does the UE (MCDATA Client) responds with a SIP 200 (OK) message? | --> | SIP 200 (OK) | 1 | P |
| 3 | Check: Does the UE (MCDATA Client) send a HTTP POST message over a TLS connection to upload the binary file to the media storage function? | --> | HTTP POST | 2 | P |
| 4 | SS (MCDATA Server) responds with a HTPP 201 Created message containing a Location header field with a URL identifying the location of the resource where the file has been stored on the media storage function. | <-- | HTTP 201 Created | - | - |
| 5 | Check: Does the UE (MCDATA Client) send a SIP MESSAGE message with a FD SIGNALLING PAYLOAD which contains the Payload content type of "FILEURL" and the URL of the file in the Payload data? (NOTE 2) | --> | SIP MESSAGE | 2 | P |
| 6 | SS (MCDATA Server) responds with a SIP 202 (Accepted) message | <-- | SIP 202 (Accepted) | - | - |
| 7 | SS (MCDATA Server) sends a disposition notification type of "FILE DOWNLOAD DEFERRED" via a SIP MESSAGE message before time TDU2 (FD non-mandatory download timer) expires. | <-- | SIP MESSAGE | - | - |
| 8 | Check: Does the UE (MCDATA Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | 3 | P |
| - | EXCEPTION: SS (MCDATA Server) releases the E-UTRA connection. | - | - | - | - |
| 9 | Check: Does the UE (MCDATA Client) deliver the notification the MCDATA User? (NOTE 1) | - | - | 3 | P |
| 10 | Make the MCData User request to access the list of deferred data group communication. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCDATA call establishment are described in TS 36.579-1 [2], subclause 5.4.3B 'Generic Test Procedure for MCData CO communication in E-UTRA'. The test sequence below shows only the MCDATA relevant messages exchanged. | - | - | - | - |
| 11 | Check: Does the UE (MCDATA Client) send a SIP MESSAGE message requesting to access the list of deferred data group communication? | --> | SIP MESSAGE | 4 | P |
| 12 | SS (MCDATA Server) responds with a SIP 200 (OK) message | <-- | SIP 200 (OK) | - | - |
| 13 | SS (MCDATA Server) sends a list of deferred group communications. | <-- | SIP MESSAGE | - | - |
| 14 | Check: Does the UE (MCDATA Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | 4 | P |
| - | EXCEPTION: SS (MCDATA Server) releases the E-UTRA connection. | - | - | - | - |
| 15 | Check: Does the UE (MCDATA Client) present the list of file URLs which were deferred to MCData User? (NOTE 1) | - | - | 4 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Timer TDU2 (FD non-mandatory download timer) starts on reception of the FD request via the SIP MESSAGE message. |  |  |  |  |  |

6.2.13.3.3	Specific message contents

Table 6.2.13.3.3-1: SIP MESSAGE (step 2a1, Table 6.2.13.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value | "g.3gpp.mcdata.fd" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Service |  |  | TS 24.282 clause 6.2.4.1 [31] |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCDATA_ID_User_A | The public user identity of the MCData User |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-info+xml" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.13.3.3-2 |  |  |  |
| MIME body part | not present | SDS SIGNALLING PAYLOAD |  |  |
| MIME body part | not present | DATA PAYLOAD |  |  |

Table 6.2.13.3.3-2: MCDATA-Info (Table 6.2.13.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "msf-disc-req" |  |  |  |
| mcdata-request-uri | not present |  |  |  |
| mcdata-calling-user-id | px_MCDATA_ID_Client_A |  |  |  |

Table 6.2.13.3.3-3: SIP 200 (OK) (steps 2a2, 12, Table 6.2.13.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP, MCDATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Record-Route | not present |  |  |  |
| Contact |  |  |  |  |
| feature-param | "+g.3gpp.mcdata.fd" |  |  |  |
| Content-Type | not present |  |  |  |
| Content-Length |  |  |  |  |
| value | "0" | No message body included |  |  |

Table 6.2.13.3.3-4: SIP MESSAGE (step 2a3, Table 6.2.13.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value | "g.3gpp.mcdata.fd" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Service |  |  | TS 24.282 clause 6.2.4.1 [31] |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCDATA_ID_User_A | The public user identity of the MCData User |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-info+xml" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.13.3.3-5 |  |  |  |
| MIME body part | not present | SDS SIGNALLING PAYLOAD |  |  |
| MIME body part | not present | DATA PAYLOAD |  |  |

Table 6.2.13.3.3-5: MCDATA-Info (Table 6.2.13.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "msf-disc-req" |  |  |  |
| mcdata-request-uri | not present |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |
| mcdata-called-party-id | not present |  |  |  |
| mcdata-client-id | not present |  |  |  |
| mcdata-controller-psi | px_MCDATA_MSF | element set to the absolute URI of the media storage function |  |  |

Table 6.2.13.3.3-6: SIP 200 (OK) (steps 2a4, 8, 14, Table 6.2.13.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1, condition INVITE-RSP, MCDATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Record-Route | not present |  |  |  |
| Contact |  |  |  |  |
| feature-param | "+g.3gpp.mcdata.fd" | The default value is "+g.3gpp.mcdata.sds" |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" | The default value is "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  |
| Content-Type | not present |  |  |  |
| Content-Length |  |  |  |  |
| value | "0" | No message body included |  |  |

Table 6.2.13.3.3-7: HTTP POST (step 3, Table 6.2.13.3.2-1)

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | px_MCDATA_MSF | The absolute URI identifying the resource on a media storage function | TS 24.282 [31], clause 10.2.2.1 |  |
| Authorization |  | The HTTP client encodes the MCData ID in the bearer access token of the Authorization header field of an HTTP request as specified in TS 24.482 [12]. | TS 24.482 [12], clause A.2.3 |  |
| authentication-scheme | "Bearer" |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| P-Asserted-Identity |  |  | TS 24.482 [12], clause A.2.3 RFC 3325 [32] |  |
| addr-spec |  |  |  |  |
| user-info and host | px_MCDATA_ID_User_A | The URI of the UE |  |  |
| Host |  |  |  |  |
|  | px_MCDATA_MSF_Hostname | hostname identifying the media storage function | TS 24.282 [31], clause 10.2.2.1 |  |
| Content-Length | length of message body |  |  |  |
| value | length of message-body |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  | TS 24.282 [31], clause 10.2.2.1 |  |
| Message-body |  |  |  |  |
| Authentication Request | not present |  |  |  |
| Message-body |  |  |  |  |
| Token request | not present |  |  |  |
| Message-body |  |  |  |  |
| KMS Initialize | not present |  |  |  |
| Message-body |  |  |  |  |
| KMS KeyProvision | not present |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCDATA-Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcdata-info+xml" |  |  |  |
| MIME-part-body | As described in Table 6.2.13.3.3-8 |  |  |  |
| MIME body part |  | DATA PAYLOAD | TS 24.282 [87] clause 10.2.2.1 |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/octet-stream" |  |  |  |
| Content-Length | set to the file size of the uploaded file |  |  |  |
| MIME-part-body | As described in Table 6.2.13.3.3-9 |  |  |  |

Table 6.2.13.3.3-8: MCDATA-Info (Table 6.2.13.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-fd" |  |  |  |
| mcdata-request-uri | px_MCDATA_Group_A |  |  |  |
| mcdata-calling-user-id | px_MCDATA_ID_User_A |  |  |  |
| mcdata-client-id | not present |  |  |  |

Table 6.2.13.3.3-9: DATA PAYLOD (Table 6.2.13.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.9-1, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Payload |  |  |  |  |
| Payload content type | "00000010" | BINARY |  |  |
| Payload data | the file to be uploaded |  |  |  |

Table 6.2.13.3.3-10: HTTP 201 Created (step 4, Table 6.2.13.3.2-1)

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Location |  |  |  |  |
| absoluteURI | px_MCDATA_FD_FILE_LOC | The location of the uploaded file |  |  |
| Content-Length | not present |  |  |  |
| Content-Type | not present |  |  |  |
| Message-body | not present |  |  |  |

Table 6.2.13.3.3-11: SIP MESSAGE (step 5, Table 6.2.13.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA, PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcdata.fd" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  |  |
| P-Preferred-Identity |  |  |  |  |
| PPreferredID-value | px_MCDATA_ID_User_A | The public user identity |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Info |  |  |
| MIME-part-body | As described in Table 6.2.13.3.3-12 |  |  |  |
| MIME body part |  | FD SIGNALLING PAYLOAD |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | FD Signalling Payload from the UE as described in TS 36.579-1 [2] Table 5.5.3.13.1-1 |  |  |  |
| MIME body part | not present | SDS SIGNALLING PAYLOAD |  |  |
| MIME body part | not present | DATA PAYLOAD |  |  |

Table 6.2.13.3.3-12: MCDATA-Info (Table 6.2.13.3.3-11)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-fd" |  |  |  |
| mcdata-request-uri | px_MCDATA_Group_A |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |
| mcdata-client-id | px_MCDATA_ID_User_A |  |  |  |

Table 6.2.13.3.3-13: SIP MESSAGE (step 7, Table 6.2.13.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA, PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  | TS 24.282 [31] clause 6.2.4.1 |  |
| ac-value |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcdata.fd" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Service |  |  | TS 24.282 [31] clause 6.2.4.1 |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Info |  |  |
| MIME-part-body | As described in Table 6.2.1.3.3-14 |  |  |  |
| MIME body part | not present | SDS SIGNALLING PAYLOAD |  |  |
| MIME body part | not present | DATA PAYLOAD |  |  |
| MIME body part |  | FD NOTIFICATION |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | FD NOTIFICATION from the SS as described in Table 6.2.13.3.3-15 |  |  |  |

Table 6.2.13.3.3-14: MCDATA-Info (Table 6.2.13.3.3-13)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | not present |  |  |  |
| mcdata-request-uri | px_MCDATA_ID_User_A |  |  |  |
| mcdata-calling-group-id | px_MCDATA_Group_A |  |  |  |
| mcdata-client-id | not present |  |  |  |

Table 6.2.13.3.3-15: FD NOTIFICATION (Table 6.2.13.3.3-13)

| Derivation Path: TS 24.282 [31], Table 15.1.6.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| FD notification message identity | "00000110" | FD NOTIFICATION | TS 24.282 [31] clause 15.2.2 |  |
| FD disposition notification type | "00000100" | FILE DOWNLOAD REQUEST DEFERRED | TS 24.282 [31] clause 15.2.6 |  |
| Date and time | The current date and time | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [31] clause 15.2.8 |  |
| Conversation ID | same value as received in MESSAGE | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [31] clause 15.2.9 |  |
| Message ID | same value as received in MESSAGE | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [31] clause 15.2.10 |  |
| Application ID | not present |  | TS 24.282 [31] clause 15.2.7 |  |

Table 6.2.13.3.3-16: SIP MESSAGE (step 11, Table 6.2.13.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value | "g.3gpp.mcdata.fd" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Service |  |  | TS 24.282 clause 6.2.4.1 [31] |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCDATA_ID_User_A | The public user identity of the MCData User |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | DEFERRED DATA REQUEST |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | DEFERRED DATA REQUEST as described in Table 6.2.13.3.3-17 |  |  |  |
| MIME body part | not present | MCData-Info |  |  |
| MIME body part | not present | SDS SIGNALLING PAYLOAD |  |  |
| MIME body part | not present | DATA PAYLOAD |  |  |

Table 6.2.13.3.3-17: DEFERRED DATA REQUEST (Table 6.2.13.3.3-16)

| Derivation Path: TS 24.282 [87] clause 15.1.11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Deferred data request message identity | "01001011" | Deferred List Access Request | TS 24.282 [87] clause 15.2.2 |  |

Table 6.2.13.3.3-18: SIP MESSAGE (step 13, Table 6.2.13.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value | "g.3gpp.mcdata.fd" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Service |  |  | TS 24.282 clause 6.2.4.1 [31] |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCDATA_ID_User_A | The public user identity of the MCData User |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-info+xml" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | DEFERRED DATA RESPONSE |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | DEFERRED DATA RESPONSE as described in Table 6.2.13.3.3-19 |  |  |  |
| MIME body part | not present | MCData-Info |  |  |
| MIME body part | not present | SDS SIGNALLING PAYLOAD |  |  |
| MIME body part | not present | DATA PAYLOAD |  |  |

Table 6.2.13.3.3-19: DEFERRED DATA RESPONSE (Table 6.2.13.3.3-18)

| Derivation Path: TS 24.282 [87] clause 15.1.12 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Deferred data response message identity | "01001100" | Deferred List Access Response | TS 24.282 [87] clause 15.2.2 |  |
| Number of payloads | "1" |  | TS 24.282 [87] clause 15.2.12 |  |
| Security parameters and Payload | Not present |  |  |  |
| Payload |  |  | TS 24.282 [87] clauses 11.3.3.2 and 15.2.13 |  |
| Length of Payload contents | Length of the payload contents |  |  |  |
| Payload content type | “00000100” | FILEURL |  |  |
| Payload contents | px_MCDATA_FD_FILE_LOC |  |  |  |

### <END OF CHANGES>