3GPP TSG-RAN5 Meeting # 91-e	R5-213280

# Electronic Meeting,  17th May - 28th May

Title:		On FR2 relative and aggregate power tolerance Measurement Uncertainties and test tolerances

Source:	Keysight Technologies

Agenda Item:		5.3.2.17

Document for:		Endorsement

# 1.	Introduction

# FR2 UL relative and aggregate power level control measurement uncertainties was already discussed in RAN5#90e ([1]) but it is still open.

# In such document the following proposals were made:

# In this paper, we provide company views on this topic and we also propose values for Test tolerances.

# The proposals on this paper are applicable for both IFF and DFF.

# 2.	Discussion

# In [1], it was proposed:

- To cancel out all MU elements except power measurement equipment uncertainty and influence of noise, similar to what was done for DL SNR MU in [2]. In our opinion, this is the correct approach.

# Proposal 1 : Adopt N/A for the MU elements other than power measurement equipment uncertainty and influence of noise for FR2 relative power measurement MU.

- To consider only linearity and impact of the averaging time for the power measurement equipment uncertainty with a total value of ±0.4dB. We agree with this statement (including the value proposed).

# Proposal 2 : Adopt +/- 0.4dB (as expanded uncertainty) for power measurement equipment uncertainty for FR2 relative power measurement MU.

- To consider for the influence of noise, the noise impact for the lowest absolute power measurement measured in the test, i.e. ±1 dB.

# Proposal 3 : Adopt 1.0dB(systematic error) as influence of noise(SNR) for FR2 relative power measurement MU.

# As indicated in [1], with proposals 1, 2 and 3, the total MU of UL relative power measurement becomes 0.4 + 1.0 = ±1.4dB.

# Regarding test tolerances, as this is a power measurement related test case, we propose to consider test tolerance equal to 65% of (measurement uncertainties – impact of noise) .

# Proposal 4 : Adopt TT = 0.65 x (MTSUIFF – 1.0) for test tolerances.

# Associated CRs can be found in [3] and [4].

# 3.	Conclusion

# Proposal 1 : Adopt N/A for the MU elements other than power measurement equipment uncertainty and influence of noise for FR2 relative power measurement MU.

# Proposal 2 : Adopt +/- 0.4dB (as expanded uncertainty) for power measurement equipment uncertainty for FR2 relative power measurement MU.

# Proposal 3 : Adopt 1.0dB(systematic error) as influence of noise(SNR) for FR2 relative power measurement MU.

# Proposal 4 : Adopt TT = 0.65 x (MTSUIFF – 1.0) for test tolerances.

# 4.	References

[1] R5-210845, “On relative power measurement uncertainty”, Anritsu, RAN5#90e

[2] R5-204089, “On gNB emulator SNR uncertainty for FR2”, RAN5#88e, Anritsu

[3] R5-213279, “Measurement uncertainties for FR2 Relative and aggregate power tolerance”, RAN5#91e, Keysight Technologies

[4] R5-213278, “Measurement uncertainties and test tolerances for FR2 Relative and aggregate power tolerance”, RAN5#91e, Keysight Technologies