3GPP TSG-RAN5 Meeting # 91-e	R5-213287

# Electronic Meeting,  17th May - 28th May

Title:	On UE Beamlock Function for different MIMO layers

Source:						Keysight Technologies

Agenda Item:			5.3.2.17

Document for:			Endorsement

# Introduction

This contribution is discussing the need to further clarify the UBF, defined in [1], and the need to augment the applicability of the UBF for UL-MIMO.

# Discussion

For UL-MIMO test cases, the current UBF definition in [1] is sufficient. Many test cases activate the UBF.

For these test cases to be executed accurately and reliably, it needs to be guaranteed that the UBF activation is applicable to all DL & UL MIMO threads.

It is therefore proposed for the OEMs and chipset vendors to confirm that upon UBF activation, all DL &UL MIMO threads beams are locked.

Proposal 1: OEMs and chipset vendors to confirm that upon UBF activation, all DL & UL MIMO threads beams affected by the test function are locked.

Upon confirmation, it is proposed to augment the UBF definition in [1], e.g., by adding the following statement

5.4.1 General The UE Beamlock test function is intended for making the UE to lock the UE antenna pattern once it has formed a beam towards the base station (SS) direction following the cell identification procedure in preparation for subsequent test procedures. Activating the UBF shall lock the antenna pattern of all active intra-band component carriers and all MIMO layers affected by the test function.

Proposal 2: Augment the UBF definition in [1] to highlight that the UBF activation applies to all MIMO layers affected by the test function as shown in [2].

# Conclusion

The following observations and proposals were made in this contribution

Proposal 1: OEMs and chipset vendors to confirm that upon UBF activation, all UL MIMO threads beams are locked.

Proposal 2: Augment the UBF definition in [1] to highlight that the UBF activation applies to all MIMO layers affected by the test function as shown in [2].

# References

- TS 38.509, “Special conformance testing functions for User Equipment (UE)”, V16.0.0

- R5-213288, “UE Beamlock Function update for MIMO”, Keysight Technologies, RAN5#91e