3GPP TSG-RAN5 Meeting # 91-e	R5-213290

# Electronic Meeting,  17th May - 28th May

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0542 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex C: Clarifications to downlink signal levels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In current specification version, SS/PBCH SSS EPRE is defined in dBm/60 kHz. However, any EPRE is defined as power per resource element. - Default Downlink power levels for NR are still in square brackets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Defined SS/PBCH SSS EPRE in dBm/SCS (without changing existing channel power values). - Square brackets removed from default downlink power levels for NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will remain misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This clarification does not imply any change to test frequencies defined in current version of the specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

Annex C (normative): Downlink Physical Channels

# C.0	Downlink signal levels

Editor’s Note : Consideration to minimize the required number of additional FR2 link is under discussion

The downlink power settings in Table C.0-1 is used unless otherwise specified in a test case.

Table C.0-1: Default Downlink power levels for NR

| SCS (kHz) |  | Unit | Channel Bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | Number of RBs |  | 66 | 132 | 264 | N/A |
|  | Channel BW power | dBm | -70 | -67 | -64 | N/A |
| 120 | Number of RBs |  | 32 | 66 | 132 | 264 |
|  | Channel BW power | dBm | -70 | -67 | -64 | -61 |
|  | SS/PBCH SSS EPRE | dBm/SCS | -99 for DL SCS= 60 kHz -96 for DL SCS = 120 kHz | -99 for DL SCS = 60 kHz -96 for DL SCS = 120 kHz | -99 for DL SCS = 60 kHz -96 for DL SCS = 120 kHz | -99 for DL SCS = 60 kHz -96 for DL SCS = 120 kHz |
| Note 1: The channel bandwidth powers are informative, based on [-99]dBm/60kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at the centre of quiet zone. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of [–99]dBm/60kHz. |  |  |  |  |  |  |

The default downlink signal level uncertainty is +/- TBD dB, for any level specified. If the uncertainty value is critical for the test purpose, a tighter uncertainty is specified for the related test case in Annex F.

For TRP measurement, DL signal may be supplied from RSRP based pathloss compensation link. Downlink signal level using RSRP based pathloss compensation link is specified in Table C.0-2 or Table C.0-3.

Table C.0-2: Downlink power levels for RSRP based pathloss compensation link for TRP measurement for n257, n258 and n260

| SCS (kHz) |  | Unit | Channel Bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | Number of RBs |  | 66 | 132 | 264 | N/A |
|  | Channel BW power | dBm | ≥ -87 | ≥ -84 | ≥ -80 | N/A |
| 120 | Number of RBs |  | 32 | 66 | 132 | 264 |
|  | Channel BW power | dBm | ≥ -87 | ≥ -84 | ≥ -80 | ≥ -77 |
|  | SS/PBCH SSS EPRE | dBm/SCS | ≥ -115.5 for DL SCS = 60 kHz ≥ -112.5 for DL SCS = 120 kHz | ≥ -115.5 for DL SCS = 60 kHz ≥ -112.5 for DL SCS = 120 kHz | ≥ -115.5 for DL SCS = 60 kHz ≥ -112.5 for DL SCS = 120 kHz | ≥ -115.5 for DL SCS = 60 kHz ≥ -112.5 for DL SCS = 120 kHz |
| Note 1: The channel bandwidth powers are informative, based on -115.5dBm/60kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at the RSRP reference point as defined in TS 38.215 [24]. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of ≥ –115.5 dBm/60kHz. |  |  |  |  |  |  |

Table C.0-3: Downlink power levels for RSRP based pathloss compensation link for TRP measurement for n261

| SCS (kHz) |  | Unit | Channel Bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | Number of RBs |  | 66 | 132 | 264 | N/A |
|  | Channel BW power | dBm | ≥ -84 | ≥ -81 | ≥ -78 | N/A |
| 120 | Number of RBs |  | 32 | 66 | 132 | 264 |
|  | Channel BW power | dBm | ≥ -84 | ≥ -81 | ≥ -78 | ≥ -75 |
|  | SS/PBCH SSS EPRE | dBm/SCS | ≥ -113 for DL SCS = 60 kHz ≥ -110 for DL SCS = 120 kHz | ≥ -113 for DL SCS = 60 kHz ≥ -110 for DL SCS = 120 kHz | ≥ -113 for DL SCS = 60 kHz ≥ -110 for DL SCS = 120 kHz | ≥ -113 for DL SCS = 60 kHz ≥ -110 for DL SCS = 120 kHz |
| Note 1: The channel bandwidth powers are informative, based on -113dBm/60kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at the RSRP reference point as defined in TS 38.215 [24]. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of ≥ –113 dBm/60kHz. |  |  |  |  |  |  |

# C.1	General

<< End of change>>