3GPP TSG-RAN5 Meeting #91-e	R5-213329

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0552 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EIS Requirements update for Rel.16 Inter-band CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Update the Inter-band DL CA requirements introduced by Rel16 core specifications and FR2 enhancements WID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update REFSENS requirements for inter-band DL CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | REFSENS requirements will be missing inter-band DL CA configuration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <Unchanged sections skipped>

##### 7.3A.2.0.2	Intra-band non-contiguous CA

For each component carrier in the intra-band non-contiguous carrier aggregation, the throughput in QPSK R=1/3 shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal) with peak reference sensitivity values determined from section 7.3.2.3, and relaxation applied to peak reference sensitivity requirement as specified in Table 7.3A.2.0.2-1.

The cumulative aggregated channel bandwidth is defined as the frequency band from the lowest edge of the lowest CC to the upper edge of the highest CC of all UL and DL configured CCs.

The normative reference for this requirement is TS 38.101-2 [3] clause 7.3A.2.2.

Table 7.3A.2.0.2-1: ΔRIB EIS Relaxation for CA operation by cumulative aggregated channel bandwidth

| Cumulative Aggregated Channel BW (MHz) | ΔRIB (dB) |
| --- | --- |
| ≤ 800 | 0.0 |
| [> 800 and ≤ 1400] | [0.5] |

#### 7.3A.2.0.3	Inter-band CA

The inter-band requirement applies for all active component carriers. The throughput for each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes [A.2.3.2] and [A.3.3.2] (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex [A.5.2.1]) with peak reference sensitivity for each carrier specified in section [7.3.2], and relaxation ΔRIB,P,n applied  to peak reference sensitivity requirement. ΔRIB,P,n is specified in Table 7.3A.2.0.3-1. The requirement on each component carrier shall be met when the power in the component carrier in the other band is set to its EIS spherical coverage requirement for inter-band CA specified in sub-clause [7.3A.3.3].

For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIB, is also applied according to the clause [7.3A.2.1] and [7.3A.2.2].

Table 7.3A.2.0.3-1: ΔRIB reference sensitivity relaxation for inter-band CA for power class 3

| NR CA bands | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |

<Unchanged Sections Skipped>

#### 7.3A.2.1	Reference sensitivity power level for CA (2DL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

- Some references are in square brackets for inter-band DL CA

<Unchanged sections skipped>

7.3A.2.1.4.2	Test Procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [10] clause 5.5.1. Message contents are defined in clause 7.3A.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321[28], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.2).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3A.2.1.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information on PCC for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to Table 7.3A.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

6.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200msec for the UE to reach PUMAX.

7.	Set the UE in the Rx beam peak direction found with a 3D EIS scan as performed in Annex K.1.2.. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Rx beam selection to complete.

8.	For each component carrier, perform EIS procedure as stated in Annex K.1.4 to calculate “averaged EIS” by changing the power level of the wanted signal with a step size of 0.2dB. For each power step measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

9.	For each component carrier, compare the dB value of the “averaged EIS” value corresponding to the Rx beam peak direction (same as that found for single carrier in clause 7.3.2) identified in step 8 to the test requirement in Tables 7.3A.2.1.5-4 to Table 7.3A.2.1.5-7. If the EIS value is lower or equal to the value in Tables 7.3A.2.1.5-4 to Table 7.3A.2.1.5-7, pass the UE. Otherwise fail the UE.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

7.3A.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.1.

7.3A.2.1.5	Test requirement

For each component carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2 and A.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5) with peak reference sensitivity specified in Tables 7.3A.2.1.5-3 to 7.3A.2.1.5-7. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 7.3A.2.1.5-1: ΔRIB EIS Relaxation per component carrier for intra-band contiguous CA

| Aggregated Channel BW ‘BWChannel_CA’ (MHz) | ΔRIB (dB) / CC |
| --- | --- |
| BWChannel_CA ≤ 800 | 0.0 |
| 800 < BWChannel_CA ≤ 1200 | 0.5 |

Table 7.3A.2.1.5-2: ΔRIB EIS Relaxation per component carrier for intra-band non-contiguous CA

| Cumulative Aggregated Channel BW (MHz) | ΔRIB (dB) / CC |
| --- | --- |
| ≤ 800 | 0.0 |
| [> 800 and ≤ 1400] | [0.5] |

Table 7.3A.2.1.5-3: ΔRIB reference sensitivity relaxation for inter-band CA for power class 3

| NR CA bands | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |

Table 7.3A.2.1.5-4: Reference sensitivity per component carrier for power class 1

| Operating band | REFSENS (dBm) / CC |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97.5+TT+ΔRIB | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB |
| n258 | -97.5+TT+ΔRIB | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB |
| n260 | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB | -85.5+TT+ΔRIB |
| n261 | -97.5+TT+ΔRIB | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB |

Table 7.3A.2.1.5-5: Reference sensitivity per component carrier for power class 2

| Operating band | REFSENS (dBm) / CC |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB | -85.5+TT+ΔRIB |
| n258 | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB | -85.5+TT+ΔRIB |
| n260 | -85.7+TT+ΔRIB | -82.7+TT+ΔRIB | -79.7+TT+ΔRIB | -76.7+TT+ΔRIB |
| n261 | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB | -85.5+TT+ΔRIB |

Table 7.3A.2.1.5-6: Reference sensitivity per component carrier for power class 3

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.3+TT+ΔRIB | -85.3+TT+ΔRIB | -82.3+TT+ΔRIB | -79.3+TT+ΔRIB |
| n258 | -88.3+TT+ΔRIB | -85.3+TT+ΔRIB | -82.3+TT+ΔRIB | -79.3+TT+ΔRIB |
| n260 | -85.7+TT+ΔRIB | -82.7+TT+ΔRIB | -79.7+TT+ΔRIB | -76.7+TT+ΔRIB |
| n261 | -88.3+TT+ΔRIB | -85.3+TT+ΔRIB | -82.3+TT+ΔRIB | -79.3+TT+ΔRIB |

Table 7.3A.2.1.5-6a: Test Tolerance per component carrier (Reference sensitivity for power class 3)

| Test Metric | f ≤ 40.8 GHz |
| --- | --- |
| IFF (Quiet Zone size ≤ 30 cm) | 3.37 dB |

Table 7.3A.2.1.5-7: Reference sensitivity per component carrier for power class 4

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97+TT+ΔRIB | -94+TT+ΔRIB | -91+TT+ΔRIB | -88+TT+ΔRIB |
| n258 | -97+TT+ΔRIB | -94+TT+ΔRIB | -91+TT+ΔRIB | -88+TT+ΔRIB |
| n260 | -95+TT+ΔRIB | -92+TT+ΔRIB | -89+TT+ΔRIB | -86+TT+ΔRIB |
| n261 | -97+TT+ΔRIB | -94+TT+ΔRIB | -91+TT+ΔRIB | -88+TT+ΔRIB |

#### <End of changes>