3GPP TSG-RAN5 Meeting #91-e	R5-213337
Electronic Meeting, 17 May– 28 May 2021

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	On 2AoA angle selection between iterations for step 3

Document for:	Discussion and Endorsement

# Introduction

This contribution discusses and propose a solution for algorithm used in angle selection between iterations for RRM 2AoAs setup 3.

# Discussion

RRM Setup 3 defines 2 active AoAs with angle changes between iterations as defined in [1] Annex A.9.3:

## A.9.3	Setup 3: 2 AoAs

There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. The relative angular offset between the directions (AoAs) of the 2 active probes, shall be changed for each test iteration. Unless otherwise specified by the test case, the TE shall alternate between a minimum of [10] different test points. A test point comprehends the relative angular offset between the active probes, and the AoA of each of the signals from the UE point of view. The relative angular offset between the active probes must change for consecutive test points.

The applicable set of relative angular offsets between the 2 active probes is given in Table A.9.3-1 for each UE power class.

Table A.9.3-1: Set of relative angular offsets between active probes for each power class

| UE Power class | Relative angular offset between active probes |
| --- | --- |
| 1 | FFS |
| 2 | FFS |
| 3 | 30°, 60°, 90°, 120° and 150° |
| 4 | FFS |

Note: if it is not possible to find at least [10] different test points meeting the required criteria (that is, the two AoAs are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] and the angular offset between them corresponds to one of the offsets in Table A.9.3-1 for the corresponding UE power class), the test shall [alternate between all the available test points].

Editor’s Note: the definition of further rules to select the different test points is FFS.

Current approach defines minimum of 10 different test points where each test point is identified as couple of angles where both angles meet with:

- EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class

- Relative angular offsets between the 2 active probes is given in [1] Table A.9.3-1 for each UE power class.

Hence, test points selection depends on UE EIS spherical coverage and Rx antenna pattern to map test points where EIS spherical coverage meets with relative angles for the active probes defined in [1] Table A.9.3-1.

However, it is assumed TE will find enough number of suitable test points meeting EIS spherical coverage and relative angular offset between 2 active probes. In order to ensure test equipment test, 2AoA setup 3 test cases, in a more optimize way and closer to a real scenario it is proposed to avoid restriction on minimum number of test points.

Proposal 1: To avoid restriction on minimum number of test points for 2AoA setup 3.

Proposal 2: New test point selection proposal based on a sweep around all test points meeting with EIS spherical coverage and relative angular offsets between the 2 active probes given in [1] Table A.9.3-1 before repeat test points between iterations.

# Conclusion

The following observations and proposals were made in this contribution.

Proposal 1: To avoid restriction on minimum number of test points for 2AoA setup 3.

Proposal 2: New test point selection proposal based on a sweep around all test points meeting with EIS spherical coverage and relative angular offsets between the 2 active probes given in [1] Table A.9.3-1 before repeat test points between iterations.

# References

- TS 38.533, User Equipment (UE) conformance specification; Radio Resource Management (RRM), V16.7.0 (2021-03)