3GPP TSG-RAN5 Meeting #91-e	R5-213340

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1957 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update message content for subband CQI reporting test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Same reason for change as in agreed R5-211664 in RAN5#90e for PMI reporting test cases. Subband CQI reporting test cases are asking to transmit HARQ feedback and CSI feedback in PUSCH in the same UL slot and UL symbols. However, according to TS 38.213, section 9: A UE does not expect to detect a DCI format scheduling a PDSCH reception or a SPS PDSCH release, or a DCI format including a One-shot HARQ-ACK request field with value 1, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission. So, when the UCI for HARQ feedback is sent in PUSCH, all the DCI 1-1 used to schedule the PDSCH transmission have to be sent before sending the the DCI 0-1 to configure the PUSCH transimission in which the UCI is expected. However, this is not possible according the DL RMC defined for these tests, where the number of slots in DL is configured to maximize the bit rate. To allow the transmission of all DL slots defined in DL RMC, it is needed to avoid the transmission of the HARQ feedback and the CSI reporting multiplexed in the same UL slot and UL symbols |  |  |  |  |  |  |  |  |
| Summary of change: |  | To avoid the transmission of the HARQ feedback and the CSI reporting multiplexed in the same UL symbols: - Modify PUSCH-TimeDomainResourceAllocationList to decrease number of symbols for PUSCH to 12, for CSI reporting. Modify PUCCH-ResourceIndex to 6 for DCI 1-1 to send the HARQ feedback in PUCCH in symbols 12 and 13. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DL RMC defined in the test spec will not be transmitted and maximum bit rate for the DL RMC cannot be reached. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 5.4.2.4	Message contents for CQI reporting requirements

##### NZP-CSI-RS for Tracking

###### CSI-RS-ResourceMapping

Table 5.4.2.4-1: CSI-RS-ResourceMapping for TRS

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | k0=0 for CSI-RS resource 1,2,3,4 | TRS |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 4 | l0 = 4 for CSI-RS resource 1 and 3 | TRS |
|  | 8 | l0 = 8 for CSI-RS resource 2 and 4 | TRS |
| nrofPorts | p1 | 1 for CSI-RS resource 1,2,3,4 | TRS |
| Cdm-Type | noCDM |  | TRS |
| Density CHOICE{ |  |  |  |
| three | Null |  | TRS |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  | TRS |
| } |  |  |  |

##### NZP CSI-RS for CSI Acquisition

###### CSI-RS-ResourceMapping

Table 5.4.2.4-2: CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 001000 | K0 = 6, row3, |  |
| } |  |  |  |
| nrofPorts | P2 |  |  |
| firstOFDMSymbolInTimeDomain | 13 | I0 = 13 |  |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

##### ZP CSI-RS for CSI Acquisition

###### CSI-RS-ResourceMapping

Table 5.4.2.4-3: ZP CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 | K0 = 4 | FR1 |
| row4 | 100 | K0=8 | FR2 |
| } |  |  |  |
| nrofPorts | P4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 | I0 = 9 | FR1 |
|  | 13 | l0 = 13 | FR2 |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | ZP CSI-FrequencyOccupation |  |  |
| } |  |  |  |

###### CSI-ResourcePeriodicityAndOffset

Table 5.4.2.4-4: ZP CSI-ResourcePeriodicityAndOffset

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 1 |  | FR1_FDD |
| slot10 | 1 |  | FR1_TDD |
| slot8 | 1 |  | FR2 |
| } |  |  |  |

###### p-ZP-CSI-RS-ResourceSet

Table 5.4.2.4-5: p-ZP-CSI-RS-ResourceSet

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-ZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| zp_CSI-RS_ResourceSetId | 0 |  |  |
| zp-CSI-RS-ResourceIdList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId{ | 1 entry |  |  |
| ZP-CSI-RS-ResourceId[1] | 0 | entry 1 ZP CSI-RS resource 1 |  |
| } |  |  |  |
| } |  |  |  |

##### CSI-IM Configuration

###### CSI-IM-Resource

Table 5.4.2.4-6: CSI-IM-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceId | CSI-IM-ResourceId |  |  |
| csi-IM-ResourceElementPattern CHOICE { |  |  |  |
| pattern0 SEQUENCE { |  |  | FR1 |
| subcarrierLocation-p0 | s4 |  |  |
| symbolLocation-p0 | 9 |  |  |
| } |  |  |  |
| pattern1 SEQUENCE { |  |  | FR2 |
| subcarrierLocation-p1 | s8 |  |  |
| symbolLocation-p1 | 13 |  |  |
| } |  |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| periodicityAndOffset | Not present |  |  |
| } |  |  |  |

###### CSI-IM-ResourceId

Table 5.4.2.4-7: CSI-IM-ResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceId | 0 |  |  |

###### CSI-IM-ResourceSet

Table 5.4.2.4-8: CSI-IM-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceSetId | CSI-IM-ResourceSetId |  |  |
| csi-IM-Resources SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) { | 1 entry |  |  |
| CSI-IM-ResourceId[1] | CSI-IM-ResourceId |  |  |
| } |  |  |  |
| } |  |  |  |

###### CSI-IM-ResourceSetId

Table 5.4.2.4-9: CSI-IM-ResourceSetId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSetId | 0 |  |  |

###### CSI-IM-ResourceConfig

Table 5.4.2.4-10: CSI-IM-ResourceConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId { | 1 entry |  |  |
| csi-IM-ResourceSetId[0] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

##### CSI-MeasConfig

Table 5.4.2.4-11: CSI-MeasConfig

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n entries | n=5 for FR1 n=7 for FR2 |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for CSI Acquisition | entry 5 CSI-RS resource 5 |  |
| NZP-CSI-RS-Resource[6] | CSI-RS-Resource for beam refinement | entry 6 CSI-RS resource 6 | FR2 |
| NZP-CSI-RS-Resource[7] | CSI-RS-Resource for beam refinement | entry 7 CSI-RS resource 7 | FR2 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | n entries | n=2 for FR1 n=3 for FR2 |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for CSI Acquisition | entry 2 |  |
| NZP-CSI-RS-ResourceSet[3] | NZP-CSI-RS-ResourceSet for beam refinement | entry 3 | FR2 |
| } |  |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { | 1 entry |  |  |
| CSI-IM-Resource[1] | CSI-IM-Resource |  |  |
| } |  |  |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet { | 1 entry |  |  |
| CSI-IM-ResourceSet[1] | CSI-IM-ResourceSet |  |  |
| } |  |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | n entries | n=3 for FR1 n=4 for FR2 |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig for TRS | entry 1 |  |
| CSI-ResourceConfig[2] | CSI-ResourceConfig for CSI Acquisition | entry 2 |  |
| CSI-ResourceConfig[3] | CSI-IM-ResourceConfig | entry 3 |  |
| CSI-ResourceConfig[4] | CSI-ResourceConfig for beam refinement | entry 4 | FR2 |
| } |  |  |  |
| } |  |  |  |

##### CSI-ReportConfig

Table 5.4.2.4-12: CSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| carrier | ServCellIndex |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId for CSI Acquisition |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId for CSI-IM |  |  |
| nzp-CSI-RS-ResourcesForInterference | not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| reportSlotConfig | CSI-ReportPeriodicityAndOffset |  |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource { |  |  |  |
| PUCCH-CSI-Resource [1] | PUCCH-CSI-Resource |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | NULL |  | FR1, FR2 |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| cqi-FormatIndicator | widebandCQI |  |  |
| pmi-FormatIndicator | widebandPMI |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands7 | '1111111'B |  | FR1 |
| subbands9 | ‘111111111’B |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | CodebookConfig |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | not present |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table2 |  | FR1 |
|  | table1 |  | FR2 |
| subbandSize | value2 |  |  |
| non-PMI-PortIndication | Not present |  |  |
| } |  |  |  |

###### CSI-ReportPeriodicityAndOffset

Table 5.4.2.4-13: CSI-ReportPeriodicityAndOffset

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportPeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots10 | 9 |  | FR1_TDD |
| slots5 | 0 |  | FR1_FDD |
| slot8 | 3 |  | FR2 |
| } |  |  |  |

###### PUCCH-CSI-Resource

Table 5.4.2.4-14: PUCCH-CSI-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-CSI-Resource ::= SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-id |  | FR1, FR2 |
| pucch-Resource | 8 |  | FR1 |
|  | 14 |  | FR2 |
| } |  |  |  |

###### CodebookConfig

Table 5.4.2.4-15: CodebookConfig

| Derivation Path: Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| subType CHOICE { |  |  |  |
| typeI-SinglePanel SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| two SEQUENCE { |  |  |  |
| twoTX-CodebookSubsetRestriction | '010000'B |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | ‘11111111’B |  |  |
| } |  |  |  |
| } |  |  |  |
| codebookMode | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### Physical layer parameters

Table 5.4.2.4-16: Phy sical layer parameters for DCI format 1_1

| Derivation Path: Table 4.3.6.1.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PUCCH resource indicator | PUCCH-ResourceId[1] = 8 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘110’B | SUBBAND_CQI |

##### PUSCH-TimeDomainResourceAllocationList

Table 5.4.2.4-17: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: Table 5.4.2.0-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 2 entries |  | SUBBAND_CQI |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| startSymbolAndLength | 55 | Start symbol(S)=0, Length(L)=12 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| startSymbolAndLength | 55 | Start symbol(S)=0, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUBBAND_CQI | For aperiodic subband CQI reporting under fading condition test cases |

#### 5.4.2.5	Message contents for PMI reporting requirements

## <<< Skip unchanged sections >>>

## <<< END OF CHANGES >>>