3GPP TSG-RAN5 Meeting #91-e	R5-213402

# Electronic Meeting, 17th May 2021 – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2275 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of unrestricted nr PDN parameter for RRC Connection Management Procedures TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To align with the contents, including tables, for the generic procedure parameter for E-UTRA/EPC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Pre test conditions have been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The preamble will not be aligned with the generic procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.2 and 8.1.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.1.1.3.2	RRC connection release / Redirection from NR to E-UTRA

8.1.1.3.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an RRCRelease message including an IE redirectionInformation with E-UTRA frequency }

then { UE enters RRC_IDLE state on E-UTRA frequency included in IE redirectionInformation }

}

8.1.1.3.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clause 5.3.8.3, TS 38.304, clause 5.2.6.

[TS 38.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this clause 60ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	stop timer T320, if running;

1>	if the RRCRelease message includes redirectedCarrierInfo indicating redirection to eutra:

2>	if cnType is included:

3>	after the cell selection, indicate the available CN Type(s) and the received cnType to upper layers;

NOTE:	Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cnType, is up to UE implementation.

[TS 38.304, clause 5.2.6]

At reception of RRCRelease message to transition the UE to RRC_IDLE or RRC_INACTIVE, UE shall attempt to camp on a suitable cell according to redirectedCarrierInfo if included in the RRCRelease message used for this transition. If the UE cannot find a suitable cell, the UE is allowed to camp on any suitable cell of the indicated RAT. If the RRCRelease message does not contain the redirectedCarrierInfo, UE shall attempt to select a suitable cell on an NR carrier. If no suitable cell is found according to the above, the UE shall perform cell selection using stored information in order to find a suitable cell to camp on.

When returning to RRC_IDLE state after UE moved to RRC_CONNECTED state from camped on any cell state, UE shall attempt to camp on an acceptable cell according to redirectedCarrierInfo, if included in the RRCRelease message. If the UE cannot find an acceptable cell, the UE is allowed to camp on any acceptable cell of the indicated RAT. If the RRCRelease message does not contain redirectedCarrierInfo UE shall attempt to select an acceptable cell on an NR frequency. If no acceptable cell is found according to the above, the UE shall continue to search for an acceptable cell of any PLMN in state any cell selection.

8.1.1.3.2.3	Test description

8.1.1.3.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell. System information combination NR-6 as defined in TS 38.508 -1 [4] clause 4.4.3.1.2 is used

-	E-UTRA Cell 1 is a suitable neighbour cell. System information combination 31 as defined in TS 38.508 [7], subclause 4.4.3.1.1

-	The parameters settings and power levels for NR Cell 1, E-UTRA Cell 1 are selected to ensure that camping on NR Cell 1 is guaranteed and no cell re-selection to E-UTRA Cell 1 can take place (E -UTRA Cell 1 priority is lower than serving NR Cell 1).

UE:

-	None

Preamble:

-	With E-UTRA Cell 1 "Serving cell" and NR Cell 1 "Non-suitable "Off" cell", the UE is brought to state 1E-A, RRC_IDLE using generic procedure parameters Connectivity (E-UTRA/EPC) and Unrestricted nr PDN (On), in accordance with the procedure described in TS 38.508-1 [4], clause 4.5.2.

-	the UE is switched-off

-	With E-UTRA Cell 1 and NR Cell 1 power levels set according to T0, the UE is brought to state 3N-A as defined in  TS 38.508-1 [4], clause 4.4A.2.

8.1.1.3.2.3.2	Test procedure sequence

Tables 8.1.1.3.2.3.2-1/ 8.1.1.3.2.3.2-1A illustrates the downlink power levels to be applied for NR Cell 1, E-UTRA Cell 1 for the test execution.

Table 8.1.1.3.2.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz |  | -91 | The power levels are such that camping on NR Cell 1 is guaranteed. |
|  | SS/PBCH SSS EPRE | dBm/SCS | -85 | - |  |

Table 8.1.1.3.2.3.2-1A: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz |  | -96 | The power levels are such that camping on NR Cell 1 is guaranteed. |
|  | SS/PBCH SSS EPRE | dBm/SCS | -82 | - |  |

Table 8.1.1.3.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCRelease message (IE redirectionInformation including ARFCN-ValueEUTRA of E-UTRA Cell 1). | <-- | NR RRC: RRCRelease | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 is performed and the UE is camped on E-UTRAN Cell 1, with connected without release‘’? | - | - | 1 | - |
| 3 | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508 [7]. | - | - | - | - |

8.1.1.3.2.3.3	Specific message contents

Table 8.1.1.3.2.3.3-1 RRCRelease (step 1, Table 8.1.1.3.2.3.2-2)

| Derivation Path: TS 38.508-1 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |
| eutra SEQUENCE { |  |  |  |
| eutraFrequency | EARFCN of E-UTRA Cell 1 |  |  |
| cnType-r15 | Epc |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### <<<< TEXT SKIPPED >>>>

##### 8.1.1.3.4	RRC connection release / Success / With priority information / E-UTRA

8.1.1.3.4.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state having received an RRCRelease message with the freqPriorityListEUTRA with higher priority frequency }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority EUTRA frequency }

then { UE reselects the cell which belongs to the higher priority EUTRA frequency }

}

(2)

with { UE in NR RRC_IDLE state having received an RRCRelease message with the freqPriorityListEUTRA with higher priority frequency and timer T320 }

ensure that {

when { T320 timer expires }

then { UE discards the cell reselection priority information provided by the cellReselectionPriorities and apply the cell reselection priority information broadcast in the system information to perform reselection to an E-UTRA cell }

}

8.1.1.3.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clause 5.3.8.3 and T 38.304, clause 5.2.4.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	stop timer T380, if running;

1>	stop timer T320, if running;

1>	stop timer T390, if running;

1>	if the security is not activated, perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

…

1>	if the RRCRelease message includes the cellReselectionPriorities:

2>	store the cell reselection priority information provided by the cellReselectionPriorities;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

[TS 38.304, clause 5.2.4.1]

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an NR frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCRelease unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values).

The UE shall only perform cell reselection evaluation for NR frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

In case UE receives RRCRelease with deprioritisationReq, UE shall consider current frequency and stored frequencies due to the previously received RRCRelease with deprioritisationReq or all the frequencies of NR to be the lowest priority frequency (i.e. lower than any of the network configured values) while T325 is running irrespective of camped RAT. The UE shall delete the stored deprioritisation request(s) when a PLMN selection is performed on request by NAS (TS 23.122 [9]).

NOTE:	UE should search for a higher priority layer for cell reselection as soon as possible after the change of priority. The minimum related performance requirements specified in TS 38.133 [8] are still applicable.

The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters a different RRC state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS (TS 23.122 [9]).

NOTE 2:	Equal priorities between RATs are not supported.

The UE shall not consider any black listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e. T320 in NR and E-UTRA), if configured, at inter-RAT cell (re)selection.

NOTE 3:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 38.331, clause 5.3.8.4]

The UE shall:

1>	if T320 expires:

2>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

2>	apply the cell reselection priority information broadcast in the system information.

[TS 38.304, clause 5.2.4.2]

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8].

-	For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells of equal or lower priority;

-	Otherwise, the UE shall perform measurements of NR inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 38.133 [8].

[TS 38.304, clause 5.2.4.5]

If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority NR or EUTRAN RAT/frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT

Otherwise, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority NR frequency shall be based on ranking for intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a lower priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority NR or E-UTRAN RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a lower priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/frequency shall take precedence over a lower priority RAT/frequency if multiple cells of different priorities fulfil the cell reselection criteria.

If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an NR frequency, the highest ranked cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6;

-	If the highest-priority frequency is from another RAT, the highest ranked cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

[TS 38.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs = Qmeas,s +Qhyst - Qoffsettemp Rn = Qmeas,n -Qoffset - Qoffsettemp

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3]. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2.

The cells shall be ranked according to the R criteria specified above by deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If rangeToBestCell is not configured, the UE shall perform cell reselection to the highest ranked cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

If rangeToBestCell is configured, then the UE shall perform cell reselection to the cell with the highest number of beams above the threshold (i.e. absThreshSS-BlocksConsolidation) among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell. If there are multiple such cells, the UE shall perform cell reselection to the highest ranked cell among them. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better than the serving cell according to the cell reselection criteria specified above during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

8.1.1.3.4.3	Test description

8.1.1.3.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, E-UTRA Cell 1 and E-UTRA Cell 3.

-	System information combination NR-6 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell.

-	System information combination 1 as defined in TS 36.508 [7] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	With E-UTRA Cell 1 "Serving cell",E-UTRA Cell 3 "Non-suitable "Off" cell" and NR Cell 1 "Non-suitable "Off" cell" in accordance with TS 38.508-1 [4], Table 6.2.2.1-3, the UE is brought to state RRC_IDLE using generic procedure parameters Connectivity (E-UTRA/EPC) and Unrestricted nr PDN (On), in accordance with the procedure described in TS 38.508-1 [4], clause 4.5.2. 4G GUTI and eKSI are assigned and security context established.

-	the UE is switched-off.

-	With E-UTRA Cell 1 "Non-suitable "Off" cell" and NR Cell 1 "Serving cell" in accordance with TS 38.508-1 [4], Table 6.2.2.1-3, the UE is brought to state 3N-A, NR RRC_ CONNECTED, in accordance with the procedure described in TS 38.508-1 [4], Table 4.5.2.2-2. 5G-GUTI and ngKSI are assigned and security context established.

-	The UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.1.3.4.3.2	Test procedure sequence

Table 8.1.1.3.4.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.1.3.4.3.2-1: Time instances of cell power level and parameter changes in FR1

|  | Parameter | Unit | NR Cell 1 | E-UTRA Cell 1 | E-UTRA Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | - | "off” | "off” |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | -88 | - | - |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | - | -73 | -73 | The power level values are assigned to satisfy both Threshx, low < SrxlevE-UTRACell 1 and Threshx, high <SrxlevE-UTRACell 3 but not to satisfy SrxlevNR Cell 1 < Threshserving, low |
|  | SS/PBCH SSS EPRE | dBm/SCS | -88 | - | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | - | "off” | "off” | The power level values are assigned to satisfy SrxlevNR Cell 1 > Snonintrasearch. (NOTE 1) |
|  | SS/PBCH SSS EPRE | dBm/SCS | -88 | - | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | - | -73 | "off” | The power level values are assigned to satisfy Threshx, high < SrxlevE-UTRA Cell 1. |
|  | SS/PBCH SSS EPRE | dBm/SCS | -88 | - | - |  |
| NOTE 1: Power level “Off” is defined in TS36.508 [7] Table 6.2.2.1-3. |  |  |  |  |  |  |

Table 8.1.1.3.4.3.2-2: Time instances of cell power level and parameter changes in FR2

|  | Parameter | Unit | NR Cell 1 | E-UTRA Cell 1 | E-UTRA Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | - | "off” | "off” |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | FFS | - | - |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | - | FFS | FFS | The power level values are assigned to satisfy both Threshx, low < SrxlevE-UTRACell 1 and Threshx, high <SrxlevE-UTRACell 3 but not to satisfy SrxlevNR Cell 1 < Threshserving, low |
|  | SS/PBCH SSS EPRE | dBm/SCS | FFS | - | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | - | "off” | "off” | The power level values are assigned to satisfy SrxlevNR Cell 1 > Snonintrasearch. (NOTE 1) |
|  | SS/PBCH SSS EPRE | dBm/SCS | FFS | - | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | - | FFS | "off” | The power level values are assigned to satisfy Threshx, high < SrxlevE-UTRA Cell 1. |
|  | SS/PBCH SSS EPRE | dBm/SCS | FFS | - | - |  |
| NOTE 1: Power level “Off” is defined in TS36.508 [7] Table 6.2.2.1-3. |  |  |  |  |  |  |

Table 8.1.1.3.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes NR Cell 1, E-UTRA Cell 1 and E-UTRA Cell 3 level according to the row "T1" in Table 8.1.1.3.4.3.2-1/2 | - | - | - | - |
| 2 | The SS transmits an RRCRelease message including the freqPriorityListEUTRA on NR Cell 1. | <-- | NR RRC: RRCRelease | - | - |
| 2A | Check: Does the UE transmits an RRCConnectionRequest message on E-UTRA cell 3? | --> | RRC: RRCConnectionRequest | 1 | P |
| 2B | SS transmits an RRCConnectionSetup message on E-UTRA cell 3. | <-- | RRC: RRCConnectionSetup | - | - |
| 2C | Check: Does the UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area? | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | 1 | P |
| 3 | Void |  |  |  |  |
| 3A | The SS transmits a SecurityModeCommand message. | <-- | E-UTRA RRC: SecurityModeCommand | - | - |
| 3B | The UE transmits a SecurityModeComplete message. | --> | E-UTRA RRC: SecurityModeComplete | - | - |
| 3C-3D | Steps 5-6 of TS 38.508-1 [4] Table 4.9.7.2.2-1 of the generic procedure are performed. | - | - | - | - |
| 3E | The SS transmits an RRCConnectionReconfiguration message to establish DRB #n | <-- | E-UTRA RRC: RRCConnectionReconfiguration | - | - |
| 3F | The UE transmits an RRCReconfigurationComplete message. | --> | E-UTRA RRC: RRCConnectionReconfigurationComplete | - | - |
| 3G | Void | - | - | 1 | - |
| 3GA | The SS sends one IP Packet to the UE on the default DRB associated with the first PDU session. | - | - | - | - |
| 3GB | The UE loop backs the IP packet received in step 3GA. | - | - | 1 | P |
| 3H | The SS transmits an RRConnectionRelease | <-- | E-UTRA RRC: RRCConnectionRelease | - | - |
| 4 | The SS changes NR Cell 1, E-UTRA Cell 1 and E-UTRA Cell 3 level according to the row "T2" in Table 8.1.1.3.4.3.2-1/2 | - | - | - | - |
| 5 | UE perform on the NR Cell 1 the mobility and periodic registration update procedure in TS 38.508-1[4] subclause 4.9.9.2.2 | - | - | - | - |
| 5AA | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration | - | - |
| 5AB | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 5A | Void | - | - | 1 | - |
| 5B | The SS sends one IP Packet to the UE on the default DRB associated with the first PDU session. | - | - | - | - |
| 5C | The UE loop backs the IP packet received in step 5B. | - | - | 1 | P |
| 6 | The SS transmits an RRCRelease message containing IE freqPriorityListEUTRA to update the cell reselection priority of E-UTRA Cell 1 and the timer T320. | <-- | NR RRC: RRCRelease | - | - |
| 7 | The SS changes power levels of NR Cell 1, E-UTRA Cell 1 and E-UTRA Cell 3 according to row “T3” in Table 8.1.1.3.4.3.2-1/2. | - | - | - | - |
| 8 | Check: While timer T320 is running, does the UE transmit an RRCSetupRequest message? | - | - | 2 | F |
| 9 | Void | - | - | 2 | - |
| 9A | Check: Does the UE transmits an RRCConnectionRequest message on E-UTRA cell 1? | --> | RRC: RRCConnectionRequest | 2 | P |
| 9B | SS transmits an RRCConnectionSetup message on E-UTRA cell 1. | <-- | RRC: RRCConnectionSetup | - | - |
| 9C | Check: Does the UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area? | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | 2 | P |
| 10 | The SS transmits a SecurityModeCommand message. | <-- | E-UTRA RRC: SecurityModeCommand | - | - |
| 11 | The UE transmits a SecurityModeComplete message. | --> | E-UTRA RRC: SecurityModeComplete | - | - |
| 12-13 | Steps 5-6 of TS 38.508-1 [4] Table 4.9.7.2.2-1 of the generic procedure are performed. | - | - | - | - |
| 14 | The SS transmits an RRCConnectionReconfiguration message to establish DRB #n | <-- | E-UTRA RRC: RRCConnectionReconfiguration | - | - |
| 15 | The UE transmits an RRCReconfigurationComplete message. | --> | E-UTRA RRC: RRCConnectionReconfigurationComplete | - | - |
| 16 | Void | - | - | 2 | - |
| 16A | The SS sends one IP Packet to the UE on the default DRB associated with the first PDU session. | - | - | - | - |
| 16B | The UE loop backs the IP packet received in step 16A. | - | - | 2 | P |

8.1.1.3.4.3.3	Specific message contents

Table 8.1.1.3.4.3.3-1: RRCRelease (step 1, Table 8.1.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| cellReselectionPriorities SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA { | 2 entries |  |  |
| freqPriorityListEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | ARFCN-ValueEUTRA for EUTRA Cell 1 frequency |  |  |
| cellReselectionPriority | 3 |  |  |
| cellReselectionSubPriority | Not present |  |  |
| } |  |  |  |
| freqPriorityListEUTRA[1] SEQUENCE { |  | entry 2 |  |
| carrierFreq | ARFCN-ValueEUTRA for EUTRA Cell 3 frequency |  |  |
| cellReselectionPriority | 5 |  |  |
| cellReselectionSubPriority | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| freqPriorityListNR SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR { | 1 entry |  |  |
| FreqPriorityNR[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | ARFCN-ValueNR for NR Cell 1 frequency |  |  |
| cellReselectionPriority | 4 |  |  |
| cellReselectionSubPriority | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| t320 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.3.4.3.3-2: SIB5 for NR cell 1 (preamble and all steps, Table 8.1.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 2 entries |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | ARFCN-ValueEUTRA for EUTRA Cell 1 frequency |  |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| CarrierFreqEUTRA[2] SEQUENCE { |  | entry 2 |  |
| carrierFreq | ARFCN-ValueEUTRA for EUTRA Cell 3 frequency |  |  |
| cellReselectionPriority | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.3.4.3.3-3: RRCRelease (step 6, Table 8.1.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| cellReselectionPriorities SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA { | 1 entry |  |  |
| freqPriorityListEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | ARFCN-ValueEUTRA of E-UTRA Cell 1 |  |  |
| cellReselectionPriority | 3 |  |  |
| cellReselectionSubPriority | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| freqPriorityListNR SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR { | 1 entry |  |  |
| FreqPriorityNR[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | ARFCN-ValueNR for NR Cell 1 frequency |  |  |
| cellReselectionPriority | 4 |  |  |
| cellReselectionSubPriority | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| t320 | min5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.3.4.3.3-4: RRCConnectionReconfiguration (steps 3E and 14 Table 8.1.1.3.4.3.2-3)

| Derivation Path: TS 36.508[7] Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated |  | RadioResourceConfigDedicated-DRB(n, m) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.1.3.4.3.3-5: RRCReconfiguration-SRB2-DRB (n, m) (step 5AA, Table 8.1.1.3.4.3.2-3)

Derivation Path: TS 38.508-1 [4] Table 4.8.1-1B

Table 8.1.1.3.4.3.3-6: REGISTRATION REQUEST (Step 5, Table 8.1.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS NAS message container | TRACKING AREA UPDATE REQUEST message | See Table 8.1.1.3.4.3.3-7 |  |

Table 8.1.1.3.4.3.3-7: TRACKING AREA UPDATE REQUEST (Table 8.1.1.3.4.3.3-6)

Derivation Path: TS 38.508-1 [4], Table 4.9.9.2.3-2 with condition Mapped EPS security context