3GPP TSG-RAN WG5 Meeting #91-e	R5-213408

Online, 17th May 2021 – 28th May 2021

Source:	Qualcomm Incorporated

Title:	Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE

# Document for:	Agreement

Agenda Item:	6.3.18.6

## 1.	Introduction

Standalone Non Public networks (SNPN) were introduced in Rel-16 and work is currently commencing in RAN5 for conformance specifications, which are tracking core specifications. There may be a possibility that a UE can be SNPN ONLY UE, i.e. designed to access  Standalone Network only [4]. For such UE, we may have to provide coverage for applicable legacy NR (Rel-15) test cases.

## 2.	Discussion

While RAN5 ensure coverage is provided for explicit requirements mentioned in core spec, for both SNPN and CAG based NPN, there may be an industry requirement for certification of SNPN only UE.

Following procedures are not defined in Rel-16 for a UE operating in SNPN, for ex-

- PLMN Selection

- IRAT to LTE

- CAG

- Autonomous Search

- IMS

- Emergency

- HO, Roaming

Problem 1

However, there are still several core spec mandatory requirements which may have to be tested. For example, out of existing  NR conformance specifications [1] for R15 UE’s, how many of those tests can be run and what shall be enforced vs what is optional?

Problem 2

After applicability, testability needs to be resolved. We can derive a similar resemblance from the in past RAN5 WI for Rel-13 eMTC (CAT M1) where applicable legacy LTE test cases were evaluated and if any test could be run with simple test environment change ( like broadcasting SIB1-BR on CAT M1\M2 bands) then we made it applicable, while all other tests were specifically marked NOT applicable for CAT M1 [2]

Similar approach can be taken for SNPN only UE, for applicable R15 NR test case can be run in an SNPN environment . Existing NR cells can be replaced with SNPN cells NR Cell broadcasting default SNPN IDs as indicated in TS 38.508-1 [3] Table 4.4.2-4. and with appropriate System information combination defined in TS 38.508-1 [3] clause 4.4.3.1.2.

To add, RAN5 has more R16 WI for verticals such as IIOT, NR-U or eURLLC and some of the test case in these areas might also become applicable for SNPN Only UE and needs to addressed for applicability to SNPN as well.

## 3.	Proposal/Way Forward

1. 	Explore and catalogue the applicable test cases from Rel-15 NR specifications which might be applicable to Rel-16 SNPN only UE and define applicability.

2. 	Explore the testability aspects without introducing new test case for test cases in 1) above.

## 4.	References

[1]	TS 38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol

[2]	 TS 36.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; EUTRA; User Equipment (UE) conformance specification; Part 1: Protocol

[3]	TS 38.508-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception User Equipment (UE) conformance specification;

[4]	TS 38.300: NR; NR and NG-RAN Overall description; Stage-2; Rel-16 (cl 16.6.1)