3GPP TSG-RAN WG5 Meeting #91-e	R5-213431

# Electronic Meeting, 17th May – 28th May 2021

Agenda Item:	6.6.9.9

Source:	MCC TF160

Title:	IUT and SS log content for MCPTT on-network test cases

Document for:	Discussion

# 1	Introduction

Recently the verification submission of the first MCPTT test case 5.1 was made with a System Simulator (SS) using the MCX IPCAN test model, see R5s210384 [7]. Analysis of this verification and discussions with the submitter highlighted the need to clarify/confirm a couple of aspects with regards to what the Implementation Under Test (IUT) is for MCPTT conformance, and the content of the SS log files.

NOTE:	The focus of the paper is on MCPTT client, and on-network scenarios. However, the topics also encompass MCDATA and MCVIDEO. So, in the present paper, one could replace MCPTT by MCS or MCX.

# 2	IUT and SUT

## 2.1	Definitions

In the context of conformance testing as specified in ISO/IEC 9646-1 [6], the "Implementation Under Test" (IUT) is a protocol implementation considered as an object for testing. This means that the test process will focus on verifying the compliance of this protocol implementation (IUT) with requirements set up in the related base standard. An IUT normally is implemented in a "System Under Test" (SUT). For testing, a SUT is connected to a test system over at least a single interface.

An abstract protocol tester provides the test behaviour for testing an IUT. To access an IUT, the corresponding abstract protocol tester needs to use lower layers to establish a proper connection to the system under test (SUT) over a physical link (Lower layers link). See Figure 2.1-1.

Figure 2.1-1: Conformance testing the IUT

When TTCN-3 is used to build the test system, the previous figure can be further refined according to the generic TTCN-3 test architecture depicted in Figure 2.1-2.

Figure 2.1-2: TTCN-3 based conformance test system

NOTE: The test system can also be described as containing two main building blocks: the “TTCN-3” (TTCN-3 test components) and the “System Simulator (SS)” (other components shown in the test system box).

## 2.2	MCPTT

The MCPTT User Equipment (UE) protocol conformance test cases are specified in prose in TS 36.579-1 [1], TS 36.579-2 [2] and TS 36.579-4 [4].

The MCPTT UE conformance test cases specify the protocol message exchanges and contents at the MCX layers (SIP, HTTP), at the EPC NAS and at the E-UTRA RRC layers.

In the MCPTT conformance testing test environment defined in the aforementioned specifications, our understanding is that the IUT is the MCPTT UE, i.e. a physical mobile device implementing the 3GPP (LTE = E-UTRA+EPC) protocol stack, on which an MCPTT client application is installed.

NOTE: The term "MCPTT UE" is specified in TS 22.179 [5].

The IUT and the SS communicate, one with the other, over the LTE radio interface. See e.g. TS 36.579-1 [] Figure 4.2.1.

Figure 4.2.1: Testing the MCPTT Client (on-network)

Statement#1: IUT = SUT = MCPTT UE = (MCPTT Client under test + LTE UE under test)

# 3	On-network test models

Two test models are specified in TS 36.579-5:

- MCX EUTRA test model:

  - TTCN-3 implements the MCX (HTTP, SIP), EPC NAS and E-UTRA RRC layers.

  - SS implements the lower layers (TCP/UDP/IP, E-UTRA L2/L1).

  - the original test model specified at RAN5.

- MCX IPCAN test model:

  - TTCN-3 implements the MCX (HTTP, SIP) layers.

  - SS implements the lower layers (TCP/UDP/IP, EPC NAS, E-UTRA RRC/L2/L1).

  - a new test model added recently as an exceptional measure (and strictly limited to MCX) because of unavailability of SS implementations supporting the MCX EUTRA test model.

# 4	TTCN-3 test case verification

Once the MCPTT TTCN-3 test case implementation has been delivered by MCC TF160, a formal process of verification of the MCPTT TTCN-3 implementation can take place.

The general RAN5 process for TTCN-3 test case verification is defined in RAN5 PRD12 [8]. It requires the involvement of multiple parties to bring in the necessary components of the test system, mainly:

- MCC TF160: developing and delivering the implementation of the protocol conformance test cases in TTCN-3 language.

- SS Vendor(s) (also known as Test Equipment (TE) Vendor(s)): providing their SS implementation (usually involving hardware components) on which the TTCN-3 test cases are integrated and executed.

- UE Vendor(s): providing the physical (hardware) UE (IUT).

In a nutshell, the SS Vendors are responsible for getting hold of UE(s), running the TTCN-3 test cases on their SS against the UE, and submitting a TTCN Change Request (CR) for formal TTCN-3 test case verification (including the list of required TTCN-3 code changes and an SS log file exhibiting a successful PASS execution of the TTCN-3 test case).

With regards to the SS log file, RAN5 PRD12 [8] specifies several requirements, and in particular:

SS logs requirement for test case agreement

The log shall exhibit (as a minimum) all protocol message exchanges between the UE and the SS and all protocol message contents, generated during TTCN test case execution, that matches with the test case prose description.

Statement#2: Regardless of the test model used, the SS log file of an MCPTT test case verification shall display the protocol message exchanges and contents for the MCPTT (HTTP, SIP), EPC NAS and E-UTRA RRC layers.

Statement#3: Even with the MCX IPCAN test model, the SS shall comply with EPC NAS and E-UTRA RRC protocol message exchanges and contents exactly as specified in TS 36.579-series specifications.

A couple of additional remarks:

- The MCX IPCAN test model had been proposed and endorsed at RAN5#88-e in R5-204094 [9] and at TSG RAN#89-e in RP-201637 [10]. At that time, discussions took place about amending the MCPTT test specifications to allow IPCAN-based SS EPC/E-UTRA implementations to possibly deviate from the protocol message exchanges and contents specified in TS 36.579-series specifications, in order to facilitate use of “off-the-shelf” eNB emulators. But no follow-on CR has been brought to RAN5 to date.

- With the MCX IPCAN test model, there is no “automated” way to check and assess compliance of NAS & RRC protocol message exchanges and contents. Only a visual inspection of the SS logs can confirm that the UE+SS implementations match with the test case prose. This is true for both RAN5 verification and Certification Body validation.

# 6	Conclusion

RAN5 is kindly requested to acknowledge the following statements:

Statement#1: IUT = SUT = MCPTT UE = (MCPTT Client under test + LTE UE under test)

Statement#2: Regardless of the test model used, the SS log file of an MCPTT test case verification shall display the protocol message exchanges and contents for the MCPTT (HTTP, SIP), EPC NAS and E-UTRA RRC layers.

Statement#3: Even with the MCX IPCAN test model, the SS shall comply with EPC NAS and E-UTRA RRC protocol message exchanges and contents exactly as specified in TS 36.579-series specifications.

In case any of those statements need to be amended, RAN5 is kindly requested to make the relevant updates in the TS 36.579-series specifications.

# 7	Way forward

Although RAN5 confirmed that the statements of section 6 are correct according to the current test specifications (and RAN5 PRD12), after discussions among the RAN5 MCX proponents, amendments are proposed to be made. Amendments are summarized in the following way forward:

IUT and SUT:

| Test Model in 36.579-5 | SUT | IUT | MCX signalling in 36.579-x (HTTP, SIP, ...) | LTE signalling in 36.579-x & 36.508 (NAS, RRC) | Comments |
| --- | --- | --- | --- | --- | --- |
| MCX EUTRA | UE | UE | normative | normative |  |
| MCX IPCAN | UE | UE MCX Client | normative | normative informative | For the MCX IPCAN test model design and system interface specification, the LTE signalling specified in TS 36.579-series and TS 36.508 is considered as the reference IPCAN behaviour. |

SS log files:

RAN5 still requests the LTE (NAS, RRC) logging to be provided by the SS, but only for information.

- EPC NAS + E-UTRA RRC message names + message contents.

- Note 1: There is no requirement on TF160 to check this LTE level logging.

- Note 2: This LTE level logging is only there to give some evidence that the MCX test case was indeed executed on top of an LTE IPCAN.

RAN5 specifications:

The RAN5 MCX test specifications must be updated to reflect and document the above way forward.

# 8	References

[1]	3GPP TS 36.579-1: "Mission Critical (MC) services over LTE; Part 1: Common test environment"

[2]	3GPP TS 36.579-2: "Mission Critical (MC) services over LTE; Part 2: Mission Critical Push To Talk (MCPTT) User Equipment (UE) Protocol conformance specification"

[3]	3GPP TS 36.579-4: "Mission Critical (MC) services over LTE; Part 4: Test Applicability and Implementation Conformance Statement (ICS) proforma specification"

[4]	3GPP TS 36.579-5: "Mission Critical (MC) services over LTE; Part 5: Abstract test suite (ATS)"

[5]	3GPP TS 22.179: "Mission Critical Push To Talk (MCPTT) over LTE; Stage 1"

[6]	ISO/IEC 9646 (all parts): "Information technology - Open Systems Interconnection - Conformance testing methodology and framework".

[7]	R5s210384: “Addition of MCPTT test case 5.1 to the MCX ATS”

[8]	RAN5 PRD12: “Process for CR Agreement and Maintenance of TTCN”

[9]	R5-204094: “MCPTT IPCAN test environment”

[10]	RP-201637: “IPCAN-based MCS test model”