3GPP TSG-RAN5 Meeting #91-e	R5-213445

# Online,  17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1888 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to PDCP Config |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When RLC bearer is in UM and is used for relatively high data traffic (e.g. IMS voice call tests) there is no mechanism apart from HARQ retransmissions to recover from data packet loss. To increase robustness, PDCP layer should be given opportunity to deliver all the so far received SDUs to upper layer even if some were missing to avoid the upper layer being stuck and waiting indefinitely for lossless reception. This is a problem especially during the IMS voice/video call tests where lossless traffic exchange is not essential but rather that flow stays continous. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added 80ms value to t-Reordering IE with condition UM (For RLC UM DRB). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It can cause higher test instability in long IMS tests (7.2x and 7.3x in TS 34.229-5). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3-99 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-212776 R5-212776r1 - Corrected the removal and addition of t-Reordering in ‘Table 4.6.3-99: PDCP-Config’. - Removed the visible cell border for t-Reordering. R5-213445 : This is the final version of R5-212776r1 |  |  |  |  |  |  |  |  |

### 4.6.3	Radio resource control information elements

#### –	PDCP-Config

Table 4.6.3-99: PDCP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| discardTimer | infinity |  |  |
| pdcp-SN-Size-UL | len18bits |  |  |
| pdcp-SN-Size-DL | len18bits |  |  |
| headerCompression CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| integrityProtection | Not present |  |  |
| statusReportRequired | true | AM is default |  |
|  | Not present |  | UM |
| outOfOrderDelivery | Not present |  |  |
| } |  |  |  |
| drb | Not present |  | SRB, Split_SRB |
| moreThanOneRLC | Not present |  |  |
| moreThanOneRLC SEQUENCE { |  |  | Split, Split_SRB, NR_split |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | CellGroupId |  |  |
|  | 0 | MCG path | Split_SRB, NR_split |
| logicalChannel | LogicalChannelIdentity |  |  |
| } |  |  |  |
| ul-DataSplitThreshold | infinity |  |  |
|  | Not present |  | Split_SRB |
| pdcp-Duplication | false |  |  |
|  | Not present | one UL path | Split_SRB |
| } |  |  |  |
| t-Reordering | Not present |  |  |
|  | 80ms |  | UM |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Split | More than one RLC |
| SRB | SRB |
| UM | RLC UM DRB |
| Split_SRB | SRB with more than one RLC |
| NR_split | MCG and split for NR-DC. |