3GPP TSG RAN WG5 Meeting #91-e	R5-213451

# Electronic Meeting, 17th - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1802 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction and editorials to default message content specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A various issues have been found in the default message content specification - SEQUENCE OF not defined as per PRD13 - Wrong indentation and missing closing brackets - Redundant spaces - Visible row borders when different values of the same element are provided - TAB used for indentation instead of spaces |  |  |  |  |  |  |  |  |
| Summary of change: |  | Various issues have been corrected - SEQUENCE OF not defined as per PRD13 - Wrong indentation and missing closing brackets - Redundant spaces - Visible row borders when different values of the same element are provided - TAB used for indentation instead of spaces |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Low quality of 3GPP specs. Possible confusion in regard to default message implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3, 5.4.2.4, 5.4.2.5, 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The present CR is the result of R5-212157 followed by one intermediate revision (r1) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.6.3	Radio resource control information elements

[Text skipped here]

#### –	BWP-DownlinkCommon

Table 4.6.3-10: BWP-DownlinkCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon |  |  |
|  | PDCCH-ConfigCommon with condition SCell_add |  | SCell_Add |
|  | PDCCH-ConfigCommon with condition InitialBWP_SIB |  | InitialBWP_SIB |
| } |  |  |  |
| pdcch-ConfigCommon | Not present |  | BWP-Id1 |
| pdsch-ConfigCommon CHOICE { |  |  |  |
| setup | PDSCH-ConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| InitialBWP_SIB | Configured via DownlinkConfigCommonSIB |
| BWP-Id1 | Additional BWP 1 |

#### –	BWP-DownlinkDedicated

Table 4.6.3-11: BWP-DownlinkDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setu | PDSCH-Config |  |  |
| } |  |  |  |
| sps-Config | Not present |  |  |
| radioLinkMonitoringConfig | Not present |  |  |
| sps-ConfigToAddModList-r16 | Not present |  |  |
| sps-ConfigToReleaseList-r16 | Not present |  |  |
| sps-ConfigDeactivationStateList-r16 | Not present |  |  |
| beamFailureRecoverySCellConfig-r16 | Not present |  |  |
| sl-PDCCH-Config-r16 | Not present |  |  |
| sl-PDCCH-Config-r16 CHOICE { |  |  | SIDELINK |
| setup | PDCCH-Config |  |  |
| } |  |  |  |
| sl-V2X-PDCCH-Config-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SIDELINK | Used for sidelink communication |

[Text skipped here]

#### 5.4.2.4	Message contents for CQI reporting requirements

[Text skipped here]

###### CSI-IM-ResourceId

Table 5.4.2.4-7: CSI-IM-ResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceId | 0 |  |  |

###### CSI-IM-ResourceSet

Table 5.4.2.4-8: CSI-IM-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceSetId | CSI-IM-ResourceSetId |  |  |
| csi-IM-Resources SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) { | 1 entry |  |  |
| CSI-IM-ResourceId[1] | CSI-IM-ResourceId | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

###### CSI-IM-ResourceSetId

Table 5.4.2.4-9: CSI-IM-ResourceSetId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSetId | 0 |  |  |

###### CSI-IM-ResourceConfig

Table 5.4.2.4-10: CSI-IM-ResourceConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId { | 1 entry |  |  |
| csi-IM-ResourceSetId[0] | 0 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

##### CSI-MeasConfig

Table 5.4.2.4-11: CSI-MeasConfig

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n entries | n=5 for FR1 n=7 for FR2 |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for CSI Acquisition | entry 5 CSI-RS resource 5 |  |
| NZP-CSI-RS-Resource[6] | CSI-RS-Resource for beam refinement | entry 6 CSI-RS resource 6 | FR2 |
| NZP-CSI-RS-Resource[7] | CSI-RS-Resource for beam refinement | entry 7 CSI-RS resource 7 | FR2 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | n entries | n=2 for FR1 n=3 for FR2 |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for CSI Acquisition | entry 2 |  |
| NZP-CSI-RS-ResourceSet[3] | NZP-CSI-RS-ResourceSet for beam refinement | entry 3 | FR2 |
| } |  |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { | 1 entry |  |  |
| CSI-IM-Resource[1] | CSI-IM-Resource |  |  |
| } |  |  |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet { | 1 entry |  |  |
| CSI-IM-ResourceSet[1] | CSI-IM-ResourceSet | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | n entries | n=3 for FR1 n=4 for FR2 |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig for TRS | entry 1 |  |
| CSI-ResourceConfig[2] | CSI-ResourceConfig for CSI Acquisition | entry 2 |  |
| CSI-ResourceConfig[3] | CSI-IM-ResourceConfig | entry 3 |  |
| CSI-ResourceConfig[4] | CSI-ResourceConfig for beam refinement | entry 4 | FR2 |
| } |  |  |  |
| } |  |  |  |

##### CSI-ReportConfig

Table 5.4.2.4-12: CSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| carrier | ServCellIndex |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId for CSI Acquisition |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId for CSI-IM |  |  |
| nzp-CSI-RS-ResourcesForInterference | not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| reportSlotConfig | CSI-ReportPeriodicityAndOffset |  |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource { | 2 entry |  |  |
| PUCCH-CSI-Resource [1] | PUCCH-CSI-Resource | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | NULL |  | FR1, FR2 |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| cqi-FormatIndicator | widebandCQI |  |  |
| pmi-FormatIndicator | widebandPMI |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands7 | '1111111'B |  | FR1 |
| subbands9 | ‘111111111’B |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | CodebookConfig |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | not present |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table2 |  | FR1 |
|  | table1 |  | FR2 |
| subbandSize | value2 |  |  |
| non-PMI-PortIndication | Not present |  |  |
| } |  |  |  |

###### CSI-ReportPeriodicityAndOffset

Table 5.4.2.4-13: CSI-ReportPeriodicityAndOffset

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportPeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots10 | 9 |  | FR1_TDD |
| slots5 | 0 |  | FR1_FDD |
| slot8 | 3 |  | FR2 |
| } |  |  |  |

###### PUCCH-CSI-Resource

Table 5.4.2.4-14: PUCCH-CSI-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-CSI-Resource ::= SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-id |  | FR1, FR2 |
| pucch-Resource | 8 |  | FR1 |
|  | 14 |  | FR2 |
| } |  |  |  |

###### CodebookConfig

Table 5.4.2.4-15: CodebookConfig

| Derivation Path: Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| subType CHOICE { |  |  |  |
| typeI-SinglePanel SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| two SEQUENCE { |  |  |  |
| twoTX-CodebookSubsetRestriction | '010000'B |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | ‘11111111’B |  |  |
| } |  |  |  |
| } |  |  |  |
| codebookMode | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 5.4.2.5	Message contents for PMI reporting requirements

[Text skipped here]

###### CSI-IM-ResourceConfig

Table 5.4.2.5-11: CSI-IM-ResourceConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId { | 1 entry |  |  |
| csi-IM-ResourceSetId[0] | 0 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | aperiodic |  |  |
| } |  |  |  |

##### CSI-MeasConfig

Table 5.4.2.5-12: CSI-MeasConfig

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n entries | n=5 for FR1 n=7 for FR2 |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for CSI Acquisition | entry 5 CSI-RS resource 5 |  |
| NZP-CSI-RS-Resource[6] | CSI-RS-Resource for beam refinement | entry 6 CSI-RS resource 6 | FR2 |
| NZP-CSI-RS-Resource[7] | CSI-RS-Resource for beam refinement | entry 7 CSI-RS resource 7 | FR2 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | n entries | n=2 for FR1 n=3 for FR2 |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for CSI Acquisition | entry 2 |  |
| NZP-CSI-RS-ResourceSet[3] | NZP-CSI-RS-ResourceSet for beam refinement | entry 3 | FR2 |
| } |  |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { | 1 entry |  |  |
| CSI-IM-Resource[1] | CSI-IM-Resource |  |  |
| } |  |  |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet { | 1 entry |  |  |
| CSI-IM-ResourceSet[1] | CSI-IM-ResourceSet |  |  |
| } |  |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | n entries | n=3 for FR1 n=4 for FR2 |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig for TRS | entry 1 |  |
| CSI-ResourceConfig[2] | CSI-ResourceConfig for CSI Acquisition | entry 2 |  |
| CSI-ResourceConfig[3] | CSI-IM-ResourceConfig | entry 3 |  |
| CSI-ResourceConfig[4] | CSI-ResourceConfig for beam refinement | entry 4 | FR2 |
| } |  |  |  |
| } |  |  |  |

##### CSI-ReportConfig

Table 5.4.2.5-13: CSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| carrier | ServCellIndex |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId for CSI Acquisition |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId for CSI-IM |  |  |
| nzp-CSI-RS-ResourcesForInterference | not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList SEQUENCE { | 2 entries |  |  |
| reportSlotOffsetList[1] | 4 | entry 1 | FR1_FDD_4Tx |
|  | 5 |  | FR1_FDD_8Tx |
|  | 8 |  | FR1_TDD |
|  | 6 |  | FR2 |
| reportSlotOffsetList[2] | 4 | entry 2 | FR1_FDD_4Tx |
|  | 5 |  | FR1_FDD_8Tx |
|  | 8 |  | FR1_TDD |
|  | 6 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | NULL |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| cqi-FormatIndicator | widebandCQI |  |  |
| pmi-FormatIndicator | widebandPMI |  |  |
| csi-ReportingBand CHOICE { |  |  | FR1 |
| subbands7 | '1111111'B |  |  |
| } |  |  |  |
| csi-ReportingBand CHOICE { |  |  | FR2 |
| subbands9 | '111111111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | CodebookConfig |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | not present |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table1 |  |  |
| subbandSize | value2 |  |  |
| non-PMI-PortIndication | Not present |  |  |
| } |  |  |  |

[Text skipped here]

### 7.3.1	Radio resource control information elements for RRM

As defined in clause 4.6.3 with the following exceptions:

[Text skipped here]

#### –	PUSCH-TimeDomainResourceAllocationList

Table 7.3.1-22: PUSCH-TimeDomainResourceAllocationList-BM

| Derivation Path: Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 1 entriy | same number of entries as reportSlotOffsetList in Table 7.3.1-12G |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k2 | 4 | Same with k2 configrued in reportSlotOffsetList in Table 7.3.1-12G |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| } |  |  |  |