3GPP TSG-RAN5 Meeting #91-e	R5-213481

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2253 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR RRC testcase 8.1.5.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As agreed in R5-206331, PDU SESSSION ESTABLISHEMENT ACCEPT message contents can be simplified: In the PDU Session Establishment Accept message the IE MappedEPSBearerContext is included based on whether the UE indicates the support for s1 capability. This adds unnecessary complexity to the testcase, and adds no value as MappedEPSBearerContet has no impact on the test purpose. The tetscases can be optimized by removing the MappedEpsBearerContext from the PDU Session Establishment Accept. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that the IE Mapped EPS Bearer Contexts not be included in the PDU Session Establishment Accept for the Preamble for these two testcases. In the table Table 8.1.5.4.1.3.3-4: PDU SESSION ESTABLISHMENT ACCEPT (Preamble) introduced a row to set Mapped EPS bearer contexts to Not Present. Introduced Tables Table 8.1.5.4.1.3.3-5 and Table 8.1.5.4.1.3.3-6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR: R5s210370 r1: Newly defined Reference QoS Flows in Table 8.1.5.4.1.3.3-7 and Table 8.1.5.4.1.3.3-8 are referred in Table 7.1.4.1.3.3-4 This document is final version of R5-213118 and one revision |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

##### 8.1.5.4.1	Counter check / Reception of CounterCheck message by the UE

8.1.5.4.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state with two DRBs established and having received 10 data packets on each DRB }

ensure that {

when { UE receives CounterCheck message with drb-CountMSB-InfoList included for DRB3(not established) and DRB1 (with the most significant bits of the count different from the actual COUNT)}

then { UE sends CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 in drb-CountInfoList IE for each DRB established, and by including the drb-Identity, the count-Uplink and the count-Downlink with the most significant bits set identical to the corresponding values in the drb-CountMSB-InfoList and the least significant bits set to zero for the DRB that is not established }

}

8.1.5.4.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clause 5.3.6.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.6.3]

Upon receiving the CounterCheck message, the UE shall:

1>	for each DRB that is established:

2>	if no COUNT exists for a given direction (uplink or downlink) because it is a uni-directional bearer configured only for the other direction:

3>	assume the COUNT value to be 0 for the unused direction;

2>	if the drb-Identity is not included in the drb-CountMSB-InfoList:

3>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 (specified in TS 38.323 [5]), respectively;

2>	else if, for at least one direction, the most significant bits of the COUNT are different from the value indicated in the drb-CountMSB-InfoList:

3>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 (specified in TS 38.323 [5]), respectively;

1>	for each DRB that is included in the drb-CountMSB-InfoList in the CounterCheck message that is not established:

2>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink with the most significant bits set identical to the corresponding values in the drb-CountMSB-InfoList and the least significant bits set to zero;

1>	submit the CounterCheckResponse message to lower layers for transmission upon which the procedure ends.

8.1.5.4.1.3	Test description

8.1.5.4.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in  the NR cell.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A on NR Cell 1 with atleast one PDU session active according to TS 38.508-1 [4], clause 4.4A.3 Table 4.4A.3-1 and Test Loop Function (On) with UE test loop mode B is established. 2 DRBs are configured where DRB1 is defined as default DRB.

8.1.5.4.1.3.2	Test procedure sequence

Table 8.1.5.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits 10 IP packets on each DRB (using QFI2 for DRB1 and QFI5 for DRB2) | - | - | - | - |
| 2 | The UE loops back the IP packets received in step 1 | - | - | - | - |
| 3 | The SS transmits CounterCheck message with drb-Identity not included in the drb-CountMSB-InfoList | <-- | NR RRC: CounterCheck | - | - |
| 4 | Check: Does the UE send CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 in drb-CountInfoList IE for each DRB established? | --> | NR RRC: CounterCheckResponse | 1 | P |

8.1.5.4.1.3.3	Specific message contents

Table 8.1.5.4.1.3.3-1: RRCReconfiguration (Preamble, Table 8.1.5.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig-2DRBs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-2DRBs | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.5.4.1.3.3-2: RadioBearerConfig-2DRBs (Preamble, Table 8.1.5.4.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-132 and condition SRB2 and DRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 2 entries |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config::= SEQUENCE { |  |  |  |
| pdu-Session | PDU session ID | Same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |
| defaultDRB | true |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with Condition DRB1 |  |  |
| } |  |  |  |
| } |  |  |  |
| DRB-ToAddMod[2] SEQUENCE |  | entry 2 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | PDU session ID | Same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |
| defaultDRB | false |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with Condition DRB2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.4.1.3.3-3: CellGroupConfig-2DRBs (Preamble, Table 8.1.5.4.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-19 and condition SRB2_DRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 3 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRB1 | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRB2 | entry 3 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.4.1.3.3-4: PDU SESSION ESTABLISHMENT ACCEPT (Preamble) (Table 8.1.5.4.1.3.2-1)

| Derivation Path: TS 38.508-1, table 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDU session ID | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| Authorized QoS rules | 3 entries |  |  |
| QoS rule [1] | Reference QoS rule #1 as defined in Table 4.8.2.1-1. | QFI=1 |  |
| QoS rule [2] | Reference QoS rule #4 as defined in Table 4.8.2.1-4 except DQR bit set to ‘0’B. | QFI=2 |  |
| QoS rule [3] | Reference QoS rule #5 as defined in Table 4.8.2.1-5. | QFI=5 |  |
| Mapped EPS Bearer contexts | Not Present |  |  |
| Authorized QoS flow descriptions | 3 entries |  |  |
| QoS flow [1] | Reference QoS flow #1 as defined in Table8.1.5.4.1.3.3-7. | QFI=1 |  |
| QoS flow [2] | Reference QoS flow #2 as defined in Table8.1.5.4.1.3.3-8. | QFI=2 |  |
| QoS flow [3] | Reference QoS flow #3 as defined in Table 4.8.2.3-3. | QFI=5 |  |

Table 8.1.5.4.1.3.3-5: CounterCheck (step 3, Table 8.1.5.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CounterCheck ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| counterCheck SEQUENCE { |  |  |  |
| drb-CountMSB-InfoList SEQUENCE (SIZE (1..maxDRB)) OF DRB-CountMSB-Info { | 2 entries |  |  |
| DRB-CountMSB-Info[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity with Condition DRB1 | DRB established |  |
| countMSB-Uplink | 1 |  |  |
| countMSB-Downlink | 1 |  |  |
| } |  |  |  |
| DRB-CountMSB-Info[2] SEQUENCE { |  | entry 2 |  |
| drb-Identity | DRB-Identity with Condition DRB3 | DRB not established |  |
| countMSB-Uplink | 3 |  |  |
| countMSB-Downlink | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.4.1.3.3-6: CounterCheckResponse (step 4, Table 8.1.5.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CounterCheckResponse ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| counterCheckResponse SEQUENCE { |  |  |  |
| drb-CountInfoList SEQUENCE (SIZE (0..maxDRB)) OF DRB-CountInfo { | 3 entries |  |  |
| DRB-CountInfo[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity with Condition DRB1 | DRB established |  |
| count-Uplink | 9 |  |  |
| count-Downlink | 9 |  |  |
| } |  |  |  |
| DRB-CountInfo[2] SEQUENCE { |  | entry 2 |  |
| drb-Identity | DRB-Identity with Condition DRB2 | DRB established but not requested |  |
| count-Uplink | 9 |  |  |
| count-Downlink | 9 |  |  |
| } |  |  |  |
| DRB-CountInfo[3] SEQUENCE { |  | entry 3 |  |
| drb-Identity | DRB-Identity with Condition DRB3 | DRB not established |  |
| count-Uplink] | 384 | MSB 3 and LSB 0 |  |
| count-Downlink | 384 | MSB 3 and LSB 0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.4.1.3.3-7:  Reference QoS flow #1 (Preamble)

| Derivation Path: TS 38.508-1, table 4.8.2.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QFI | ‘00 0001’B | QFI 1 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 1001’B | 5QI 9 |  |

Table 8.1.5.4.1.3.3-8: Reference QoS flow #2 (Preamble)

| Derivation Path: TS 38.508-1, table 4.8.2.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0010’B | QFI 2 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

<End of Change>