3GPP TSG-RAN5 Meeting #91-e	R5-213558

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1820 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test procedure to establish sidelink unicast mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, To add test procedures to establish sidelink unicast mode |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Test procedures to establish sidelink unicast mode are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.22(new), 4.9.23(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.508-1 CR 1819 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-212453r2 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.9.22	Procedure for establishing unicast mode NR sidelink communication / Initiating UE side

4.9.22.1	Scope

The purpose of this procedure is to establish unicast mode sidelink communication.

4.9.22.2	Procedure description

4.9.22.2.1	Initial conditions

The UE is in state 1N-B, 3N-B or 4-A.

4.9.22.2.2	Procedure sequence

Table 4.9.22.2.2-1: Procedure for establishing unicast mode sidelink communication (Initiating UE side)

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | The UE is configured by upper layer to establish unicast mode link. NOTE: This can be done by sending AT COMMAND +CV2XDTS.(FFS) | - | - |
| 2 | The UE sends a DIRECT LINK ESTABLISHMENT REQUEST message. | --> | PC5-S: DIRECT LINK ESTABLISHMENT REQUEST |
| 3 | The SS-UE sends a DIRECT LINK SECURITY MODE COMMAND message. | <-- | PC5-S: DIRECT LINK SECURITY MODE COMMAND |
| 4 | The UE sends a DIRECT LINK SECURITY MODE COMPLETE message. | --> | PC5-S: DIRECT LINK SECURITY MODE COMPLETE |
| 5 | The SS-UE sends a DIRECT LINK ESTABLISHMENT ACCEPT message. | <-- | PC5-S: DIRECT LINK ESTABLISHMENT ACCEPT |
| 6 | The UE sends a RRCReconfigurationSidelink message to establish a unicast mode SL DRB. | --> | PC5-RRC: RRCReconfigurationSidelink |
| 7 | The SS-UE sends a RRCReconfigurationSidelinkComplete message. | <-- | PC5-RRC: RRCReconfigurationSidelinkComplete |

4.9.22.3	Specific message contents

FFS

4.9.23	Procedure for establishing unicast mode NR sidelink communication / Peer UE side

4.9.23.1	Scope

The purpose of this procedure is to establish unicast mode sidelink communication.

4.9.23.2	Procedure description

4.9.23.2.1	Initial conditions

The UE is in state 1N-B, 3N-B or 4-A.

4.9.23.2.2	Procedure sequence

Table 4.9.23.2.2-1: Procedure for establishing unicast mode sidelink communication (Peer UE side)

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | The SS-UE sends a DIRECT LINK ESTABLISHMENT REQUEST message. | <-- | PC5-S: DIRECT LINK ESTABLISHMENT REQUEST |
| 2 | The UE sends a DIRECT LINK SECURITY MODE COMMAND message. | --> | PC5-S: DIRECT LINK SECURITY MODE COMMAND |
| 3 | The SS-UE sends a DIRECT LINK SECURITY MODE COMPLETE message. | <-- | PC5-S: DIRECT LINK SECURITY MODE COMPLETE |
| 4 | The UE sends a DIRECT LINK ESTABLISHMENT ACCEPT message. | --> | PC5-S: DIRECT LINK ESTABLISHMENT ACCEPT |
| 5 | The SS-UE sends a RRCReconfigurationSidelink message to establish a unicast mode SL DRB. | <-- | PC5-RRC: RRCReconfigurationSidelink |
| 6 | The UE sends a RRCReconfigurationSidelinkComplete message. | --> | PC5-RRC: RRCReconfigurationSidelinkComplete |

4.9.21.3	Specific message contents

FFS

<End of modified section 1>