3GPP TSG-RAN5 Meeting #91-e	R5-213559

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1821 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test state for NR sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, To add test states for NR SL tests |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Test states for NR SL tests are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.508-1 CR 1819 TS/TR 38.509 CR 0035 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-212454r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.4A	Test states

4.4A.1	General

The purpose of the test states is to get the UE into specific 5GC and RRC protocol states in the initial condition of test cases. Each test state is identified by a test state ID. The syntax used for test state IDs is described in sub-clause 4.4A.4. The list of defined test states and the associated UE 5GC and RRC/ N3AN protocol states are specified in sub-clause 4.4A.2.

A test case may request that one or more test functions and/or configurations are activated/configured by the SS as part of the procedure used for the requested test state. The test case requests the additional test functions and/or configurations by specifying one or more test state parameters. The list of defined test state parameters is specified in sub-clause 4.4A.3.

4.4A.2	Test states and associated 5GC and RRC protocol states

Table 4.4A.2-0: 5GC and RRC/N3AN protocol states for UE Switched Off

| 5GS state ID | Connectivity | RRC/N3AN state | 5GMM modes | 5GMM sublayer | 5GSM sublayer | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 0-A | - | - | - | - | - | UE switched off. No change to PLMN stored in the USIM |
| 0N-B | NR | - | - | - | - | UE switched off with the PLMN under test stored in the USIM |
| 0E-B | E-UTRA | - | - | - | - |  |
| 0W-B | WLAN | - | - | - | - |  |

Table 4.4A.2-1: 5GC and RRC/N3AN protocol states for IDLE

| 5GS state ID | Connectivity | RRC/N3AN state | 5GMM modes | 5GMM sublayer | 5GSM sublayer |
| --- | --- | --- | --- | --- | --- |
| 1N-A | NR | NR RRC_IDLE | 5GMM-IDLE | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 1E-A | E-UTRA | EUTRA RRC_IDLE | 5GMM-IDLE | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 1W-A | WLAN | Ipsec_SA_Released | 5GMM-IDLE | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 1N-B | NR | NR RRC_IDLE | 5GMM-IDLE | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |

Table 4.4A.2-2: 5GC and RRC protocol states for INACTIVE

| 5GS state ID | Connectivity | RRC state | 5GMM modes | 5GMM sublayer | 5GSM sublayer |
| --- | --- | --- | --- | --- | --- |
| 2N-A | NR | NR RRC_INACTIVE | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 2E-A | E-UTRA | EUTRA RRC_ INACTIVE | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |

Table 4.4A.2-3: 5GC and RRC/N3AN protocol states for CONNECTED

| 5GS state ID | Connectivity | RRC/N3AN state | 5GMM modes | 5GMM sublayer | 5GSM sublayer |
| --- | --- | --- | --- | --- | --- |
| 3N-A | NR | NR RRC_ CONNECTED | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 3E-A | E-UTRA | EUTRA RRC_ CONNECTED | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 3W-A | WLAN | Ipsec_SA_Established | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 3N-B | NR | NR RRC_ CONNECTED | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |

Table 4.4A.2-4: 5GC and RRC/N3AN protocol states for Out of Coverage

| 5GS state ID | Connectivity | RRC/N3AN state | 5GMM modes | 5GMM sublayer | 5GSM sublayer |
| --- | --- | --- | --- | --- | --- |
| 4-A | - | - | - | - | - |

4.4A.3	Test state parameters

Table 4.4A.3-1 lists the test functions and configurations that a test case can request to be activated/configured. A test case requests a test function or configuration to be used in the preamble by including the test state parameter text in the preamble statement of the test case in italics.

Editor’s Note: The test state parameters are currently limited to test functions required by standalone NR. Additional test state parameters will be added in future as needed. E.g. for EN-DC, NE-DC and NGEN-DC there will be a need for parameters for bearer type (MCG and SCG, MCG and split or MCG only).

Table 4.4A.3-1: Test state parameters

| Test state parameter | Description |
| --- | --- |
| UE test loop mode <X> prepared | If included the UE test mode is activated in the preamble indicating that UE test loop mode <X> will be activated in the test case test procedure, where <X> is A, B or E. (Note 1, Note 2, Note 3, Note 4) |
| UE test loop mode <X> active | If included the UE Test Mode and UE test loop mode <X> will be activated in the preamble, where <X> is A, B or E. (Note 1, Note 2, Note 3, Note 4) |
| Note 1: See TS 38.509 [11], clause 5.2.2 for details of UE test mode. Note 2: See TS 38.509 [11], clause 5.3.4.1 for details of UE test loop mode A. Note 3: See TS 38.509 [11], clause 5.3.4.2.2 for details of UE test loop mode B. Note 4: See TS 38.509 [11], clause 5.3.4.3 for details of UE test loop mode E. |  |

4.4A.4	Test state ID syntax

A test state ID is defined as:

<RRC state><Connectivity>-<Variant>

, where <RRC state>, <Connectivity> and <Variant> are defined in Table 4.4A.2-1.

Table 4.4A.4-1: Test state fields

| Test state field | Value | Description |
| --- | --- | --- |
| <RRC state> | 0 | Indicates that the requested test state will end up in SWITCHED_OFF state. |
|  | 1 | Indicates that the requested test state will end up in RRC_IDLE/Ipsec_SA_Released state. |
|  | 2 | Indicates that the requested test state will end up in RRC_INACTIVE state. |
|  | 3 | Indicates that the requested test state will end up in RRC_CONNECTED/Ipsec_SA_Released state. |
|  | 4 | Indicates that the requested test state will end up in Out_of_Coverage state. |
| <Connectivity> | E | E-UTRA is used as the initial access. |
|  | N | NR is used as the initial access. |
|  | W | Un trusted non 3GPP Access over WLAN is used as the initial access |
| <Variant> | A | A, B, C etc. used to represent different variants within a <RRC state><Connectivity> group of test states. |

4.4A.5	Mapping of test state IDs and test parameters to generic procedures, generic procedure parameters and specific message conditions

Depending on the test case preamble requested test state ID and test parameters the SS shall:

1> use the applicable generic procedure as specified in Table 4.4A.5-1 using the:

2> applicable generic procedure parameters as specified in Table 4.4A.5-1 and Table 4.4A.5-2; and

2> applicable message conditions as specified in Table 4.4A.5-2.

Table 4.4A.5-1: Test state ID mapping to generic procedures and Connectivity generic procedure parameter

| Test state ID |  |  |  |  |  |  |  | Generic Procedure |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RRC state field |  | Connectivity field |  |  | Variant field |  |  | Name |  | Generic procedure parameter (Note 1) | Clause |
| 0 |  |  | - |  |  | A |  |  | SWITCHED_OFF |  | 4.5.5 |
| 0 |  |  | N |  |  | B |  |  | SWITCHED_OFF | Connectivity=NR | 4.5.5 |
| 0 |  |  | E |  |  | B |  |  | SWITCHED_OFF | Connectivity=E-UTRA | 4.5.5 |
| 0 | W |  |  | B |  |  | SWITCHED_OFF |  |  | Connectivity=WLAN | 4.5.5 |
| 1 |  | N |  |  | A |  |  | RRC_IDLE |  | Connectivity=NR | 4.5.2 |
| 1 |  | E |  |  | A |  |  | RRC_IDLE |  | Connectivity=E-UTRA | 4.5.2 |
| 1 | W |  |  | A |  |  | Ipsec_SA_Released |  |  | Connectivity=WLAN | 4.5.2 |
| 1 | N |  |  | B |  |  | RRC_IDLE_with_SL |  |  | Connectivity=NR, Sidelink = On | 4.5.2 |
| 2 |  | N |  |  | A |  |  | RRC_INACTIVE |  | Connectivity=NR | 4.5.3 |
| 2 |  | E |  |  | A |  |  | RRC_INACTIVE |  | Connectivity=E-UTRA | 4.5.3 |
| 3 |  | N |  |  | A |  |  | RRC_CONNECTED |  | Connectivity=NR | 4.5.4 |
| 3 |  | E |  |  | A |  |  | RRC_CONNECTED |  | Connectivity=E-UTRA | 4.5.4 |
| 3 | W |  |  | A |  |  | Ipsec_SA_Established |  |  | Connectivity=WLAN | 4.5.4 |
| 3 | N |  |  | B |  |  | RRC_CONNECTED_with_SL |  |  | Connectivity=NR, Sidelink = On | 4.5.4 |
| 4 | - |  |  | A |  |  | Out_of_Coverage |  |  | Sidelink = On | 4.5.7 |
| Note 1: In addition to the Connectivity parameter specified in this table the applicable additional generic procedure parameters and conditions as stated in Table 4.4A.5-2 shall be used |  |  |  |  |  |  |  |  |  |  |  |

Table 4.4A.5-2: Additional generic procedure parameters and message conditions

| Test state | Additional generic | Specific message conditions |  |
| --- | --- | --- | --- |
| parameter | procedure parameter(s) | Message | Condition |
| UE test loop mode A prepared | Test Mode=On | Note 1 | Note 1 |
| UE test loop mode B prepared | Test Mode=On | ACTIVATE UE TEST MODE (Table FFS) | UE test loop mode B |
| UE test loop mode E prepared | Test Mode=On | ACTIVATE UE TEST MODE (Table FFS) | UE test loop mode E |
| UE test loop mode A active | Test Loop Function=On | Note 1 | Note 1 |
| UE test loop mode B active | Test Loop Function=On | ACTIVATE UE TEST MODE (Table FFS) | UE test loop mode B |
|  |  | CLOSE UE TEST LOOP (Table FFS) | UE test loop mode B |
| UE test loop mode E active | Test Loop Function=On | ACTIVATE UE TEST MODE (Table FFS) | UE test loop mode E |
|  |  | CLOSE UE TEST LOOP (Table FFS) | UE test loop mode E |
| Note 1: For test state parameters UE test loop mode A prepared and UE test loop mode A active there is no specific message conditions needed as the default UE test loop mode in the messages ACTIVATE UE TEST MODE and CLOSE UE TEST LOOP is UE test loop mode A. |  |  |  |

<End of modified section 1>