3GPP TSG-RAN WG5 Meeting #91-e	R5-213576

# Electronic Meeting, 17th May 2021 – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2228 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Rel-16 NPN TC 6.5.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NPN TC 6.5.1.2 SNPN / PLMN Selection in Automatic Mode |  |  |  |  |  |  |  |  |
| Summary of change: |  | NPN TC 6.5.1.2 SNPN / PLMN Selection in Automatic Mode is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NPN WP implementation will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 CR 1891 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-212804R5-212804r1: At Step 3 We need to add a Timer Value because the UE doesn’t transmit the RRC message. R5-213576: This is the final version of R5-212804r1 |  |  |  |  |  |  |  |  |

#### 6.5.1.2	SNPN Selection in Automatic Mode

6.5.1.2.1	Test Purpose (TP)

(1)

with { UE in Automatic SNPN selection mode and SNPN cell is available for which an entry does not exist in the "list of subscriber data" }

ensure that {

when { UE is switched on or return to coverage }

then { UE does not only select the SNPN cell and continues searching for an allowable SNPN Cell }

}

(2)

with { UE in Automatic SNPN selection mode and a Registered SNPN cell along with another better ranked allowable SNPN cell becomes available  }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects the Registered SNPN cell and UE attempts a Registration on the selected cell }

}

(3)

with { UE in Automatic SNPN selection mode and Registration on a SNPN cell is unsuccessful}

ensure that {

when { UE continues searching for a allowable SNPN Cell and a new allowable SNPN cell becomes available }

then { the UE registers on the new SNPN cell }

}

6.5.1.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.122 clauses 4.9.3.1.0 and 4.9.3.1.1. Unless otherwise stated these are Rel-16 requirements.

[TS 23.122, clause 4.9.3.1.0]

At switch on, or following recovery from lack of coverage, the MS selects the registered SNPN (if it is available) using NG-RAN access technology and if necessary (in the case of recovery from lack of coverage, see subclause 4.5.2) attempts to perform an LR.

NOTE 1:	The MS in automatic SNPN selection mode can end the SNPN search procedure once the registered SNPN is found on NG-RAN access technology.

NOTE 2:	An MS in automatic SNPN selection mode can use location information to determine which SNPNs can be available in its present location.

If successful registration is achieved, the MS indicates the selected SNPN.

If there is no registered SNPN, or registration is not possible due to the SNPN being unavailable or registration failure, the MS follows the procedure in subclause 4.9.3.1.1 or subclause 4.9.3.1.2 depending on its SNPN selection mode. At switch on, the MS shall use the SNPN selection mode that was used before switching off.

NOTE 3:	If successful registration is achieved, then the current serving SNPN becomes the registered SNPN and the MS does not store the previous registered SNPN for later use.

If registration is not possible on recovery from lack of coverage due to the registered SNPN being unavailable, an MS may, optionally, continue looking for the registered SNPN for an implementation dependent time.

NOTE 4:	An MS registered to an SNPN should behave as described above only if one or more PDU sessions are currently active.

[TS 23.122, clause 4.9.3.1.0]

The MS selects another SNPN, if available, allowable, and identified by an SNPN identity in an entry of the "list of subscriber data" in the ME. If more than one SNPN are available, allowable, and each of them is identified by an SNPN identity in an entry of the "list of subscriber data" in the ME, how the MS selects one of those SNPNs is MS implementation specific.

The MS shall limit its search for the SNPN to the NG-RAN access technology.

Once the MS selects the SNPN, the MS attempts registrations on the selected SNPN using the NG-RAN access technology, the subscriber identifier and the credentials from an entry of the "list of subscriber data" with the SNPN identity matching the selected SNPN.

If successful registration is achieved, the MS indicates the selected SNPN.

If registration cannot be achieved because no SNPNs are available, allowable, and identified by an SNPN identity in an entry of the "list of subscriber data" in the ME, the MS indicates "no service" to the user, waits until a new SNPN is available, allowable, and identified by an SNPN identity in an entry of the "list of subscriber data" in the ME and then repeats the procedure.

If there were one or more SNPNs which were available, allowable, and identified by an SNPN identity in an entry of the "list of subscriber data" in the ME but an LR failure made registration on those SNPNs unsuccessful, the MS selects one of those SNPNs again and enters a limited service state.

6.5.1.2.3	Test description

6.5.1.2.3.1	Pre-test conditions

System Simulator:

-	3 SNPN cells NR Cell 1, NR cell 2, NR Cell 3 are configured broadcasting default SNPN IDs as indicated in TS 38.508-1 [4] Table 4.4.2-4.

-	System information combination NR-12 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The UE is in Automatic SNPN selection mode.

-	The UE is provisioned with a “list of subscriber data” to allow access to SNPN identified by NR cell 1 and NR cell 3.

Preamble:

-	Ensure that the UE has cleared the Registered SNPN. And the UE is in state Switched OFF (state 0-A).

6.5.1.2.3.2	Test procedure sequence

Table 6.5.1.2.3.2-1/2 shows the cell configurations used during the test. Subsequent configurations marked “T1”, “T2” and “T3” are applied at the points indicated in the Main behaviour description in Table 6.5.1.2.3.2-3. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 and Table 6.2.2.2-2 for FR2.

Table 6.5.1.2.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 3 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -88 | “Off” | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | “Off” | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 | “Off” | -78 | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |

Table 6.5.1.2.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 3 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | FFS | “Off” | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | “Off” | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | FFS | “Off” | FFS | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |

Table 6.5.1.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.5.1.2.3.2-1/2. | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE transmit an RRCSetupRequest on NR Cell 2 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 4 | SS adjusts cell levels according to row T2 of table 6.5.1.2.3.2-1/2. | - | - | - | - |
| 5 | Check: Does the UE send a RRCSetupRequest on NR Cell 1? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 6-23 | Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 1. NOTE: The UE performs registration and the RRC connection is not released. | - | - | - | - |
| 24 | The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4] | - | - | - | - |
| 25 | SS adjusts cell levels according to row T3 of table 6.5.1.2.3.2-1/2. | - | - | - | - |
| 26 | The UE is Switched ON. | - | - | - | - |
| 27 | Check: Does the UE send a RRCSetupRequest on NR Cell 1? | --> | NR RRC: RRCSetupRequest | 2 | P |
| 28-35 | Steps 2-9 of Table 4.5.2.2-2 in TS38.508-1 [4] are performed on NR Cell 1. | - | - | - | - |
| 36 | The SS transmits a REGISTRATION REJECT with cause #15 (No suitable cells in tracking area). | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION REJECT | - | - |
| 37 | The SS releases the RRC connection | - | - | - | - |
| 38 | Check: Does the UE send a RRCSetupRequest on NR Cell 3? | --> | NR RRC: RRCSetupRequest | 3 | P |
| 39-56 | Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 3. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |

6.5.1.2.3.3	Specific message contents

FFS